# NVMeVirt FTL 매핑 단위 실험 기록서

KSC 2026 · FTL 매핑 단위 × fio 쓰기 크기 · 랜덤 쓰기(OS 페이지 캐시 drop 비교) · 순차 쓰기(쓰기 버퍼 병합 모델) · 랜덤 쓰기 bs 8K·64K 추가

| 항목 | 내용 |
| --- | --- |
| 주 데이터셋 | main3x3_20261008 — 매핑 4K·16K·32K × bs 4K·16K·32K × 3 회 × 페이지 캐시 drop/no-drop, wbuffix 모델 (wbuffix_drop 27회 + wbuffix_nodrop 27회) |
| 보조 데이터셋 | main_20261008 — 첫 설계(매핑 4–128K × bs 4–128K × 3 회) 중 설계 변경 전까지 측정한 부분 (wbuffix 53회) |
| 추가 실험 (순차 쓰기) | seq3x3_20261008 — 매핑 4K·16K·32K × bs 4K·16K·32K × 3 회, fio 순차 쓰기, 모델 wbuffix·merge, 페이지 캐시 그대로 (wbuffix 27회 + merge 27회) — 8 절 |
| 추가 실험 (랜덤 쓰기 bs 8K·64K) | randbs_20261008 — 매핑 4K·16K·32K × bs 8K·64K × 3 회, wbuffix, 페이지 캐시 그대로 (18회). 주 데이터셋 nodrop 의 bs 4K·16K·32K 와 합친 매핑 3 × bs 5 보기: rand3x5_20261008 — 9 절 |
| 작성일 · 시각 기준 | 2026-10-08 · 이 문서의 시각은 모두 KST (서버 시계와 원자료 로그는 UTC = KST − 9 시간) |
| 실험 수행 | Sangwon8799 (실험 서버 dccearth), 스크립트 작성·실행·기록: Claude Code (Claude Opus 5.5) |
| 저장소 | github.com/Sangwon8799/nvmevirt (main) 서버: 실험 중 /home/dccearth/jsw/KSC2026/nvmevirt → 실험 후 /home/dccearth/jsw/nvmevirt 로 이동 |
| 주 데이터셋 소스 커밋 | e598e75860c2954a934bfe9637b4190b485cd294 (원자료에 적힌 커밋 1d6cd03·a93ef76 은 같은 코드로 다시 만든 e598e75 이다 — 10 절) |
| 순차 쓰기 소스 커밋 | 66446ea9017556e3e368925a8104a97db2ce1ad4 (주 데이터셋 커밋 + 쓰기 버퍼 병합 모델·순차 쓰기 스크립트) |
| 랜덤 bs 8K·64K 소스 커밋 | a86fc8790f7b82121fed2a27d76bf93074a32f0a (wbuffix 모듈은 주 데이터셋과 같은 파일) |
| NVMeVirt 원본 | github.com/snu-csl/nvmevirt @ 61c90f7 (2026-05-21) |

이 문서는 실험 시작부터 결과를 얻기까지의 전 과정을 기록한다. 다른 사람이 같은 서버 구성에서 문서만 보고 같은 실험을 다시 할 수 있도록 환경·버전·바꾼 설정값·사용한 스크립트 전문·실행 순서·결과를 모두 적었다. 진행 중 실험 설계가 한 번 바뀌었으며(1.3 절), 바뀐 최종 설계의 결과가 주 데이터셋이다. 그 뒤 같은 3 × 3 설계로 순차 쓰기를 추가로 측정했다(8 절). 이어서 랜덤 쓰기에 bs 8K·64K 를 추가로 측정해 bs 를 5 단계로 넓혔다(9 절).

목차: Word 에서 열 때 '필드 업데이트'를 누르면 채워진다.

# 1. 실험 개요

## 1.1 목적

고용량 SSD 는 4 KiB 매핑을 유지하면 L2P(논리→물리) 매핑 표가 용량의 약 0.1 % 만큼 DRAM 을 차지한다(1 TB 에 약 1 GB). 매핑 단위를 16 KiB 이상으로 키우면 DRAM 은 줄지만, 매핑 단위보다 작은 쓰기는 매핑 단위 전체를 새로 써야 하므로 성능이 떨어질 수 있다. 이 실험은 NVMeVirt(커널 모듈 기반 NVMe SSD 에뮬레이터)의 conventional SSD 모델에서 FTL 매핑 단위를 바꾸고, fio 랜덤 쓰기 블록 크기별로 대역폭(BW), IOPS, 완료 지연(clat)을 잰다. 연구 계획(연구 계획 흐름.docx)의 「(4k mapping ssd) randwrite … → BW, clat / (16k/32k mapping ssd) …」 를 따른다. 또한 OS 페이지 캐시를 비우는 명령(echo 3 > /proc/sys/vm/drop_caches)을 실험 전에 실행하는지에 따라 결과가 달라지는지 확인한다. 추가로 같은 매핑 단위·bs 조합에서 순차 쓰기를 측정한다. 이때 NVMeVirt 가 매핑 단위보다 작은 쓰기를 쓰기 버퍼에서 합치지 않는 한계가 결과를 좌우하므로, 원래 모델(wbuffix)과 이 연구에서 추가한 쓰기 버퍼 병합 모델(merge)을 함께 잰다(8 절).

## 1.2 최종 실험 행렬과 고정 조건 (주 데이터셋)

표 1. 최종 실험 행렬과 고정 조건

| 구분 | 값 |
| --- | --- |
| 변수 ① FTL 매핑 단위 | 4K, 16K, 32K (매핑 단위별로 모듈을 따로 빌드) |
| 변수 ② fio 블록 크기(bs) | 4K, 16K, 32K |
| 변수 ③ OS 페이지 캐시 | nodrop: 그대로 / drop: insmod 직후 sync; echo 3 > /proc/sys/vm/drop_caches. 같은 (매핑, bs, 회차)에서 두 조건을 연달아 측정하며, 순서는 홀수 회차 nodrop→drop, 짝수 회차 drop→nodrop |
| 반복 | 조합마다 3 회 (회차마다 rmmod → insmod 로 장치 초기화) — 9 × 2 × 3 = 54 회 |
| NVMeVirt 모델 | wbuffix = 요청 설정 + 쓰기 버퍼 계산 수정 1 줄 (5.4 절; 사용자 선택) |
| NVMeVirt 모드 | Conventional SSD (SAMSUNG_970PRO 묶음), 블록 2 MiB (BLKS_PER_PLN = 384), flash page 32 KiB |
| 저장 용량 | 예약 메모리 12 GiB (물리 주소 12–24 GiB), 호스트에 보이는 용량 11.21 GiB (OP 7 %) |
| NVMeVirt CPU | 3 개: cpu3 = 디스패처, cpu4·cpu5 = I/O 워커 (isolcpus=3-5) |
| fio | ioengine=libaio, direct=1, rw=randwrite, iodepth=32, numjobs=1, runtime=60 s, ramp_time=0, 장치 전체 |
| 측정값 | 60 s 평균 BW·IOPS·clat(평균, p50, p99, p99.9), 0.5 s 간격 시계열, 첫 GC 시각, GC 전/후 BW, 쓰기 증폭(WAF) |

## 1.3 실험 설계 변경 경위

표 2. 설계 변경 경위

| 시각 (KST) | 내용 |
| --- | --- |
| 13:4x | 첫 지시: 매핑 4·8·16·32·64·128K, 64K 이상은 flash page = 매핑 단위, BLK 2 MiB, 3 회 반복, 60 s, ramp_time 0 등. bs 는 연구 계획에 따라 4–128K 로 정함 |
| 13:49–15:22 | NVMeVirt 코드 감사(에이전트 51 개): 매핑 단위보다 작은 쓰기에서 쓰기 버퍼가 과다 반환되는 결함 발견(5.4 절) |
| 14:20–14:46 | 첫 설계 6 × 6 × 3 시작. 당시에는 쓰기 버퍼 계산을 고치지 않은 원본 모델(base)을 먼저 돌렸다. 이 모델은 bs < 매핑 단위 조합에서 5.4 절의 결함 때문에 측정이 유효하지 않다 |
| 14:49–15:51 | 스크립트가 실패한 회차를 기록하고 계속하도록 고친 뒤, 쓰기 버퍼 계산을 고친 모델(wbuffix)로 실행 (53 회 완료) |
| 15:5x | 선배 요청: 매핑·bs 를 4K·16K·32K 로 줄이고, drop_caches 사용 여부에 따른 차이를 확인 → 15:51:23 회차 경계에서 중단 |
| 15:5x | 모델 선택: 사용자가 쓰기 버퍼 계산을 고친 wbuffix 만 쓰기로 함. drop/no-drop 은 회차마다 교대로 측정하기로 함 |
| 16:00:35– | 최종 설계(주 데이터셋) 실행 (17:03:57 완료) |
| 19:53 | 사용자 요청: 순차 쓰기도 같은 3 × 3 으로 측정. NVMeVirt 가 매핑 단위보다 작은 쓰기를 쓰기 버퍼에서 합치지 않는 한계(5.5 절)를 설명했고, 사용자가 '원래 모델(wbuffix)과 병합 모델(merge) 둘 다 측정', '페이지 캐시 nodrop 만'을 골랐다 |
| 19:58–20:13 | 쓰기 버퍼 병합 모델(WBUF_MERGE) 작성(19:58–19:59) → 에이전트 5 개의 독립 코드 검토(20:01–20:13, 결함 없음, 8.2 절) → 커밋 66446ea, merge 모듈 빌드 |
| 20:18–20:23 | 사전 점검: 커널 기본 설정에서 블록 계층이 순차 4K 요청을 평균 약 120 KiB 로 합치는 것을 발견 → 순차 쓰기는 nomerges=2 로 측정 (8.3 절) |
| 20:23:24–21:26:45 | 순차 쓰기 실험 실행 (8 절) |
| 22:08 | 사용자 요청: 랜덤 쓰기에 8K·64K 도 측정. 사용자가 'bs 만 추가(매핑은 4K·16K·32K 그대로)', '새 조합만 측정하고 기존 bs 4K·16K·32K 값은 주 데이터셋 nodrop 값을 재사용'을 골랐다 |
| 22:11:48–22:32:55 | 랜덤 쓰기 bs 8K·64K 실험 실행 (9 절) |
| 23:33 | 사용자 요청: 원본 모델(base)로 잰 결과는 보려던 실험이 아니므로 문서와 저장소에서 뺌. 원자료(첫 설계의 base 회차와 스모크 테스트)와 그 수치·표·그림을 23:38 부터 최신 상태에서 지웠다 (10 절) |

## 1.4 결과 요약

페이지 캐시 drop(sync; echo 3 > /proc/sys/vm/drop_caches)은 결과에 영향이 없다. 27 쌍(9 조합 × 3 회)의 대역폭 차이는 평균 +0.009 %(95 % CI −0.084 ~ +0.101 %, p = 0.85)이고, 조합별 차이도 −0.21 ~ +0.39 %(모든 p > 0.15)다. IOPS·평균/p99 지연·GC 전후 대역폭·WAF 도 유의한 차이가 없다.

drop 조건에서 일관되게 달라진 것은 fio 시작 표지부터 첫 GC 까지의 시간뿐이다. 23–36 ms(+0.51 %, p < 1e-12) 늦은데, 캐시를 비운 뒤 sudo·fio 실행 파일을 디스크에서 다시 읽느라 fio 시작이 늦어진 것으로 보인다. 장치 성능과는 무관하다.

bs ≥ 매핑 단위에서는 60 s 평균 대역폭이 416–482 MiB/s 로 매핑 단위에 따른 차이가 작다. bs 16K 는 매핑 4K 481.0, 16K 420.4 MiB/s(−12.6 %)이고, bs 32K 는 매핑 4K 482.3, 16K 441.6, 32K 424.6 MiB/s(−8.4 %, −11.9 %)다.

bs < 매핑 단위에서는 성능이 크게 떨어진다. 매핑 16K·bs 4K 는 129.7 MiB/s 로 매핑 4K·bs 4K(416.0) 대비 −68.8 %, 매핑 32K·bs 4K 는 68.3 MiB/s(−83.6 %), 매핑 32K·bs 16K 는 219.6 MiB/s 로 매핑 4K·bs 16K(481.0) 대비 −54.3 % 다.

그 원인은 쓰기 증폭이다. WAF_total(NAND 바이트 ÷ 호스트 바이트)은 bs ≥ 매핑에서 3.1–3.5 인데, 16K/4K 8.8, 32K/4K 17.1, 32K/16K 6.0 이다. 이는 매핑 단위/bs 배의 부분 쓰기 증폭(4·8·2 배)에 GC 증폭(2.1–3.0)이 곱해진 값이다.

GC 이전 구간의 대역폭은 NAND 프로그램 한계로 정해진다. bs ≥ 매핑에서는 2,001–2,218 MiB/s 로, 16 die 합 한계 2,233 MiB/s 의 90–99 % 다. bs < 매핑에서는 그 한계 × bs/매핑에 가깝다(16K/4K 535.6 = 한계 × 1/4 의 96 %, 32K/4K 256.1 = × 1/8 의 92 %, 32K/16K 1,026.9 = × 1/2 의 92 %).

장치가 빈 상태에서 시작하므로 모든 조합에서 첫 GC 는 5.7–6.3 s 에 시작한다(물리 공간의 380/384 line 이 찼을 때). 60 s 평균에는 GC 이전 약 6 s(약 2 GB/s)가 크게 반영된다. GC 이후 구간 평균은 bs ≥ 매핑에서 240–307 MiB/s, bs < 매핑에서 47–130 MiB/s 다.

GC 이후 대역폭은 fio 의 randommap 주기를 따라 출렁인다. bs ≥ 매핑에서는 GC 시작 직후 약 130 MiB/s 까지 떨어졌다가 회복하고, 장치를 한 바퀴 다 쓸 무렵(약 37–52 s) 1,500–1,900 MiB/s 로 치솟은 뒤 다시 떨어진다. bs < 매핑에서는 치솟지 않고 계속 떨어진다. 그래서 마지막 20 s 평균은 조합에 따라 순서가 바뀐다(예: bs 16K 에서 매핑 4K 244.8 < 매핑 16K 323.4 MiB/s). 결과를 비교할 때는 60 s 평균과 GC 이후 구간 평균을 함께 보아야 한다.

평균 완료 지연은 IOPS 와 반비례한다(iodepth 32 고정, Little 의 법칙). 4K/4K 299 µs, 16K/4K 963 µs, 32K/4K 1,829 µs 다. p99 지연은 모든 조합에서 13–18 ms 로, GC 로 쓰기 버퍼가 막히는 구간이 결정한다.

반복 간 대역폭 편차는 매우 작다. 변동계수 평균 0.15 %, 최대 0.37 %(4K/4K, nodrop; drop 은 0.35 %)다. fio 난수 순서가 고정이라(randrepeat=1) 1–3 회차의 시계열이 거의 겹친다.

## 1.5 순차 쓰기 추가 실험 결과 요약

순차 쓰기에서 bs ≥ 매핑 단위인 조합은 두 모델 모두 60 s 평균 2,087–2,233 MiB/s 다. bs 16K·32K 는 2,232.2–2,232.8 MiB/s 로, NAND 프로그램 한계(16 die 합 2,233 MiB/s)의 99.96 % 이상이다. bs 4K 는 매핑 4K(두 모델)와 병합 모델의 매핑 16K·32K 에서 약 535 K IOPS, 2,087–2,095 MiB/s(한계의 93.4–93.8 %)에서 멈춘다(요청 수 처리 쪽 한계로 보인다 — 추정). 이 조합들에서 두 모델의 차이는 최대 0.14 % 로, 반복 편차(변동계수 최대 0.40 %) 수준이다. 두 모델이 같은 코드 경로를 쓰기 때문이다.

원래 모델(wbuffix)에서 bs < 매핑 단위인 순차 쓰기는 크게 느려진다. 16K/4K 334.9, 32K/4K 209.5, 32K/16K 369.0 MiB/s 로, 같은 bs 의 매핑 4K 대비 −84.0 %, −90.0 %, −83.5 % 다. NVMeVirt 는 같은 매핑 단위로 이어지는 작은 쓰기를 합치지 않고 쓰기마다 매핑 단위 페이지를 새로 쓰기 때문이다. GC 이전 구간도 NAND 한계 × bs/매핑의 99.4–99.8 %(556.1, 278.5, 1,109.9 MiB/s)에 머문다.

쓰기 버퍼 병합 모델(merge)에서는 이 불이익이 사라진다. 16K/4K 2,094.4, 32K/4K 2,094.6, 32K/16K 2,232.2 MiB/s 로, 같은 bs 의 매핑 4K(2,086.7 / 2,086.7 / 2,232.7) 대비 +0.37 %, +0.38 %, −0.02 % 다. wbuffix 보다는 6.25 배, 10.00 배, 6.05 배 빠르다. 즉 순차 쓰기는 쓰기 버퍼가 작은 쓰기를 합치기만 하면 매핑 단위를 16K·32K 로 키워도 대역폭 손실이 없다.

WAF_total 은 병합 모델에서 9 조합 모두 1.000 이다. 원래 모델은 bs ≥ 매핑에서 1.000, bs < 매핑에서 16K/4K 5.404, 32K/4K 9.155, 32K/16K 3.405 다. 원래 모델의 값은 부분 쓰기 증폭(매핑/bs = 4·8·2 배)과 GC 증폭(1.351, 1.144, 1.702)의 곱이다. GC 증폭은 매핑 단위마다 마지막 판 하나만 유효하게 남기 때문에 생긴다. 16K/4K·32K/4K 는 단순 계산 1/(1 − bs/매핑) = 1.333·1.143 과 거의 같고, 32K/16K 는 단순 계산 2.000 보다 작다.

랜덤 쓰기와 비교하면 같은 조합에서 순차 쓰기(merge)가 bs ≥ 매핑에서 4.6–5.3 배, bs < 매핑에서 10.2–30.7 배 빠르다. 랜덤 쓰기(주 데이터셋 nodrop)는 bs ≥ 매핑에서 GC 복사 때문에 416–482 MiB/s(WAF 3.1–3.5)다. 랜덤 쓰기의 bs < 매핑 손실(−54–84 %)은 다음 쓰기가 다른 매핑 단위로 가서 합칠 것이 없으므로 쓰기 버퍼 병합으로도 줄지 않는다(8.3 절 C: merge −0.7 %). 반면 순차 쓰기의 손실은 병합으로 없어진다.

병합 통계(3 회 평균)에서 열린 단위 하나당 합류 횟수는 16K/4K 3.000, 32K/4K 7.000, 32K/16K 1.000 으로 매핑/bs − 1 과 정확히 같다. 덜 찬 채 쓴 단위(evict)는 회차당 169·37·11 개로, 연 단위의 0.0003–0.002 % 뿐이다. 장치 끝에서 처음으로 돌아갈 때 생기고, 4K 에서 더 많은 것은 드물게 요청 순서가 바뀌기 때문으로 보인다(추정).

## 1.6 랜덤 쓰기 bs 8K·64K 추가 실험 결과 요약

bs 8K 에서 대역폭은 매핑 4K 437.0, 16K 217.3, 32K 131.9 MiB/s 다. 매핑 4K 대비 −50.3 %, −69.8 % 다. bs 64K 에서는 486.3, 486.4, 447.1 MiB/s 로, 매핑 16K 는 4K 와 같고(+0.02 %) 32K 는 −8.1 % 다.

결과는 bs 의 절대 크기보다 bs/매핑 비율로 거의 정해진다. 비율이 같은 조합끼리는 대역폭이 3 % 안에서 같고 WAF_total 도 거의 같다(표기는 다른 곳과 같이 매핑/bs). 비율 1(4K/4K, 16K/16K, 32K/32K)은 416.0·420.4·424.6 MiB/s, WAF 3.12 다. 비율 2(4K/8K, 16K/32K, 32K/64K)는 437.0·441.6·447.1 MiB/s, WAF 3.31–3.32 다. 비율 4(4K/16K, 16K/64K)는 481.0·486.4 MiB/s 다. 비율 1/2(16K/8K, 32K/16K)은 217.3·219.6 MiB/s, WAF 5.90·5.96 이고, 비율 1/4(16K/4K, 32K/8K)은 129.7·131.9 MiB/s, WAF 8.82·8.97 이다. flash page 는 모든 조합에서 32 KiB 로 같다.

bs ≥ 매핑인 10 조합은 416.0–486.4 MiB/s, WAF_total 3.12–3.55 다. 같은 bs 에서 매핑 4K 대비 차이는 −12.6 %(bs 16K, 매핑 16K)에서 +0.02 %(bs 64K, 매핑 16K) 사이다. 매핑 4K 는 bs 를 4K → 64K 로 키우면 416.0 → 486.3 MiB/s(+16.9 %)로 오른다.

bs < 매핑인 5 조합의 대역폭은 같은 매핑에서 bs = 매핑일 때의 0.16–0.52 배로, 조합마다 bs/매핑(0.125–0.5)보다 조금 크다. WAF_total 은 부분 쓰기 증폭(매핑/bs = 2·4·8 배)과 GC 증폭(2.1–3.0)의 곱이다. 16K/8K 는 5.90 = 2 × 2.95, 32K/8K 는 8.97 = 4 × 2.243 이다.

GC 이전 구간은 NAND 프로그램 한계로 정해진다. bs ≥ 매핑에서는 2,001–2,218 MiB/s 이고, bs < 매핑에서는 한계 × bs/매핑의 92–96 % 다(예: 16K/8K 1,072.0, 32K/8K 512.6 MiB/s). 첫 GC 는 모든 조합에서 5.69–6.32 s 에 시작한다. GC 이후 구간 평균은 bs ≥ 매핑에서 240–312 MiB/s, bs < 매핑에서 47–131 MiB/s 다.

# 2. 실험 환경

## 2.1 하드웨어

표 3. 하드웨어

| 항목 | 값 |
| --- | --- |
| 서버(호스트명) | dccearth |
| 메인보드 | Micro-Star International Co., Ltd. MAG B760M MORTAR (MS-7E01) |
| BIOS | M.30 (05/16/2023) |
| CPU | 13th Gen Intel(R) Core(TM) i5-13600K OS 에 보이는 CPU 6개 (코어당 스레드 1), 최대 5100.0000 MHz, L3 24 MiB (1 instance) |
| 메모리 | 총 24 GB (DDR5-5600, 슬롯 4 개 중 2 개 사용, 최대 128 GB) Controller0-DIMMA2: 8 GB DDR5-5600 Samsung M323R1GB4DB0-CWMOL Controller1-DIMMB2: 16 GB DDR5-5600 Samsung M323R2GA3DB0-CWMOL (DIMMA1·DIMMB1 비어 있음; 출처 main_20261008/env_before/12_dimm.txt, sudo dmidecode -t memory) |
| NUMA | 1 노드 |
| 시스템 디스크 | Samsung SSD 980 PRO 500GB (/dev/nvme0n1, 루트 파일시스템) — 실험 대상 아님 |
| 실험 대상 장치 | NVMeVirt 가상 NVMe (모델 CSL_Virt_MN_01, 적재 후 /dev/nvme1n1) |

## 2.2 운영체제·커널·부팅 설정

표 4. 운영체제·커널·부팅 설정

| 항목 | 값 |
| --- | --- |
| OS | Ubuntu 24.04.4 LTS |
| 커널 | 6.8.0-142-generic (x86_64-linux-gnu-gcc-13 (Ubuntu 13.3.0-6ubuntu2~24.04.1) 13.3.0, GNU ld (GNU Binutils for Ubuntu) 2.42) |
| 커널 명령줄 | memmap=12G$12G isolcpus=3-5 |
| /etc/default/grub | GRUB_CMDLINE_LINUX="memmap=12G\\\$12G isolcpus=3-5" |
| 격리 CPU | 3-5 |
| 예약 메모리 확인 | /proc/iomem: 300000000-5ffffffff : Reserved (12.000–24.000 GiB, 12 GiB) e820 user map: [mem 0x0000000300000000-0x00000005ffffffff] reserved (스냅샷 아님 — 14:06 KST 에 sudo dmesg 로 직접 확인. env_before 의 dmesg e820 grep 은 커널 링 버퍼가 이미 덮여 비어 있다, 부록 C) |
| CPU 주파수 | intel_pstate active, governor powersave, EPP balance_performance, 터보 켜짐(no_turbo=0), 800–5100 MHz (모두 기본값, 바꾸지 않음) |
| THP | always [madvise] never |
| 콘솔 로그 레벨 | kernel.printk = 4 4 1 7 (기본값) — KERN_ERR 메시지는 tty0 콘솔에도 출력됨 (5.4 절 참고) |
| dmesg 제한 | kernel.dmesg_restrict = 1 → dmesg 는 sudo 로 읽음 |

## 2.3 소프트웨어 버전

표 5. 소프트웨어 버전

| 소프트웨어 | 버전 | 비고 |
| --- | --- | --- |
| fio | fio-3.36 | Ubuntu 패키지 3.36-1ubuntu0.1 |
| libaio | 0.3.113-6build1.1 | ioengine=libaio |
| gcc | 13.3.0 | 패키지 gcc-13 13.3.0-6ubuntu2~24.04.1 (커널 빌드에 쓴 것과 같은 버전) |
| make | 4.3 |  |
| 커널 헤더 | 6.8.0-142.142 | NVMeVirt 모듈 빌드용 |
| nvme-cli | 2.8 | 장치 정보 기록용 |
| util-linux (dmesg) | 2.39.3 |  |
| Python | 3.12.3 | 분석·문서 생성 (exp/.venv) |
| numpy / matplotlib / python-docx | 2.5.3 / 3.11.2 / 1.2.0 | exp/requirements.txt |
| git | 2.43.0 |  |
| bash | 5.2.21(1)-release |  |

## 2.4 NVMeVirt 버전

표 6. NVMeVirt 버전

| 항목 | 값 |
| --- | --- |
| 원본 저장소 | https://github.com/snu-csl/nvmevirt (branch main) |
| 원본 커밋 | 61c90f7758cbd9545b4a4727e89377bf88eab060 2026-05-21 11:05:11 +0900 · Merge pull request #72 (zns append return slba) |
| 모듈 버전 문자열 | NVMeVirt: Version 1.10 for >> Samsung 970 Pro SSD << (적재 시 dmesg) |
| 실험용 포크 | https://github.com/Sangwon8799/nvmevirt (원본 이력 유지, 소스는 nvmevirt/ 하위 폴더로 이동) |
| 포크 커밋 (실험 시점) | e598e75 exp: final 3x3 design with OS page-cache comparison 949ae38 exp: keep going when a run fails; plot tool; report generators 5769378 exp: scripts for the KSC2026 mapping-unit experiment c02b9fd nvmevirt: KSC2026 mapping-unit configuration d508610 Move NVMeVirt sources into nvmevirt/ subdirectory |

## 2.5 CPU 배치

표 7. CPU 배치

| CPU | 역할 | 근거 |
| --- | --- | --- |
| cpu0–2 | 운영체제, fio(고정하지 않음 — isolcpus 때문에 0–2 에서만 실행됨), dmesg 기록 프로세스 | isolcpus=3-5 |
| cpu3 | NVMeVirt 디스패처 (I/O 명령 처리, FTL, GC 를 모두 이 스레드가 수행) | cpus= 의 첫 값 · dmesg 'nvmev_dispatcher started on cpu 3' |
| cpu4 | NVMeVirt I/O 워커 0 (데이터 복사·완료 처리) — 실제로 모든 I/O 를 처리 | dmesg 'nvmev_io_worker_0 started on cpu 4' |
| cpu5 | NVMeVirt I/O 워커 1 — I/O 는 받지 않고 폴링만 함. 가상 장치의 인터럽트(IRQ 15)가 이 CPU 로 전달되어 호스트 nvme 완료 처리가 여기서 실행됨 | 가상 장치가 MSI-X 없이 레거시 IO-APIC IRQ 15 하나(nvme1q0·nvme1q1 공유)만 받아 I/O 큐가 1 개('1/0/0 default/read/poll queues'). CONFIG_NVMEV_IO_WORKER_BY_SQ 로 큐 번호에 따라 워커를 고르므로 워커 0 만 쓰인다. /proc/irq/15/effective_affinity_list = 5 (smp_affinity_list 0-5), irqbalance 미설치 |

# 3. 실험 전 준비 (세팅)

## 3.1 이 실험 이전에 해 둔 서버 설정

다음은 이 실험 세션 전에 이미 적용되어 있던 설정이다. 실험 전에 그대로인지 확인만 했다(2.2 절의 값).

# /etc/default/grub — NVMeVirt 저장 공간 예약(12 GiB, 물리 주소 12 GiB 부터)과 NVMeVirt 전용 CPU 격리

GRUB_CMDLINE_LINUX="memmap=12G\\\$12G isolcpus=3-5"

sudo update-grub

sudo reboot

# 확인

cat /proc/cmdline                       # ... memmap=12G$12G isolcpus=3-5

sudo grep -i 'System RAM\|Reserved' /proc/iomem   # 300000000-5ffffffff : Reserved

sudo apt install fio                    # fio 3.36 (Ubuntu 24.04 패키지)

# 모듈 빌드·분석·실행에 필요한 나머지 패키지 — 이 서버에는 이미 설치되어 있었다(버전은 표 5). 새 서버에서는 함께 설치한다.

sudo apt install build-essential linux-headers-$(uname -r) python3-venv tmux nvme-cli git

# irqbalance 는 설치되어 있지 않다(dpkg-query -W irqbalance → 없음). 설치된 서버라면 끄고 재부팅해 IRQ 위치를 고정한다.

# NVMeVirt 적재 후 확인: grep nvme1 /proc/interrupts; cat /proc/irq/<IRQ>/effective_affinity_list   # 이 서버: IRQ 15 → cpu5 (2.5 절)

## 3.2 저장소 준비 (fork → 하위 폴더 구성 → push)

원본 이력을 그대로 둔 채 모든 파일을 nvmevirt/ 하위 폴더로 옮기고(내용 변경 없음), 실험용 파일은 exp/ 에 두었다. GitHub 에는 사용자가 만든 빈 저장소 Sangwon8799/nvmevirt 에 올렸다(서버에는 GitHub 키가 없고, 사용자 PC 의 ssh-agent 를 SSH agent forwarding 으로 사용).

cd /home/dccearth/jsw/KSC2026

git clone https://github.com/snu-csl/nvmevirt.git nvmevirt && cd nvmevirt   # 61c90f7

git remote rename origin upstream

mkdir nvmevirt

git ls-tree --name-only HEAD | while read f; do git mv "$f" nvmevirt/; done

git commit -m "Move NVMeVirt sources into nvmevirt/ subdirectory"

# (소스 수정 · exp/ 추가 후 커밋 — 3.4 절, 부록 A)

git remote add origin git@github.com:Sangwon8799/nvmevirt.git

git push -u origin main

표 8. 저장소 구성

| 경로 | 내용 |
| --- | --- |
| nvmevirt/ | NVMeVirt 커널 모듈 소스 (원본 + 3.4 절 수정) |
| exp/common.sh | 공통 설정값 (memmap, cpus, 매핑 단위·bs 목록, 반복 횟수, fio 값, 빌드 변형) |
| exp/build_modules.sh | 매핑 단위별 nvmev.ko 빌드 → exp/modules/<변형>/nvmev_map<단위>.ko (+ SHA256SUMS) |
| exp/run_experiment.sh | 회차 실행: rmmod → insmod → fio 60 s → rmmod, 커널 로그 기록, 이어서 하기 지원 |
| exp/collect_env.sh | 실험 전·후 환경 스냅샷 |
| exp/jobs/randwrite.fio.in | fio 작업 파일 틀 (회차마다 값을 채워 job.fio 로 저장) |
| exp/jobs/seqwrite.fio.in | 순차 쓰기 틀 (rw=write; 8 절) |
| exp/analyze.py | 결과 집계(CSV), drop/no-drop 쌍 비교, 그림 |
| exp/plot.py | 그래프 도구 (bs/map/heatmap/ts/compare/all) |
| exp/run_all.sh | 최종 설계: wbuffix 모듈 → 3×3×3×{nodrop, drop} → 분석 |
| exp/run_all_6x6.sh | 첫 설계: 빌드 → wbuffix 6×6×3 → 분석 (보조 데이터셋에 사용) |
| exp/run_all_seq.sh | 순차 쓰기: 3×3×3 × {wbuffix, merge}, nomerges=2 → 분석 |
| exp/make_gallery.py | 한 실험의 모든 그림을 HTML 한 파일로 모음 |
| exp/run_all_rand_bs.sh | 랜덤 쓰기 bs 8K·64K: 매핑 3 × bs 2 × 3 회 → 분석 → link_runs.py 로 합친 보기 → 분석 |
| exp/link_runs.py | 여러 데이터셋의 회차 폴더를 상대 심볼릭 링크로 모은 보기(view) 데이터셋을 만듦 |
| exp/report/ | 이 문서·인계 기록 생성기 (make_report.py, docx_helpers.py, make_md_results.py, findings_ko.txt, findings_seq_ko.txt, findings_randbs_ko.txt, make_handoff.sh), 감사 결과, sudoers 사본, 이 문서(.docx) |
| exp/results/main3x3_20261008/ | 주 데이터셋 (wbuffix_nodrop/, wbuffix_drop/, env_before·env_after, analysis/) |
| exp/results/main_20261008/ | 보조 데이터셋 (wbuffix/, env_before·env_after_stop, analysis/) |
| exp/results/seq3x3_20261008/ | 순차 쓰기 실험 (wbuffix/, merge/, env_before·env_after, analysis/) |
| exp/results/randbs_20261008/ | 랜덤 쓰기 bs 8K·64K (wbuffix/, env_before·env_after, analysis/) |
| exp/results/rand3x5_20261008/ | 합친 보기: wbuffix/ 아래 회차는 주 데이터셋 wbuffix_nodrop/ 과 randbs_20261008/wbuffix/ 로 가는 링크 (SOURCES.txt), analysis/·plots/ |
| exp/results/pre_*/ | 사전 점검 (GC_STATS 영향 pre_gcstats_ab, 페이지 캐시 drop 시험 pre_cache_test, 순차 쓰기 점검 pre_seq_*·pre_rand_merge_check) |
| EXPERIMENT_LOG_FOR_CLAUDE.md | 다른 Claude 에게 넘기는 상세 기록(모든 지시·결정·수치) |

## 3.3 실행 권한 (sudoers)

insmod·rmmod·블록 장치에 대한 fio·dmesg 는 root 권한이 필요하다. 실험을 무인으로 돌리기 위해 필요한 명령만 비밀번호 없이 쓰도록 규칙을 추가했다(14:06 KST 설치, 14:08 /dev/kmsg 추가, 15:55 drop_caches 추가, 18:33 제거, 20:17 순차 쓰기 실험을 위해 다시 설치, 21:34 제거, 22:11 랜덤 쓰기 bs 8K·64K 실험을 위해 다시 설치, 23:08 제거). /dev/kmsg 쓰기는 회차의 시작·끝 표지를 커널 로그에 남겨 NVMeVirt 메시지와 같은 시계로 시간을 재기 위해, /proc/sys/vm/drop_caches 쓰기는 페이지 캐시 drop 조건에, /sys/block/nvme1n1/queue/nomerges 쓰기는 순차 쓰기 실험에서 블록 계층 요청 병합을 끄는 데(8.1 절) 쓴다.

# KSC2026 NVMeVirt mapping-unit experiment — remove after the experiment:

#   sudo rm /etc/sudoers.d/nvmevirt-exp

# (chown path = <clone>/exp/*; the nomerges rule names the NVMeVirt namespace of this server, nvme1n1)

dccearth ALL=(root) NOPASSWD: /usr/sbin/insmod, /usr/sbin/rmmod, /usr/bin/fio, /usr/bin/dmesg, /usr/sbin/nvme, /usr/bin/cat /proc/iomem, /usr/bin/tee /dev/kmsg, /usr/bin/tee /proc/sys/vm/drop_caches, /usr/bin/tee /sys/block/nvme1n1/queue/nomerges, /usr/bin/chown -R dccearth\:dccearth /home/dccearth/jsw/nvmevirt/exp/*

# 설치 (저장소 최상위 디렉터리에서; 문법 검사 후 설치)

sudo visudo -cf exp/report/nvmevirt-exp.sudoers && sudo install -m 0440 -o root -g root exp/report/nvmevirt-exp.sudoers /etc/sudoers.d/nvmevirt-exp

# 실험 후 제거

sudo rm /etc/sudoers.d/nvmevirt-exp

위 규칙은 순차 쓰기 실험(8 절) 때 설치한 것이다(20:17 KST, 사용자가 직접 설치). 랜덤 쓰기 실험(주·보조 데이터셋) 때의 규칙에는 nomerges 항목이 없었고, 마지막 chown 경로가 저장소를 옮기기 전 위치(/home/dccearth/jsw/KSC2026/nvmevirt/exp/*)였다(태그 ksc2026-final 의 같은 파일). chown 항목은 사용자(dccearth)와 저장소 위치에 묶여 있다. run_experiment.sh 는 회차마다 'sudo -n chown -R <사용자>:<사용자> <회차 폴더>' 를 실행하고, 이것이 실패하면 set -e 때문에 첫 회차의 rmmod 직후 DONE 표지 없이 멈춘다(사전 점검은 chown 을 확인하지 않는다). nomerges 항목은 이 서버의 NVMeVirt 장치 이름(nvme1n1)을 그대로 적었다(sudoers 인자의 * 는 공백과 / 까지 맞으므로 와일드카드를 쓰지 않았다). 다른 곳에 clone 했거나 장치 이름이 다르면 아래처럼 바꾼 규칙을 설치한다(4.3 절).

<USER> ALL=(root) NOPASSWD: /usr/sbin/insmod, /usr/sbin/rmmod, /usr/bin/fio, /usr/bin/dmesg, /usr/sbin/nvme, /usr/bin/cat /proc/iomem, /usr/bin/tee /dev/kmsg, /usr/bin/tee /proc/sys/vm/drop_caches, /usr/bin/tee /sys/block/<NVMeVirt 장치>/queue/nomerges, /usr/bin/chown -R <USER>\:<USER> <REPO>/exp/*

## 3.4 NVMeVirt 소스 수정 사항

원본(61c90f7) 대비 바꾼 것은 아래가 전부다. 전체 diff 는 부록 B 에 있다.

표 9. NVMeVirt 수정 사항

| 파일 | 항목 | 원본 값 | 실험 값 | 이유 |
| --- | --- | --- | --- | --- |
| Kbuild | 빌드 대상 | CONFIG_NVMEVIRT_NVM (Optane) | CONFIG_NVMEVIRT_SSD (BASE_SSD=SAMSUNG_970PRO) | conventional SSD(FTL·GC 모델) 사용 |
| Kbuild | MAPPING_UNIT | 없음 | make 변수 (기본 4096) | 매핑 단위별 모듈을 한 소스에서 빌드 |
| ssd.c:73 | secs_per_pg | 4096 / LBA_SIZE | MAPPING_UNIT / LBA_SIZE | FTL 매핑 단위(= FTL page) 변경 |
| ssd_config.h | BLKS_PER_PLN | 8192 | 384 | 블록 2 MiB: 12 GiB ÷ 4 파티션 ÷ (2 ch × 2 LUN × 1 plane) ÷ 384 |
| ssd_config.h | FLASH_PAGE_SIZE | KB(32) | 32 KiB (매핑 ≤ 32K) = 매핑 단위 (64K, 128K) | FLASH_PAGE_SIZE % pgsz == 0 assert 통과 |
| ssd.c | 적재 로그 | – | 'KSC2026: mapping unit=…' 1 줄 | 회차마다 매핑 단위 적용 확인 |
| conv_ftl.c | 적재 로그 | – | 'KSC2026: WBUF_FIX=… GC_STATS=…' 1 줄 (#if 밖, 항상 출력) | 회차마다 빌드 스위치 확인 |
| conv_ftl.c/.h | GC_STATS (선택) | – | 모든 측정 빌드에 켬 | 첫 GC 시각, 호스트/GC 페이지 수 기록 (관찰만, 영향 없음 — 5.3 절) |
| conv_ftl.c | WBUF_FIX (선택) | – | 모든 측정 빌드에 켬 (wbuffix, merge) | 쓰기 버퍼 과다 반환 수정 (5.4 절) |
| conv_ftl.c/.h, Kbuild | WBUF_MERGE (선택) | – | merge 만 켬 (순차 쓰기 실험) | 매핑 단위보다 작은 쓰기를 쓰기 버퍼에서 병합 (8.2 절) |

빌드 명령 (매핑 단위 하나):

cd nvmevirt

make clean

make MAPPING_UNIT=16384 GC_STATS=1 WBUF_FIX=1   # wbuffix 변형 (모든 랜덤·순차 쓰기 측정)

make MAPPING_UNIT=4096 GC_STATS=1               # base 변형: 원본 쓰기 버퍼 계산. 측정에 쓰지 않는다 (5.3 절의 계측 영향 확인에만 사용)

make MAPPING_UNIT=16384 GC_STATS=1 WBUF_FIX=1 WBUF_MERGE=1   # merge 변형 (순차 쓰기 실험, 커밋 66446ea 부터)

# exp/build_modules.sh <변형> 이 6 개 매핑 단위를 차례로 빌드해 exp/modules/<변형>/ 에 보관한다

# 빌드된 .ko 에는 소스의 절대 경로가 들어가므로, SHA-256 비교는 같은 경로에서 빌드한 경우에만 의미가 있다 (원본 값: 4.2 절)

## 3.5 매핑 단위별 FTL 파라미터 (적재 로그로 확인한 값)

표 10. 매핑 단위별 파라미터 (dmesg 적재 로그)

| 매핑 단위 | flash page (KiB) | 블록당 페이지 | 블록 | 파티션당 line | 전체 FTL 페이지 | L2P 표 (MiB) | 쓰기 버퍼 | 호스트 용량 (MiB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 32 | 512 | 2 MiB | 384 | 3,145,728 | 24.00 | 1 MiB | 11483 |
| 8K | 32 | 256 | 2 MiB | 384 | 1,572,864 | 12.00 | 1 MiB | 11483 |
| 16K | 32 | 128 | 2 MiB | 384 | 786,432 | 6.00 | 1 MiB | 11483 |
| 32K | 32 | 64 | 2 MiB | 384 | 393,216 | 3.00 | 1 MiB | 11483 |
| 64K | 64 | 32 | 2 MiB | 384 | 196,608 | 1.50 | 2 MiB | 11483 |
| 128K | 128 | 16 | 2 MiB | 384 | 98,304 | 0.75 | 4 MiB | 11483 |

공통: 파티션 4 개(SSD_PARTITIONS), 파티션마다 채널 2 · 채널당 LUN 2 · LUN 당 plane 1, 파티션 물리 용량 3 GiB, line = 블록 4 개 = 8 MiB, 물리 12,883,853,312 B / 논리 12,040,984,403 B (physical/logical × 100 = 107), 장치 크기 12,040,984,064 B. L2P 표 크기 = 전체 FTL 페이지 × 8 B (maptbl 항목 크기) — 매핑 단위가 커질수록 1/2 씩 줄어든다. 쓰기 버퍼(GLOBAL_WB_SIZE)는 원본 식 '채널 × LUN × oneshot page × 2' 를 그대로 써서 flash page 가 커지는 64K·128K 에서 2·4 MiB 가 된다.

## 3.6 바꾸지 않은 기본값 (970 Pro 묶음)

NVMeVirt 설정조사 보고서(산출물/NVMeVirt_설정조사_보고서.docx, 부록 A)가 정리한 기본값을 그대로 썼다.

표 11. 유지한 NVMeVirt 기본값

| 항목 | 값 | 위치 |
| --- | --- | --- |
| NAND 채널 / 채널당 LUN / LUN 당 plane | 8 / 2 / 1 (총 16 die) | ssd_config.h NAND_CHANNELS, LUNS_PER_NAND_CH, PLNS_PER_LUN |
| SSD 파티션 | 4 (채널을 4 등분, 파티션 = 독립 FTL 인스턴스, LPN % 4 로 분배) | SSD_PARTITIONS |
| 셀 종류 | MLC | CELL_MODE |
| oneshot page | flash page × 1 | ONESHOT_PAGE_SIZE |
| 읽기 지연 tR | LSB 30.0 µs / MSB 42.0 µs (4 KiB 읽기 29.8 / 41.8 µs) | NAND_READ_LATENCY_*, NAND_4KB_READ_LATENCY_* |
| 프로그램 지연 tPROG | 185 µs | NAND_PROG_LATENCY |
| 지우기 지연 tBERS | 0 | NAND_ERASE_LATENCY |
| 펌웨어 지연 | 4 KiB 읽기 21.5 µs, 읽기 30.49 µs, 쓰기 버퍼 4.0 µs + 0.46 µs/4 KiB, 채널 0 | FW_* |
| 채널 / PCIe 대역폭 | 800 MB/s / 3,360 MB/s | NAND_CHANNEL_BANDWIDTH, PCIE_BANDWIDTH |
| 채널 1 회 전송 최대 | 16 KiB | MAX_CH_XFER_SIZE |
| OP 비율 | 7 % | OP_AREA_PERCENT |
| 쓰기 버퍼 | 채널 × LUN × oneshot × 2 (1 MiB @ 32 KiB page) | GLOBAL_WB_SIZE |
| 쓰기 조기 완료 | 켬 (버퍼에 들어가면 완료 응답) | WRITE_EARLY_COMPLETION 1 |
| MDTS | 6 (최대 전송 256 KiB → 128 KiB 요청은 나뉘지 않음) | MDTS |
| GC 문턱 | free line ≤ 2 이면 foreground GC (line 1 개씩) | conv_ftl.c gc_thres_lines(_high) = 2 |
| GC 지연 모델 | 켬 | enable_gc_delay = 1 |
| LBA 크기 | 512 B | LBA_BITS 9 |

## 3.7 설정조사 보고서 추천값과 다른 점

표 12. 설정조사 보고서 추천값과의 차이

| 항목 | 보고서 추천 | 이번 실험 | 이유 |
| --- | --- | --- | --- |
| 용량 · memmap | 64 GiB | 12 GiB (memmap=12G$12G) | 서버 메모리 24 GB 중 상위 12 GB 사용 (사용자 지정) |
| 블록 | 2 MiB (BLKS_PER_PLN 2048 @ 64 GiB) | 2 MiB (BLKS_PER_PLN 384 @ 12 GiB) | 용량에 맞춰 같은 2 MiB 유지 |
| NVMeVirt CPU | 5 (디스패처 1 + 워커 4) | 3 (디스패처 1 + 워커 2) | 서버 CPU 6 개 (사용자 지정) |
| 매핑 단위 | 4 → 8·16·32 KiB | 최종 4·16·32 KiB (첫 설계는 4–128 KiB, 64K·128K 는 flash page 도 같이 키움) | 사용자·선배 지정 |
| fio bs | 4k–128k | 최종 4·16·32 KiB (첫 설계는 4–128 KiB) | 선배 지정 |
| fio 시간 | 사전 채움 후 300 s | 포맷 직후 60 s, ramp_time 0, 3 회 | GC 이전 구간까지 함께 측정 (사용자 지정) |
| fio 나머지 | libaio · randwrite · QD 32 · jobs 1 | 같음 | – |
| OS 페이지 캐시 | 언급 없음 | drop / no-drop 두 조건 측정 | 선배 질문 |

## 3.8 모듈 적재 파라미터

sudo insmod exp/modules/<변형>/nvmev_map<단위>.ko memmap_start=12G memmap_size=12G cpus=3,4,5

memmap_start/size 는 GRUB 의 memmap=12G$12G 와 같은 영역이다. NVMeVirt 는 앞 1 MiB 를 큐 등에 쓰고 나머지 12,287 MiB 를 저장 공간으로 쓴다(dmesg 'Storage: 0x300100000-0x600000000 (12287 MiB)'). cpus 의 첫 값이 디스패처, 나머지가 I/O 워커다.

## 3.9 fio 작업 파일

틀(exp/jobs/randwrite.fio.in)의 @값@ 을 회차마다 채워 회차 폴더에 job.fio 로 저장한다. 아래는 틀과 실제 예(매핑 4K, bs 4K, 1 회차)이다.

; KSC2026 NVMeVirt mapping-unit experiment — fresh-device random write.

; Template: run_experiment.sh replaces @VARS@ and saves the concrete job file as <run dir>/job.fio

[global]

filename=@DEV@

ioengine=libaio

direct=1

rw=randwrite

bs=@BS@

iodepth=@IODEPTH@

numjobs=1

time_based=1

runtime=@RUNTIME@

ramp_time=@RAMP@

group_reporting=1

; same random offset sequence in every run (fio default, written out for clarity)

randrepeat=1

percentile_list=50:90:95:99:99.9:99.99

; time series (averaged over @LOG_MSEC@ ms) to see the pre-GC and post-GC parts of the run

write_bw_log=@LOG_PREFIX@

write_iops_log=@LOG_PREFIX@

write_lat_log=@LOG_PREFIX@

log_avg_msec=@LOG_MSEC@

[randwrite]

; KSC2026 NVMeVirt mapping-unit experiment — fresh-device random write.

; Template: run_experiment.sh replaces @VARS@ and saves the concrete job file as <run dir>/job.fio

[global]

filename=/dev/nvme1n1

ioengine=libaio

direct=1

rw=randwrite

bs=4k

iodepth=32

numjobs=1

time_based=1

runtime=60

ramp_time=0

group_reporting=1

; same random offset sequence in every run (fio default, written out for clarity)

randrepeat=1

percentile_list=50:90:95:99:99.9:99.99

; time series (averaged over 500 ms) to see the pre-GC and post-GC parts of the run

write_bw_log=/home/dccearth/jsw/KSC2026/nvmevirt/exp/results/main3x3_20261008/wbuffix_nodrop/map4k_bs4k_r1/fio

write_iops_log=/home/dccearth/jsw/KSC2026/nvmevirt/exp/results/main3x3_20261008/wbuffix_nodrop/map4k_bs4k_r1/fio

write_lat_log=/home/dccearth/jsw/KSC2026/nvmevirt/exp/results/main3x3_20261008/wbuffix_nodrop/map4k_bs4k_r1/fio

log_avg_msec=500

[randwrite]

filename 은 회차마다 모델명 CSL_Virt 로 찾은 장치다. 크기(11–12 GiB)·파티션 없음·마운트 안 됨·루트 디스크 아님을 확인한 뒤에만 쓴다.

size 를 주지 않아 장치 전체(12,040,984,064 B)에 랜덤 쓰기를 한다. randrepeat=1(기본값)이라 회차·조합마다 같은 난수 순서를 쓴다.

norandommap 은 기본값(0)이다: fio 가 한 바퀴 동안 같은 블록을 두 번 쓰지 않는다.

write_bw_log/write_iops_log/write_lat_log 로 0.5 s 평균 시계열을 남긴다(fio_bw.1.log 등, 단위 KiB/s · IOPS · ns).

순차 쓰기 실험(8 절)의 틀 exp/jobs/seqwrite.fio.in 은 위와 비교해 rw=write, randrepeat 줄 삭제, 작업 이름 [seqwrite] 만 다르다. time_based 라 장치 끝(12,040,984,064 B)에 닿으면 offset 0 으로 돌아가 계속 쓴다.

; KSC2026 NVMeVirt mapping-unit experiment — fresh-device sequential write.

; Template: run_experiment.sh replaces @VARS@ and saves the concrete job file as <run dir>/job.fio

[global]

filename=@DEV@

ioengine=libaio

direct=1

rw=write

; time_based: after the last block of the device fio wraps around to offset 0 and keeps writing sequentially

bs=@BS@

iodepth=@IODEPTH@

numjobs=1

time_based=1

runtime=@RUNTIME@

ramp_time=@RAMP@

group_reporting=1

percentile_list=50:90:95:99:99.9:99.99

; time series (averaged over @LOG_MSEC@ ms) to see the pre-GC and post-GC parts of the run

write_bw_log=@LOG_PREFIX@

write_iops_log=@LOG_PREFIX@

write_lat_log=@LOG_PREFIX@

log_avg_msec=@LOG_MSEC@

[seqwrite]

## 3.10 OS 페이지 캐시 조건 (drop / nodrop)

drop 조건은 장치 확인 직후(5 초 대기 전) 다음을 실행한다. nodrop 조건은 아무것도 하지 않는다. 두 조건 모두 /proc/meminfo 의 MemFree·Buffers·Cached·Dirty·Writeback 을 drop 전, drop 후, fio 직전에 회차 폴더의 cache.txt 에 기록한다.

sync

echo 3 | sudo -n tee /proc/sys/vm/drop_caches > /dev/null      # 3 = page cache + dentries/inodes

fio 는 direct=1(O_DIRECT)로 블록 장치에 쓰므로 데이터 경로가 페이지 캐시를 거치지 않는다. NVMeVirt 저장 공간은 GRUB memmap 으로 커널 관리에서 뺀 메모리이고, 회차마다 rmmod/insmod 로 장치를 새로 만든다. 따라서 이론상 영향이 없어야 하며, 이를 실측으로 확인하는 것이 이 조건의 목적이다.

# 4. 실험 절차

## 4.1 회차 하나의 절차

nvmev 모듈이 올라와 있으면 rmmod 로 내린다(이전 FTL 상태 제거).

커널 로그를 끝까지 따라가며 기록하는 'sudo dmesg -W' 를 회차 폴더의 kernel.log 로 시작한다(채널 모델 오류 줄은 개수만 센다 — 5.4 절).

커널 로그에 'KSC2026-MARK <회차> insmod' 표지를 남기고 insmod 한다(memmap_start=12G memmap_size=12G cpus=3,4,5).

모델명 CSL_Virt_MN_01 인 블록 장치가 생길 때까지 기다리고, 적재 로그의 'KSC2026: mapping unit=<바이트> B' 로 매핑 단위가 맞는지 확인한다. 틀리면 실험을 멈춘다.

장치 크기·파티션·마운트·루트 디스크 여부를 확인한다. 페이지 캐시 조건이 drop 이면 sync; echo 3 > /proc/sys/vm/drop_caches 를 실행하고, 두 조건 모두 meminfo 를 cache.txt 에 적는다(3.10 절). job.fio 와 meta.txt 를 만든다.

5 초 쉰 뒤 'fio-start' 표지를 남기고 fio 를 60 초 실행한다(JSON 결과 + 0.5 초 시계열).

'fio-end', 'rmmod' 표지를 남기고 rmmod 한다 — 이때 GC 통계(파티션별 호스트/GC 페이지 수)가 로그에 찍힌다.

커널 로그를 run 구간(dmesg_run.txt)과 rmmod 구간(dmesg_unload.txt)으로 나누고, 결과 파일 소유자를 사용자로 바꾼다.

fio 종료 코드와 JSON 의 error 필드가 모두 0 이면 DONE 표지를, 아니면 FAILED 표지(원인 커널 줄 포함)를 만들고 다음 회차로 넘어간다. DONE·FAILED 회차는 다시 실행할 때 건너뛴다. fio 는 감시 타이머(60 s + 180 s 에 SIGTERM, 다시 60 s 뒤 SIGKILL) 아래에서 실행한다.

## 4.2 실행 순서와 시간

표 13. 실행 순서와 시각 (KST)

| 데이터셋 · 변형 | 완료 회차 | 순서 | run.log 첫 시각 | 마지막 시각 |
| --- | --- | --- | --- | --- |
| 주 · wbuffix_drop | 27 | 반복 1→3, 매핑 4K→32K, bs 4K→32K, (nodrop, drop) 연달아 — 짝수 반복은 drop 먼저 | 2026-10-08 16:00:35 | 2026-10-08 17:03:57 |
| 주 · wbuffix_nodrop | 27 | 반복 1→3, 매핑 4K→32K, bs 4K→32K, (nodrop, drop) 연달아 — 짝수 반복은 drop 먼저 | 2026-10-08 16:00:35 | 2026-10-08 17:03:57 |
| 보조 · wbuffix | 53 | 반복 1→3, 매핑 4K→128K, bs 4K→128K | 2026-10-08 14:49:14 | 2026-10-08 15:51:36 |
| 순차 · wbuffix | 27 | 반복 r 마다 wbuffix 한 바퀴 → merge 한 바퀴 (바퀴 안: 매핑 4K→32K, bs 4K→32K) | 2026-10-08 20:23:24 | 2026-10-08 21:16:12 |
| 순차 · merge | 27 | 반복 r 마다 wbuffix 한 바퀴 → merge 한 바퀴 (바퀴 안: 매핑 4K→32K, bs 4K→32K) | 2026-10-08 20:33:58 | 2026-10-08 21:26:45 |
| 랜덤 bs 8K·64K · wbuffix | 18 | 반복 1→3, 매핑 4K→32K, bs 8K→64K | 2026-10-08 22:11:48 | 2026-10-08 22:32:55 |

반복을 바깥 루프에 둬서 시간에 따른 서버 상태 변화가 특정 조합에 몰리지 않게 했다. 회차 하나는 적재·대기·측정·내림을 합쳐 약 70 초 걸린다.

보조 데이터셋 경과: 첫 설계는 14:20:28 에 시작했다(모듈 빌드 14:20:28–14:21:02). 쓰기 버퍼 계산을 고친 모델(wbuffix)의 회차는 `bash run_experiment.sh main_20261008 wbuffix` 로 14:49:14 에 시작했고(run.log 첫 줄; 스크립트 커밋 2a462a3 → push 전 949ae38 로 다시 만듦, 10 절), 15:51:23 에 설계 변경으로 회차 경계에서 멈췄다. 모듈은 14:20 에 한 번 빌드한 것을 끝까지(주 데이터셋 포함) 그대로 썼다(SHA-256 확인). 14:21–14:47 에는 원본 모델(base)을 돌렸는데, 그 결과는 사용자 요청으로 뺐다(10 절).

주 데이터셋에 쓴 wbuffix 모듈은 커밋 5769378 상태의 /home/dccearth/jsw/KSC2026/nvmevirt/nvmevirt 에서 make MAPPING_UNIT=<바이트> GC_STATS=1 WBUF_FIX=1 로 빌드했다(2026-10-08 14:20:45–14:21:02 KST, gcc 13.3.0, 커널 헤더 6.8.0-142). 쓴 모듈의 SHA-256: nvmev_map4k.ko b522a8b1306eabaedf57b4f749cf4a13776e6882ec5be7162e17b5defec5120e; nvmev_map16k.ko 033e5343ab408069cbd4c238162036a7d9ef9a49162c45232a6f67b3ea283bc9; nvmev_map32k.ko b02b7c04cfc2ce20a0bf8ea09cf8d036e19e152d27a0824bae434d671f833d8a. 6 개 단위 전체는 결과 폴더의 modules_SHA256SUMS 와 회차별 meta.txt 의 module 줄에 있다(.ko 는 git 에 넣지 않았다).

## 4.3 재현 명령

git clone https://github.com/Sangwon8799/nvmevirt.git nvmevirt && cd nvmevirt   # 공개 저장소 (SSH 키 불필요)

git checkout ksc2026-v3   # 태그 = 실험 스크립트 e598e75 + 순차 쓰기 스크립트·병합 모델 66446ea + 랜덤 bs 8K·64K 스크립트 a86fc87 + 이 문서·생성기·결과

# 이 커밋에는 결과(exp/results/)도 들어 있다. 아래 명령을 같은 실험 이름으로 실행하면 DONE 표지가 있는 회차는 건너뛴다.

# 새로 재려면 실험 이름을 바꾼다(예: main3x3_<날짜>, seq3x3_<날짜>, randbs_<날짜>).

cd exp

python3 -m venv .venv && ./.venv/bin/pip install -r requirements.txt

bash build_modules.sh wbuffix   # make MAPPING_UNIT=<바이트> GC_STATS=1 WBUF_FIX=1 (매핑 단위 6 개). .ko 는 git 에 없으므로 clone 뒤에는 빌드가 필요하다

# 3.1 절 GRUB 설정·패키지가 있어야 한다. 3.3 절 sudoers 는 이 clone 의 경로와 내 사용자 이름으로 바꿔 설치한다:

sed -e "s|/home/dccearth/jsw/nvmevirt/exp/\*|$(cd .. && pwd)/exp/*|" -e "s|dccearth|$(id -un)|g" report/nvmevirt-exp.sudoers > /tmp/nvmevirt-exp.sudoers   # 장치 이름이 nvme1n1 이 아니면 nomerges 경로도 바꾼다

sudo visudo -cf /tmp/nvmevirt-exp.sudoers && sudo install -m 0440 -o root -g root /tmp/nvmevirt-exp.sudoers /etc/sudoers.d/nvmevirt-exp

tmux new -s ksc2026 'bash run_all.sh main3x3_20261008 2>&1 | tee -a results/run_all_main3x3_20261008.log'   # 주 데이터셋 (약 65 분)

tmux new -s ksc6x6 'bash run_all_6x6.sh main_20261008 2>&1 | tee -a results/run_all_main_20261008.log'   # (선택) 첫 설계 6×6×3, wbuffix (약 2.1 시간)

# 사전 점검 (5.3·5.6 절; 당시에는 대기 시간을 줄여 실행. 폴더 이름은 실험 후 pre_* 로 바꿈)

MAPS=4k bash build_modules.sh base && MAPS=4k bash build_modules.sh plain   # 5.3 전용: 원본 쓰기 버퍼 계산 + GC_STATS 켬/끔 (bs = 매핑 단위에서만 쓴다)

for v in plain base; do MAPS=4k BSS=4k REPS=3 RUNTIME=20 SETTLE_SEC=3 bash run_experiment.sh pre_gcstats_ab $v; done         # 5.3

MAPS="4k 16k" BSS=4k REPS=2 RUNTIME=8 SETTLE_SEC=2 CACHE_MODES="nodrop drop" bash run_experiment.sh pre_cache_test wbuffix   # 5.6

./.venv/bin/python plot.py all --exp results/main3x3_20261008    # 그래프

# 순차 쓰기 실험 (8 절): wbuffix 모듈(위에서 빌드) + merge 모듈, 3×3×3×2 = 54 회 (약 64 분)

MAPS="4k 16k 32k" bash build_modules.sh merge

tmux new -s ksc2026seq 'bash run_all_seq.sh seq3x3_20261008 2>&1 | tee -a results/run_all_seq3x3_20261008.log'

# 순차 쓰기 사전 점검 (8.3 절; 15 s 회차)

export REPS=1 RUNTIME=15 CACHE_MODES=nodrop

WORKLOAD=seqwrite NOMERGES= MAPS=16k BSS=4k bash run_experiment.sh pre_seq_blkmerge_default wbuffix

for v in wbuffix merge; do WORKLOAD=seqwrite NOMERGES=2 MAPS="16k 32k" BSS="4k 16k" bash run_experiment.sh pre_seq_check $v; done

for v in wbuffix merge; do WORKLOAD=randwrite NOMERGES= MAPS=16k BSS=4k bash run_experiment.sh pre_rand_merge_check $v; done

unset REPS RUNTIME CACHE_MODES

./.venv/bin/python plot.py all --exp results/seq3x3_20261008

# 랜덤 쓰기 bs 8K·64K (9 절): wbuffix 모듈(위에서 빌드), 매핑 3 × bs 2 × 3 = 18 회 (약 21 분) → 주 데이터셋과 합친 보기

tmux new -s ksc2026rbs 'bash run_all_rand_bs.sh randbs_20261008 main3x3_20261008 rand3x5_20261008 2>&1 | tee -a results/run_all_randbs_20261008.log'

./.venv/bin/python plot.py all --exp results/rand3x5_20261008

./.venv/bin/python report/make_report.py results/main3x3_20261008 --supp results/main_20261008 --seq results/seq3x3_20261008 --rbs results/randbs_20261008 --rview results/rand3x5_20261008   # 이 문서 (선택 단계를 건너뛰었으면 해당 인자 생략)

위 sed 는 3.3 절 규칙의 chown 경로와 사용자 이름을 바꾼다. `MAPS=4k bash build_modules.sh base` 는 modules/base/SHA256SUMS 를 4K 한 줄로 다시 쓴다.

## 4.4 결과 파일

표 14. 회차 폴더의 파일

| 파일 (회차 폴더 exp/results/<EXP>/<변형>_<nodrop¦drop>/map<단위>_bs<크기>_r<회>/ — CACHE_MODES=nodrop 단독이면 <변형>/map…/) | 내용 |
| --- | --- |
| fio.json | fio 결과 (JSON) — BW, IOPS, clat/slat/lat 통계, 백분위수 |
| fio_stdout.txt | fio 표준 출력·오류와 감시 타이머 기록 (정상 회차는 비어 있음; 실패 회차에는 I/O error 줄) |
| fio_bw.1.log / fio_iops.1.log / fio_lat.1.log / fio_clat.1.log / fio_slat.1.log | 0.5 s 평균 시계열 (ms, 값, 방향, bs, offset) |
| job.fio | 그 회차에 실제로 쓴 fio 작업 파일 |
| meta.txt | 변형·캐시 조건·단위·bs·회차, 장치, 모듈 SHA-256, insmod 인자, git 커밋, 시작/끝(UTC), fio 종료 코드, 채널 모델 오류 수, 커널 경고 수. 순차 쓰기 실험부터: workload(rw), blk_queue(nomerges·스케줄러·max_sectors_kb·write_cache), blk_writes(fio 동안 블록 계층 쓰기 요청 수·합쳐진 수·섹터 수) |
| cache.txt | 페이지 캐시 조건과 meminfo (drop 전/후, fio 직전) |
| DONE / FAILED | 정상 종료 표지 / fio 실패 표지(원인 커널 줄 포함) |
| dmesg_load.txt | insmod 직후 커널 로그 (NVMeVirt 구성·파라미터) |
| dmesg_run.txt | fio 실행 중 커널 로그 (첫 GC 줄 포함, 채널 모델 오류 줄 제외) |
| dmesg_unload.txt | rmmod 시 커널 로그 (파티션별 GC 통계; merge 변형은 파티션별 병합 통계도) |
| kernel.log / chmodel_msgs.txt | 회차 전체 커널 로그 (채널 모델 오류는 처음 20 줄만) / 그 오류 줄 수 |

변형 폴더(<변형>_<nodrop|drop>/ 또는 <변형>/)에는 run.log, git_head.txt, modules_SHA256SUMS, modules_build_info.txt, job 틀 <WORKLOAD>.fio.in(랜덤 쓰기 randwrite.fio.in, 순차 쓰기 seqwrite.fio.in), nvmevirt_vs_upstream.diff, nvmevirt_uncommitted.diff 가 있다. 실험 폴더(exp/results/<EXP>/)에는 env_before/, env_after_<변형>/ (보조 데이터셋은 중단 후 만든 env_after_stop/), analysis/(analyze.py), plots/(plot.py) 가 있다. 원자료의 시각은 서버 시계 UTC 다.

## 4.5 지표 정의

BW, IOPS: fio JSON 의 60 s 평균 (bw 는 KiB/s → MiB/s).

clat: 제출 후 완료까지 지연 (fio clat_ns). 평균과 p50/p99/p99.9.

첫 10 s / 마지막 20 s BW: 0.5 s 시계열에서 0–10 s, 40–60 s 구간 평균.

GC 시작 시각: 커널 로그의 'fio-start' 표지부터 첫 'KSC2026: first GC' 줄까지의 시간 (4 개 파티션 중 가장 이른 것).

GC 전/후 BW: GC 시작 시각 이전/이후 시계열 구간의 평균.

WAF_GC: (호스트 페이지 쓰기 + GC 페이지 복사) ÷ 호스트 페이지 쓰기 — 매핑 단위 페이지 기준.

WAF_total: (호스트 + GC 페이지) × 매핑 단위 ÷ fio 가 쓴 바이트 — 매핑 단위보다 작은 쓰기의 증폭(매핑 단위/bs)까지 포함.

블록 계층 요청 (순차 쓰기 실험): fio 동안 /sys/block/<장치>/stat 의 쓰기 요청 수·합쳐진 수·평균 요청 크기(KiB).

병합 통계 (merge 변형): open = 부분 쓰기로 연 매핑 단위 수, merge = 열린 단위에 합류한 쓰기 수, full = 가득 차서 flash 에 쓴 단위 수, evict = 다른 부분 쓰기에 자리를 내주며 덜 찬 채 쓴 단위 수, direct = 단위 전체를 덮어 바로 쓴 수, still_open = rmmod 때 열려 있던 단위 수 (4 파티션 합).

# 5. 사전 점검과 발견 사항

## 5.1 예약 메모리와 장치 확인

$ sudo cat /proc/iomem    (4 GiB 위)

100000000-2ffffffff : System RAM      #  4 – 12 GiB

300000000-5ffffffff : Reserved        # 12 – 24 GiB  ← NVMeVirt 저장 공간

600000000-67f7fffff : System RAM      # 24 – 26 GiB (PCI hole 재배치분)

$ dmesg (insmod 직후, 매핑 4K)

NVMeVirt: Version 1.10 for >> Samsung 970 Pro SSD <<

NVMeVirt: Storage: 0x300100000-0x600000000 (12287 MiB)

NVMeVirt: Total Capacity(GiB,MiB)=3,3072 chs=2 luns=4 lines=384 blk-size(MiB,KiB)=2,2048 line-size(MiB,KiB)=8,8192

NVMeVirt: KSC2026: mapping unit=4096 B, flash page=32768 B, oneshot page=32768 B, pgs_per_blk=512, blks_per_pl=384, write buffer=1048576 B

NVMeVirt: FTL physical space: 12883853312, logical space: 12040984403 (physical/logical * 100 = 107)

NVMeVirt: ns 0/1: size 11483 MiB

NVMeVirt: nvmev_io_worker_0 started on cpu 4 (node 0)

NVMeVirt: nvmev_io_worker_1 started on cpu 5 (node 0)

NVMeVirt: nvmev_dispatcher started on cpu 3 (node 0)

nvme nvme1: 1/0/0 default/read/poll queues

매핑 단위 6 개 모두 init 단계의 assert 를 통과하고 블록은 정확히 2 MiB 가 된다(블록 원시 크기 2,096,982 B 가 oneshot page 단위로 올림되어 2,097,152 B). 64K·128K 는 FLASH_PAGE_SIZE 를 그대로 32 KiB 로 두면 ssd.c:98 의 assert 로 적재가 실패한다(사용자가 지적한 대로 flash page 를 매핑 단위와 같게 함).

## 5.2 스모크 테스트

본 실험 전에 10 초짜리 짧은 실행(매핑 4K·128K × bs 4K·128K)으로 회차 절차·결과 파일·커널 로그 수집이 끝까지 도는지 확인했다. 이 과정에서 두 가지를 고쳤다: (1) dmesg 시각과 /proc/uptime 의 차이 때문에 시각으로 로그를 자르던 방법이 실패해 커널 로그 표지(/dev/kmsg) 방식으로 바꿨고, (2) NVMeVirt 의 '[chmodel_request] Need to increase array size' 오류가 한꺼번에 많이 찍히면 256 KiB 커널 링 버퍼가 덮어써질 수 있어, 커널 로그를 실시간으로 따라가며 그 줄은 개수만 세도록 했다(모든 본 실험 회차에서 이 수는 0 이다). 스모크 테스트는 대부분 쓰기 버퍼 계산을 고치지 않은 원본 모델로 돌렸고, 3 차에는 wbuffix 도 함께 돌렸다. 원본 모델로 잰 측정값과 결과 폴더는 남기지 않았다.

## 5.3 GC_STATS 계측의 영향 확인

표 15. GC_STATS 계측 유무 비교 (매핑 4K, bs 4K, 20 s, 3 회, insmod 후 대기 3 s)

| 빌드 | 회 | MiB/s | IOPS | clat 평균 µs |
| --- | --- | --- | --- | --- |
| plain (GC_STATS 끔) | 1 | 724.1 | 185,361 | 171.6 |
| plain (GC_STATS 끔) | 2 | 721.9 | 184,807 | 172.1 |
| plain (GC_STATS 끔) | 3 | 722.1 | 184,868 | 172.1 |
| base (GC_STATS 켬) | 1 | 724.3 | 185,428 | 171.5 |
| base (GC_STATS 켬) | 2 | 726.0 | 185,864 | 171.1 |
| base (GC_STATS 켬) | 3 | 724.4 | 185,444 | 171.5 |

GC 통계(첫 GC 시각, 호스트/GC 페이지 수)를 남기는 계측은 디스패처 스레드에서 정수 증가 몇 번과 첫 GC 때 printk 1 줄만 더한다. 요청 수가 가장 많아 디스패처의 요청당 작업이 결과에 가장 잘 드러나는 4K/4K 에서 계측을 끈 빌드와 켠 빌드를 20 초씩 3 회 비교했다. 평균 차이(+0.31 %, 계측을 켠 빌드가 오히려 빠름)가 반복 간 범위(끈 빌드 0.30 %, 켠 빌드 0.23 %)와 같은 수준이어서 계측 비용이 드러나지 않음을 확인한 뒤 모든 측정 빌드에 켰다. 이 비교의 두 빌드(plain·base)는 원본 쓰기 버퍼 계산이다. 4K/4K 는 bs = 매핑 단위라 할당량이 wbuffix 와 같으므로(5.4 절) 이 결과는 wbuffix 에도 그대로 적용된다.

## 5.4 발견 — 원본 코드는 매핑 단위보다 작은 쓰기에서 쓰기 버퍼를 과다 반환한다

NVMeVirt conventional SSD 의 conv_write() 는 요청 크기만큼 쓰기 버퍼를 할당하고(buffer_allocate(wbuf, LBA_TO_BYTE(nr_lba))), flash page(wordline)가 다 차서 프로그램될 때 'oneshot page 크기'만큼 반환한다(schedule_internal_operation(…, pgs_per_oneshotpg × pgsz)). bs ≥ 매핑 단위이면 둘이 같지만, bs < 매핑 단위이면 4 KiB 쓰기 하나가 매핑 단위 페이지 하나를 통째로 차지하므로 할당(bs)보다 반환(매핑 단위)이 많다. 예: 매핑 128K·bs 4K 는 요청마다 4 KiB 할당, 124 KiB 초과 반환.

그 결과 버퍼의 남은 양이 끝없이 늘어 쓰기 버퍼가 호스트를 붙잡는 역할(back-pressure)을 못 한다. 쓰기 조기 완료(WRITE_EARLY_COMPLETION=1)라 NAND·GC 지연이 호스트에 전달되는 통로는 이 버퍼뿐이다.

NAND 작업이 실제 시간보다 계속 앞서 쌓이고, 채널 모델의 시간 창(96K × 4 µs ≈ 393 ms)을 넘으면 NVMeVirt 가 채널 전송 시간을 0 으로 처리하고 요청마다 '[chmodel_request] Need to increase array size' 를 printk 한다(rate limit 없음). 콘솔 로그 레벨 4 라 이 오류는 tty0 콘솔과 systemd-journald 에도 기록된다.

따라서 원본 계산으로는 bs < 매핑 단위인 조합의 타이밍 모델이 정상 범위를 벗어난다. 5.3 절의 계측 영향 확인(bs = 매핑 단위, plain·base)을 뺀 이 문서의 모든 결과는 아래 수정을 적용한 모델로 쟀다.

코드 감사 에이전트 세 개(쓰기 경로·버퍼, 초기화·기하, GC·타이밍 렌즈)가 모두 독립적으로 같은 결론을 냈고(부록 E), 반박을 시도한 검증 6 회가 모두 핵심을 확인했다(세 건은 세부 수치·범위만 정정).

수정(WBUF_FIX=1, wbuffix 변형): 할당량을 실제로 프로그램될 매핑 단위 페이지 수에 맞춘다. PCIe·펌웨어 전송 시간 계산에는 원래대로 요청 크기를 쓴다. bs ≥ 매핑 단위인 조합에서는 할당량이 원본과 같다.

#if KSC_WBUF_FIX

/* KSC2026: a write occupies whole mapping units in the write buffer; this is also what

* schedule_internal_operation() releases once the flash page is programmed */

wbuf_bytes = (end_lpn - start_lpn + 1) * spp->pgsz;

#else

wbuf_bytes = LBA_TO_BYTE(nr_lba);

#endif

allocated_buf_size = buffer_allocate(wbuf, wbuf_bytes);

if (allocated_buf_size < wbuf_bytes)

return false;

## 5.5 결과 해석 시 주의할 모델 특성

표 16. 해석 시 주의할 모델 특성

| 특성 | 내용 | 영향 |
| --- | --- | --- |
| flash page 변경 (64K·128K) | 64K·128K 는 flash page 를 키웠는데 tPROG 는 185 µs 그대로라 die 당 프로그램 대역폭이 32K 대비 1.7·2.6 배가 되고, 쓰기 버퍼도 2·4 MiB 로 커진다. | 32K → 64K 경계의 차이는 매핑 단위만의 효과가 아니다. 64K·128K 의 큰 bs 결과는 더 빠른 NAND 의 영향을 받는다. |
| RMW 미모델 | 매핑 단위보다 작은 쓰기도 옛 페이지를 읽지 않는다(read-modify-write 의 NAND 읽기 없음). 무효화 후 새 페이지 전체를 프로그램만 한다. | 작은 bs·큰 매핑 단위의 불이익이 실제보다 작게 나온다 (wbuffix 에서도). |
| 쓰기 조기 완료 | 호스트 완료 시각은 쓰기 버퍼까지의 시간이다. NAND·GC 비용은 버퍼가 차서 막힐 때만 호스트에 보인다. | GC 영향은 처리량 저하·지연 증가(큐 대기)로 나타난다. |
| 지우기 지연 0 | tBERS = 0 (기본값). | GC 비용이 실제보다 작다 (모든 조합에 같게 적용). |
| 매핑 표 = 호스트 메모리 배열 | DRAM 캐시(DFTL) 모델이 없다. | 매핑 표 크기 감소 효과는 성능에 나타나지 않는다 — 3.5 절의 L2P 크기로 따로 보고. |
| 활성 I/O 워커 1 개 | I/O 큐가 1 개라 cpu4 워커만 일한다. | 논문에는 '디스패처 1 + 워커 2 (활성 1)' 로 적는 것이 정확하다. |
| GC 이전 처리량의 한계 | bs ≥ 매핑 단위이면 GC 전 처리량은 NAND 프로그램 한계(16 die 합 2,233 MiB/s @32K page, 64K 3,805, 128K 5,871) 또는 PCIe 한계(3,357 MiB/s)에 가깝다(4K/4K 실측 2,001 MiB/s = NAND 한계의 90 %). bs < 매핑 단위이면 GC 전 처리량은 그 한계 × bs/매핑 수준이다(6.1 절의 GC 이전 구간 표). | GC 전 구간은 NAND 모델을, GC 후 구간은 GC 모델을 반영한다 — 6 절의 GC 전/후 표로 나눠 본다. |
| fio randommap 주기 현상 | fio 기본값(norandommap=0)은 한 바퀴(11.21 GiB) 동안 같은 블록을 다시 쓰지 않는다. 첫 바퀴에는 무효 페이지가 없어 GC 가 시작되는 순간(둘째 바퀴 약 0.66 GiB 지점) 희생 line 에 유효 페이지가 거의 가득해 대역폭이 급락하고, 둘째 바퀴가 진행될수록 회복하다가 끝 무렵 첫 바퀴의 line 이 모두 무효가 되며 치솟은 뒤 셋째 바퀴에서 다시 떨어진다. | 60 s 평균에 이 주기가 섞인다. 시계열(6.3 절)을 함께 봐야 한다. GC 구간은 정상 상태가 아니다. |
| GC 시작 시각 차이 | 포맷 직후 60 s 라 조합마다 GC 이전 구간 비율이 다르다. | 60 s 평균과 함께 GC 전/후 BW, 시계열을 같이 본다. |
| randrepeat=1 | 모든 회차가 같은 난수 순서를 쓴다. | 3 회 반복은 에뮬레이터 타이밍 편차만 담는다. |
| 작은 쓰기 병합 없음 | 매핑 단위보다 작은 쓰기는 명령마다 매핑 단위 페이지 하나를 새로 쓴다. 같은 단위로 이어서 오는 쓰기도 쓰기 버퍼에서 합치지 않는다(원본 동작이며 wbuffix 도 같다). | 랜덤 쓰기에서는 실제 SSD 도 단위를 새로 써야 하므로 영향이 작다(RMW 읽기만 빠짐). 순차 쓰기에서는 결과를 좌우한다 — 8 절에서 병합 모델(merge)과 함께 본다. |
| 블록 계층 병합 | 커널 기본(mq-deadline, nomerges=0)에서는 인접한 요청을 합친다. | 랜덤 쓰기는 합칠 요청이 거의 없다(15 s 동안 fio 쓰기 1,144,511 개 중 1 개, 8.3 절; bs 8K·64K 회차는 요청의 0.054 % 이하, 9.1 절). 순차 쓰기는 nomerges=2 로 끄고 측정했다. |

## 5.6 페이지 캐시 drop 사전 시험

표 17. 페이지 캐시 drop 사전 시험 (wbuffix, 8 s, 2 회, insmod 후 대기 2 s, 15:57–15:59 KST)

| 조건 | 매핑 | bs | 회 | MiB/s | clat 평균 µs | Cached 전 (MiB) | Cached 후 (MiB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| nodrop | 4K | 4K | 1 | 1553.2 | 79.5 | 288 | 288 |
| nodrop | 4K | 4K | 2 | 1553.2 | 79.5 | 271 | 271 |
| nodrop | 16K | 4K | 1 | 431.4 | 288.7 | 273 | 273 |
| nodrop | 16K | 4K | 2 | 430.8 | 289.1 | 271 | 271 |
| drop | 4K | 4K | 1 | 1550.6 | 79.6 | 289 | 240 |
| drop | 4K | 4K | 2 | 1552.2 | 79.5 | 273 | 240 |
| drop | 16K | 4K | 1 | 431.3 | 288.7 | 273 | 240 |
| drop | 16K | 4K | 2 | 430.8 | 289.1 | 273 | 240 |

본 실험 전에 drop 절차가 동작하는지 확인했다. 시험 직전 서버의 페이지 캐시는 약 10 GB(Cached 10,227,884 kB)였고 drop 후 약 240 MiB 로 줄었다. 그 전 실험들의 커널 로그가 journald 파일로 디스크에 쓰이며 쌓인 것으로 보인다. 8 초 시험에서 drop/no-drop 차이는 조합별 평균으로 4K/4K −0.12 %, 16K/4K −0.005 % 였고, 회차별로는 최대 0.17 %(4K/4K 1 회차: 1553.2 → 1550.6 MiB/s)였다.

# 6. 결과 — 주 데이터셋 (main3x3_20261008)

## 6.1 성능 표 — 페이지 캐시 그대로 (wbuffix_nodrop)

모든 값은 3 회 평균이다(± 는 표본 표준편차). * 는 bs < 매핑 단위 조합이다. drop 조건의 값은 6.2 절과 부록 D 에 있다(조합별 3 회 평균의 차이: 대역폭·IOPS·평균 지연 0.4 % 이내, 첫 GC 시각·GC 이후/마지막 20 s 대역폭·WAF 0.7 % 이내, p99 지연 1 % 이내).

표 18. 쓰기 대역폭 MiB/s, 60 s 평균 — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 416.0 ±1.6 | 481.0 ±0.1 | 482.3 ±0.2 |
| 16K | 129.7 ±0.0 * | 420.4 ±1.3 | 441.6 ±1.3 |
| 32K | 68.3 ±0.0 * | 219.6 ±0.4 * | 424.6 ±0.2 |

표 19. IOPS, 60 s 평균 — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 106,495 | 30,781 | 15,433 |
| 16K | 33,209 * | 26,905 | 14,131 |
| 32K | 17,482 * | 14,052 * | 13,589 |

표 20. 평균 완료 지연 µs — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 299.5 ±1.1 | 1,038 ±0.2 | 2,072 ±0.8 |
| 16K | 962.5 ±0.3 * | 1,188 ±3.6 | 2,263 ±6.7 |
| 32K | 1,829 ±1.3 * | 2,276 ±3.9 * | 2,353 ±1.0 |

표 21. p99 완료 지연 µs — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 13,347 | 17,695 | 18,219 |
| 16K | 14,047 * | 16,843 | 18,219 |
| 32K | 14,483 * | 17,607 * | 17,695 |

표 22. 첫 GC 시각 s (fio 시작 기준) — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 6.32 ±0.04 | 5.69 ±0.00 | 5.69 ±0.00 |
| 16K | 5.91 ±0.00 * | 5.92 ±0.00 | 5.84 ±0.00 |
| 32K | 6.06 ±0.00 * | 6.06 ±0.00 * | 6.10 ±0.00 |

표 23. GC 이전 구간 평균 대역폭 MiB/s — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 2,001 | 2,218 | 2,218 |
| 16K | 535.6 * | 2,143 | 2,173 |
| 32K | 256.1 * | 1,027 * | 2,033 |

표 24. GC 이후 구간 평균 대역폭 MiB/s — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 240.1 | 305.8 | 307.2 |
| 16K | 88.8 * | 246.6 | 267.0 |
| 32K | 47.4 * | 129.9 * | 246.0 |

표 25. 마지막 20 s 평균 대역폭 MiB/s — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 322.0 | 244.8 | 247.2 |
| 16K | 58.2 * | 323.4 | 321.4 |
| 32K | 32.6 * | 82.2 * | 322.8 |

표 26. GC 쓰기 증폭 WAF_GC (매핑 단위 페이지 기준) — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 3.12 | 3.55 | 3.54 |
| 16K | 2.20 * | 3.12 | 3.32 |
| 32K | 2.13 * | 2.98 * | 3.12 |

표 27. 전체 쓰기 증폭 WAF_total (NAND 바이트 ÷ 호스트 바이트) — wbuffix_nodrop

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 3.12 | 3.55 | 3.54 |
| 16K | 8.82 * | 3.12 | 3.32 |
| 32K | 17.08 * | 5.96 * | 3.12 |

그림 1. bs 에 따른 쓰기 대역폭 (매핑 단위별, 평균 ± 표준편차) — wbuffix_nodrop

그림 2. 매핑 단위 × bs 평균 대역폭 — wbuffix_nodrop

그림 3. 평균 완료 지연 (로그 축) — wbuffix_nodrop

그림 4. p99 완료 지연 (로그 축) — wbuffix_nodrop

그림 5. 전체 쓰기 증폭 — wbuffix_nodrop

## 6.2 OS 페이지 캐시 drop 의 영향

같은 (매핑, bs, 회차)에서 연달아 잰 nodrop·drop 두 회차를 짝지어 차이(drop − nodrop)를 보았다. p 는 짝지은 t-검정(양측, 자유도 = 쌍 수 − 1)이다. 맨 아래 '전체' 행은 9 조합 × 3 회 = 27 쌍의 상대 차이를 모은 평균과 95 % 신뢰구간이다.

표 28. 페이지 캐시 drop 영향 — 대역폭 (bw_MiBps)

| 매핑 | bs | 쌍 | nodrop 평균 | drop 평균 | 차이 (drop−nodrop) | 95 % CI | p |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K | 3 | 416.00 | 417.60 | +0.387 % |  | 0.180 |
| 4K | 16K | 3 | 480.96 | 480.83 | -0.028 % |  | 0.379 |
| 4K | 32K | 3 | 482.27 | 482.30 | +0.007 % |  | 0.778 |
| 16K | 4K | 3 | 129.72 | 129.85 | +0.101 % |  | 0.509 |
| 16K | 16K | 3 | 420.39 | 420.33 | -0.013 % |  | 0.935 |
| 16K | 32K | 3 | 441.58 | 440.64 | -0.213 % |  | 0.366 |
| 32K | 4K | 3 | 68.29 | 68.26 | -0.048 % |  | 0.439 |
| 32K | 16K | 3 | 219.57 | 219.51 | -0.025 % |  | 0.856 |
| 32K | 32K | 3 | 424.65 | 424.26 | -0.091 % |  | 0.160 |
| 전체 |  | 27 | – | – | +0.009 % | [-0.084, +0.101] % | 0.850 |

표 29. 페이지 캐시 drop 영향 — 평균 완료 지연 (clat_mean_us)

| 매핑 | bs | 쌍 | nodrop 평균 | drop 평균 | 차이 (drop−nodrop) | 95 % CI | p |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K | 3 | 299.45 | 298.30 | -0.385 % |  | 0.180 |
| 4K | 16K | 3 | 1,038.12 | 1,038.41 | +0.028 % |  | 0.387 |
| 4K | 32K | 3 | 2,071.61 | 2,071.49 | -0.006 % |  | 0.820 |
| 16K | 4K | 3 | 962.52 | 961.54 | -0.102 % |  | 0.501 |
| 16K | 16K | 3 | 1,187.88 | 1,188.03 | +0.014 % |  | 0.936 |
| 16K | 32K | 3 | 2,262.69 | 2,267.52 | +0.214 % |  | 0.368 |
| 32K | 4K | 3 | 1,829.24 | 1,830.13 | +0.048 % |  | 0.442 |
| 32K | 16K | 3 | 2,275.72 | 2,276.24 | +0.023 % |  | 0.866 |
| 32K | 32K | 3 | 2,352.95 | 2,355.09 | +0.091 % |  | 0.159 |
| 전체 |  | 27 | – | – | -0.008 % | [-0.101, +0.084] % | 0.852 |

표 30. 페이지 캐시 drop 영향 — p99 지연 (clat_p99_us)

| 매핑 | bs | 쌍 | nodrop 평균 | drop 평균 | 차이 (drop−nodrop) | 95 % CI | p |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K | 3 | 13,347.50 | 13,391.19 | +0.328 % |  | 0.423 |
| 4K | 16K | 3 | 17,694.72 | 17,694.72 | +0.000 % |  | – |
| 4K | 32K | 3 | 18,219.01 | 18,219.01 | +0.000 % |  | – |
| 16K | 4K | 3 | 14,046.55 | 14,133.93 | +0.623 % |  | 0.184 |
| 16K | 16K | 3 | 16,842.75 | 16,711.68 | -0.775 % |  | 0.184 |
| 16K | 32K | 3 | 18,219.01 | 18,219.01 | +0.000 % |  | – |
| 32K | 4K | 3 | 14,483.46 | 14,396.07 | -0.603 % |  | 0.184 |
| 32K | 16K | 3 | 17,607.34 | 17,519.96 | -0.494 % |  | 0.423 |
| 32K | 32K | 3 | 17,694.72 | 17,519.96 | -0.988 % |  | 0.184 |
| 전체 |  | 27 | – | – | -0.212 % | [-0.486, +0.061] % | 0.123 |

표 31. 페이지 캐시 drop 영향 — 첫 GC 시각 (gc_onset_s)

| 매핑 | bs | 쌍 | nodrop 평균 | drop 평균 | 차이 (drop−nodrop) | 95 % CI | p |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K | 3 | 6.32 | 6.34 | +0.362 % |  | 0.446 |
| 4K | 16K | 3 | 5.69 | 5.72 | +0.513 % |  | 0.012 |
| 4K | 32K | 3 | 5.69 | 5.73 | +0.570 % |  | 0.000 |
| 16K | 4K | 3 | 5.91 | 5.95 | +0.578 % |  | 0.002 |
| 16K | 16K | 3 | 5.92 | 5.96 | +0.601 % |  | 0.004 |
| 16K | 32K | 3 | 5.84 | 5.88 | +0.531 % |  | 0.001 |
| 32K | 4K | 3 | 6.06 | 6.09 | +0.538 % |  | 0.000 |
| 32K | 16K | 3 | 6.06 | 6.09 | +0.463 % |  | 0.004 |
| 32K | 32K | 3 | 6.10 | 6.12 | +0.450 % |  | 0.005 |
| 전체 |  | 27 | – | – | +0.512 % | [+0.432, +0.592] % | 0.000 |

그림 6. nodrop / drop 대역폭 비교 (회색 = bs < 매핑 단위)

## 6.3 시간에 따른 대역폭

각 칸은 (매핑 단위, bs) 조합 하나이며 3 회차를 겹쳐 그렸다. 점선은 그 회차의 첫 GC 시각이다. 대역폭이 GC 시작 직후 급락했다가 fio randommap 주기에 따라 회복·급등하는 모양(5.5 절)을 볼 수 있다.

그림 7. 0.5 s 평균 쓰기 대역폭 시계열 — wbuffix_drop

그림 8. 0.5 s 평균 쓰기 대역폭 시계열 — wbuffix_nodrop

## 6.4 반복 간 편차

표 32. 대역폭의 반복 간 변동계수

| 변형 | 평균 변동계수(CV) | 최대 CV | 최대 CV 조합 (매핑/bs) |
| --- | --- | --- | --- |
| wbuffix_drop | 0.15 % | 0.35 % | 4K / 4K |
| wbuffix_nodrop | 0.15 % | 0.37 % | 4K / 4K |

## 6.5 관찰

페이지 캐시 drop(sync; echo 3 > /proc/sys/vm/drop_caches)은 결과에 영향이 없다. 27 쌍(9 조합 × 3 회)의 대역폭 차이는 평균 +0.009 %(95 % CI −0.084 ~ +0.101 %, p = 0.85)이고, 조합별 차이도 −0.21 ~ +0.39 %(모든 p > 0.15)다. IOPS·평균/p99 지연·GC 전후 대역폭·WAF 도 유의한 차이가 없다.

drop 조건에서 일관되게 달라진 것은 fio 시작 표지부터 첫 GC 까지의 시간뿐이다. 23–36 ms(+0.51 %, p < 1e-12) 늦은데, 캐시를 비운 뒤 sudo·fio 실행 파일을 디스크에서 다시 읽느라 fio 시작이 늦어진 것으로 보인다. 장치 성능과는 무관하다.

bs ≥ 매핑 단위에서는 60 s 평균 대역폭이 416–482 MiB/s 로 매핑 단위에 따른 차이가 작다. bs 16K 는 매핑 4K 481.0, 16K 420.4 MiB/s(−12.6 %)이고, bs 32K 는 매핑 4K 482.3, 16K 441.6, 32K 424.6 MiB/s(−8.4 %, −11.9 %)다.

bs < 매핑 단위에서는 성능이 크게 떨어진다. 매핑 16K·bs 4K 는 129.7 MiB/s 로 매핑 4K·bs 4K(416.0) 대비 −68.8 %, 매핑 32K·bs 4K 는 68.3 MiB/s(−83.6 %), 매핑 32K·bs 16K 는 219.6 MiB/s 로 매핑 4K·bs 16K(481.0) 대비 −54.3 % 다.

그 원인은 쓰기 증폭이다. WAF_total(NAND 바이트 ÷ 호스트 바이트)은 bs ≥ 매핑에서 3.1–3.5 인데, 16K/4K 8.8, 32K/4K 17.1, 32K/16K 6.0 이다. 이는 매핑 단위/bs 배의 부분 쓰기 증폭(4·8·2 배)에 GC 증폭(2.1–3.0)이 곱해진 값이다.

GC 이전 구간의 대역폭은 NAND 프로그램 한계로 정해진다. bs ≥ 매핑에서는 2,001–2,218 MiB/s 로, 16 die 합 한계 2,233 MiB/s 의 90–99 % 다. bs < 매핑에서는 그 한계 × bs/매핑에 가깝다(16K/4K 535.6 = 한계 × 1/4 의 96 %, 32K/4K 256.1 = × 1/8 의 92 %, 32K/16K 1,026.9 = × 1/2 의 92 %).

장치가 빈 상태에서 시작하므로 모든 조합에서 첫 GC 는 5.7–6.3 s 에 시작한다(물리 공간의 380/384 line 이 찼을 때). 60 s 평균에는 GC 이전 약 6 s(약 2 GB/s)가 크게 반영된다. GC 이후 구간 평균은 bs ≥ 매핑에서 240–307 MiB/s, bs < 매핑에서 47–130 MiB/s 다.

GC 이후 대역폭은 fio 의 randommap 주기를 따라 출렁인다. bs ≥ 매핑에서는 GC 시작 직후 약 130 MiB/s 까지 떨어졌다가 회복하고, 장치를 한 바퀴 다 쓸 무렵(약 37–52 s) 1,500–1,900 MiB/s 로 치솟은 뒤 다시 떨어진다. bs < 매핑에서는 치솟지 않고 계속 떨어진다. 그래서 마지막 20 s 평균은 조합에 따라 순서가 바뀐다(예: bs 16K 에서 매핑 4K 244.8 < 매핑 16K 323.4 MiB/s). 결과를 비교할 때는 60 s 평균과 GC 이후 구간 평균을 함께 보아야 한다.

평균 완료 지연은 IOPS 와 반비례한다(iodepth 32 고정, Little 의 법칙). 4K/4K 299 µs, 16K/4K 963 µs, 32K/4K 1,829 µs 다. p99 지연은 모든 조합에서 13–18 ms 로, GC 로 쓰기 버퍼가 막히는 구간이 결정한다.

반복 간 대역폭 편차는 매우 작다. 변동계수 평균 0.15 %, 최대 0.37 %(4K/4K, nodrop; drop 은 0.35 %)다. fio 난수 순서가 고정이라(randrepeat=1) 1–3 회차의 시계열이 거의 겹친다.

L2P 매핑 표 크기(물리 12 GiB 의 전체 FTL 페이지 × 항목 8 B, NVMeVirt maptbl 기준)는 매핑 4K 24 MiB, 16K 6 MiB, 32K 3 MiB 로 매핑 단위에 반비례한다(논리 11.2 GiB 장치 기준으로 계산하면 22.4/5.6/2.8 MiB). 다만 NVMeVirt 에는 매핑 캐시(DRAM) 모델이 없어 이 이득은 성능 수치에 나타나지 않는다.

해석 시 한계: NVMeVirt 는 부분 쓰기의 read-modify-write(옛 데이터 읽기)를 모델링하지 않으므로, bs < 매핑 단위의 불이익은 실제 SSD 보다 작게 나올 수 있다. 쓰기 조기 완료라 NAND·GC 비용은 쓰기 버퍼가 찰 때만 호스트에 보인다. 지우기 지연은 0 이다.

# 7. 보조 데이터셋 — 첫 설계 (main_20261008, 부분)

첫 설계(매핑 4–128K × bs 4–128K × 3 회)는 설계 변경으로 15:51:23 KST 에 멈췄다. 완료한 회차는 wbuffix 53 회다(반복 1 의 36 조합 전부와 반복 2 의 17 조합). 주 데이터셋과 같은 모듈 바이너리·절차이며 페이지 캐시는 그대로(nodrop)였다. 칸의 (n=…) 는 완료 회차 수다. 64K·128K 는 flash page 도 함께 커져 NAND 성능이 달라진다(5.5 절). 당시 먼저 돌린 원본 모델(base)의 결과는 bs < 매핑 단위 조합이 5.4 절의 결함 때문에 유효한 측정이 아니고 보려던 실험도 아니어서, 사용자 요청으로 모두 뺐다(10 절).

표 33. 쓰기 대역폭 MiB/s — 보조 · wbuffix (* bs < 매핑)

| 매핑 \ bs | 4K | 8K | 16K | 32K | 64K | 128K |
| --- | --- | --- | --- | --- | --- | --- |
| 4K | 418.6 ±1.3 (n=2) | 438.7 ±0.9 (n=2) | 480.9 ±0.0 (n=2) | 482.3 ±0.1 (n=2) | 486.3 ±0.0 (n=2) | 496.8 ±0.0 (n=2) |
| 8K | 216.7 ±0.1 (n=2) * | 418.5 ±2.2 (n=2) | 437.9 ±1.6 (n=2) | 482.3 ±0.1 (n=2) | 486.4 ±0.0 (n=2) | 496.7 ±0.0 (n=2) |
| 16K | 129.8 ±0.2 (n=2) * | 217.8 ±0.3 (n=2) * | 420.5 ±0.3 (n=2) | 442.2 ±3.8 (n=2) | 486.3 ±0.1 (n=2) | 496.9 ±0.0 (n=1) |
| 32K | 68.3 ±0.0 (n=1) * | 131.7 ±0.0 (n=1) * | 219.3 ±0.0 (n=1) * | 424.5 ±0.0 (n=1) | 446.7 ±0.0 (n=1) | 496.7 ±0.0 (n=1) |
| 64K | 47.1 ±0.0 (n=1) * | 93.3 ±0.0 (n=1) * | 175.2 ±0.0 (n=1) * | 285.6 ±0.0 (n=1) * | 645.1 ±0.0 (n=1) | 666.0 ±0.0 (n=1) |
| 128K | 28.1 ±0.0 (n=1) * | 56.1 ±0.0 (n=1) * | 110.9 ±0.0 (n=1) * | 205.0 ±0.0 (n=1) * | 340.7 ±0.0 (n=1) * | 837.8 ±0.0 (n=1) |

표 34. 전체 쓰기 증폭 WAF_total — 보조 · wbuffix (* bs < 매핑)

| 매핑 \ bs | 4K | 8K | 16K | 32K | 64K | 128K |
| --- | --- | --- | --- | --- | --- | --- |
| 4K | 3.15 (n=2) | 3.33 (n=2) | 3.55 (n=2) | 3.54 (n=2) | 3.53 (n=2) | 3.49 (n=2) |
| 8K | 5.89 (n=2) * | 3.13 (n=2) | 3.31 (n=2) | 3.54 (n=2) | 3.53 (n=2) | 3.49 (n=2) |
| 16K | 8.83 (n=2) * | 5.92 (n=2) * | 3.12 (n=2) | 3.32 (n=2) | 3.53 (n=2) | 3.49 (n=1) |
| 32K | 17.07 (n=1) * | 8.95 (n=1) * | 5.94 (n=1) * | 3.12 (n=1) | 3.30 (n=1) | 3.49 (n=1) |
| 64K | 48.20 (n=1) * | 24.43 (n=1) * | 13.14 (n=1) * | 8.25 (n=1) * | 3.61 (n=1) | 3.67 (n=1) |
| 128K | 112.72 (n=1) * | 56.45 (n=1) * | 28.52 (n=1) * | 15.32 (n=1) * | 9.09 (n=1) * | 3.65 (n=1) |

그림 9. 보조 · wbuffix 시계열 (6 × 6)

그림 10. 보조 · wbuffix 평균 대역폭

# 8. 추가 실험 — 순차 쓰기 (seq3x3_20261008)

## 8.1 목적과 설계

주 데이터셋과 같은 매핑 단위·bs 조합에서 랜덤 쓰기 대신 순차 쓰기를 쟀다(사용자 요청, 19:53 KST). 순차 쓰기에서는 같은 매핑 단위로 작은 쓰기가 연달아 오므로, 실제 SSD 는 쓰기 버퍼에서 이를 합쳐 매핑 단위 페이지를 한 번만 쓴다. NVMeVirt 원래 모델은 합치지 않고 쓰기마다 페이지를 새로 쓴다(5.5 절). 그래서 원래 모델(wbuffix, 주 데이터셋과 같은 모듈 파일)과, 이 연구에서 추가한 쓰기 버퍼 병합 모델(merge, 8.2 절)을 함께 쟀다(사용자 선택).

표 35. 순차 쓰기 실험 설계

| 구분 | 값 |
| --- | --- |
| 변수 | 매핑 단위 4K·16K·32K × fio bs 4K·16K·32K × 모델 {wbuffix, merge} |
| 반복 | 조합마다 3 회 (회차마다 rmmod → insmod) — 9 × 2 × 3 = 54 회 |
| 순서 | 반복 r = 1, 2, 3 마다 wbuffix 9 조합 → merge 9 조합 (run_all_seq.sh) |
| fio | rw=write (순차, offset 0 부터, 장치 끝에서 0 으로 돌아감), 나머지는 주 데이터셋과 같음: libaio, direct=1, iodepth 32, numjobs 1, 60 s, ramp_time 0 |
| OS 페이지 캐시 | 그대로 (nodrop) — 주 데이터셋에서 drop 영향이 없었음(6.2 절), 사용자 선택 |
| 블록 계층 | 회차마다 insmod 직후 echo 2 > /sys/block/nvme1n1/queue/nomerges (요청 병합 끔). 스케줄러는 커널 기본 mq-deadline |
| 모듈 | wbuffix: 주 데이터셋과 같은 .ko (SHA-256 같음) / merge: 커밋 66446ea 에서 make MAPPING_UNIT=<바이트> GC_STATS=1 WBUF_FIX=1 WBUF_MERGE=1 |
| 그 밖 | 장치·CPU·insmod 인자·SETTLE 5 s·결과 파일 구성은 주 데이터셋과 같음 (3–4 절) |

블록 계층 병합을 끈 이유: 커널 기본 설정(nomerges=0)에서는 fio 의 순차 4K 쓰기(QD 32)가 블록 계층에서 합쳐져 장치에 평균 약 120 KiB 요청으로 도착했다(8.3 절 표의 첫 줄). 이 상태에서는 bs 축이 의미를 잃는다. 랜덤 쓰기는 인접한 요청이 거의 없어 영향이 없다(8.3 절).

## 8.2 쓰기 버퍼 병합 모델 (merge, WBUF_MERGE=1)

파티션(FTL 인스턴스 4 개)마다 '열린 매핑 단위' 하나를 둔다. 매핑 단위 일부만 덮는 쓰기가 오면 그 단위를 열고 쓰기 버퍼에서 매핑 단위 하나만큼을 잡는다.

같은 단위로 다음 쓰기가 오면 버퍼를 더 잡지 않고 합류시킨다(섹터 비트맵). 단위의 모든 섹터가 채워지면 그때 매핑 단위 페이지 하나를 flash 에 쓴다(원본과 같은 쓰기 경로: 페이지 배정 → wordline 이 차면 프로그램 → 버퍼 반납).

같은 파티션에 다른 단위의 부분 쓰기가 오면 열린 단위를 덜 찬 채로 쓰고(evict, 원본과 같이 옛 데이터 읽기는 모델링하지 않음) 새 단위를 연다.

매핑 단위 전체를 덮는 쓰기는 원본(wbuffix)과 똑같이 바로 쓴다. 따라서 bs ≥ 매핑 단위이면 두 모델의 동작이 같다.

버퍼 장부: 쓰는 페이지마다 매핑 단위 하나가 정확히 한 번 할당되어 반납과 맞는다. 계속 붙잡히는 양은 파티션당 열린 단위 1 개와 덜 찬 wordline 으로 128 KiB 이하다(버퍼 1 MiB).

한계: 파티션마다 열린 단위가 1 개뿐이다(실제 SSD 는 버퍼 전체를 여러 단위에 쓸 수 있다). 하나의 순차 흐름에는 충분하다. FUA·FLUSH 는 원본처럼 열린 단위를 강제로 쓰지 않는다(이 장치는 VWC=0 이라 커널이 보내지 않음).

WBUF_MERGE=0 인 빌드(wbuffix 등)는 이 변경 전과 같은 기계어다(디스어셈블리 비교로 확인).

코드 검토: 실행 전에 에이전트 4 개가 관점 하나씩(쓰기 버퍼 장부, FTL 상태, 실험에 맞는 모델 동작, 커널 안전성) 독립적으로 읽고, 지적마다 반박 검증 1 개를 두었다(20:01–20:13 KST). 고칠 결함은 없었다. 장부 로직은 Python 으로 옮겨 무작위 명령 약 180 만 개(매핑 단위 3 종 × 시드 30 × 명령 2 만)로 퍼징했다(위반 없음). 지적 1 건(FUA/FLUSH 가 열린 단위를 내보내지 않음)은 원본과 같은 단순화로 판정되어 주석만 보강했다. rmmod 때 파티션마다 병합 통계(open·merge·full·evict·direct·still_open)를 찍는다.

static bool ksc_conv_write_merge(struct nvmev_ns *ns, struct nvmev_request *req,

struct nvmev_result *ret)

{

struct conv_ftl *conv_ftls = (struct conv_ftl *)ns->ftls;

struct ssdparams *spp = &conv_ftls[0].ssd->sp;

struct buffer *wbuf = conv_ftls[0].ssd->write_buffer;

struct nvme_command *cmd = req->cmd;

uint64_t lba = cmd->rw.slba;

uint64_t nr_lba = (cmd->rw.length + 1);

uint64_t secs_per_pg = spp->secs_per_pg;

uint64_t start_lpn = lba / secs_per_pg;

uint64_t end_lpn = (lba + nr_lba - 1) / secs_per_pg;

uint32_t nr_parts = ns->nr_parts;

uint64_t sim_open[SSD_PARTITIONS];

uint64_t lpn, wbuf_bytes = 0;

uint64_t nsecs_latest, nsecs_xfer_completed;

uint32_t i;

struct nand_cmd swr = {

.type = USER_IO,

.cmd = NAND_WRITE,

.interleave_pci_dma = false,

.xfer_size = spp->pgsz * spp->pgs_per_oneshotpg,

};

NVMEV_ASSERT(nr_parts <= SSD_PARTITIONS);

NVMEV_ASSERT(secs_per_pg == MAPPING_UNIT / LBA_SIZE);

if ((end_lpn / nr_parts) >= spp->tt_pgs) {

NVMEV_ERROR("%s: lpn passed FTL range (start_lpn=%lld > tt_pgs=%ld)\n", __func__,

start_lpn, spp->tt_pgs);

return false;

}

/* write buffer: one mapping unit for every unit this command opens or writes directly; replay

* the slot decisions of the loop below so that the amount matches exactly */

for (i = 0; i < nr_parts; i++)

sim_open[i] = conv_ftls[i].ksc_open_lpn;

for (lpn = start_lpn; lpn <= end_lpn; lpn++) {

uint64_t s = (lpn == start_lpn) ? lba % secs_per_pg : 0;

uint64_t e = (lpn == end_lpn) ? (lba + nr_lba - 1) % secs_per_pg : secs_per_pg - 1;

uint32_t p = lpn % nr_parts;

if (sim_open[p] == lpn)

continue;

wbuf_bytes += spp->pgsz;

if (!(s == 0 && e == secs_per_pg - 1))

sim_open[p] = lpn;

}

if (wbuf_bytes && buffer_allocate(wbuf, wbuf_bytes) < wbuf_bytes)

return false;

nsecs_latest = ssd_advance_write_buffer(conv_ftls[0].ssd, req->nsecs_start, LBA_TO_BYTE(nr_lba));

nsecs_xfer_completed = nsecs_latest;

swr.stime = nsecs_latest;

for (lpn = start_lpn; lpn <= end_lpn; lpn++) {

uint64_t s = (lpn == start_lpn) ? lba % secs_per_pg : 0;

uint64_t e = (lpn == end_lpn) ? (lba + nr_lba - 1) % secs_per_pg : secs_per_pg - 1;

struct conv_ftl *f = &conv_ftls[lpn % nr_parts];

uint64_t done = 0;

if (f->ksc_open_lpn == lpn) {

bitmap_set(f->ksc_open_mask, s, e - s + 1);

f->ksc_mg_merge++;

if (bitmap_full(f->ksc_open_mask, secs_per_pg)) {

f->ksc_open_lpn = INVALID_LPN;

f->ksc_mg_full++;

done = ksc_write_lpn(conv_ftls, nr_parts, lpn, &swr, req->sq_id, wbuf);

}

} else if (s == 0 && e == secs_per_pg - 1) {

f->ksc_mg_direct++;

done = ksc_write_lpn(conv_ftls, nr_parts, lpn, &swr, req->sq_id, wbuf);

} else {

if (f->ksc_open_lpn != INVALID_LPN) {

uint64_t old = f->ksc_open_lpn;

f->ksc_open_lpn = INVALID_LPN;

f->ksc_mg_evict++;

done = ksc_write_lpn(conv_ftls, nr_parts, old, &swr, req->sq_id, wbuf);

}

f->ksc_open_lpn = lpn;

bitmap_zero(f->ksc_open_mask, secs_per_pg);

bitmap_set(f->ksc_open_mask, s, e - s + 1);

f->ksc_mg_open++;

}

nsecs_latest = max(done, nsecs_latest);

}

if ((cmd->rw.control & NVME_RW_FUA) || (spp->write_early_completion == 0)) {

/* Wait all flash operations */

ret->nsecs_target = nsecs_latest;

} else {

/* Early completion */

ret->nsecs_target = nsecs_xfer_completed;

}

ret->status = NVME_SC_SUCCESS;

return true;

}

## 8.3 사전 점검

표 36. 순차 쓰기 사전 점검 (15 s, 1 회, 20:18–20:23 KST; 호스트 WAF = FTL 이 쓴 매핑 단위 페이지 × 매핑 단위 ÷ fio 바이트)

| 점검 | 모델 | 매핑 / bs | nomerges | MiB/s | 블록 계층 합친 수 | 평균 요청 KiB | 호스트 WAF | WAF total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A. 블록 계층 기본값 | wbuffix | 16K / 4K | 0 | 935.5 | 3,472,902 | 120.1 | 1.101 | 1.319 |
| B. 순차, 병합 끔 | wbuffix | 16K / 4K | 2 | 411.0 | 0 | 4.0 | 4.000 | 4.675 |
| B. 순차, 병합 끔 | wbuffix | 16K / 16K | 2 | 2,231.8 | 0 | 16.0 | 1.000 | 1.000 |
| B. 순차, 병합 끔 | wbuffix | 32K / 4K | 2 | 232.9 | 0 | 4.0 | 8.000 | 8.646 |
| B. 순차, 병합 끔 | wbuffix | 32K / 16K | 2 | 654.6 | 0 | 16.0 | 2.000 | 2.761 |
| B. 순차, 병합 끔 | merge | 16K / 4K | 2 | 2,105.5 | 0 | 4.0 | 1.000 | 1.000 |
| B. 순차, 병합 끔 | merge | 16K / 16K | 2 | 2,232.6 | 0 | 16.0 | 1.000 | 1.000 |
| B. 순차, 병합 끔 | merge | 32K / 4K | 2 | 2,101.6 | 0 | 4.0 | 1.000 | 1.000 |
| B. 순차, 병합 끔 | merge | 32K / 16K | 2 | 2,232.5 | 0 | 16.0 | 1.000 | 1.000 |
| C. 랜덤 (주 데이터셋 설정) | wbuffix | 16K / 4K | 0 | 297.9 | 1 | 4.0 | 4.000 | 4.961 |
| C. 랜덤 (주 데이터셋 설정) | merge | 16K / 4K | 0 | 295.8 | 1 | 4.0 | 4.000 | 4.937 |

A: 커널 기본값에서는 순차 4K 쓰기가 평균 약 120 KiB 요청으로 합쳐졌다. B: 병합을 끄면 장치가 받는 요청이 fio 의 bs 와 같아진다. 이때 wbuffix 의 호스트 WAF 는 정확히 매핑/bs(4·8·2)이고, merge 는 1.000 이다(16K/4K 에서 단위마다 쓰기 4 개를 합쳐 한 번 씀). bs = 매핑이면 두 모델의 결과가 같다. C: 랜덤 쓰기에서는 합칠 쓰기가 거의 없어 두 모델이 같고(−0.7 %), 블록 계층 병합도 15 s 동안 1 건뿐이다. 따라서 주 데이터셋(블록 계층 기본값)도 병합 영향이 없다.

## 8.4 결과 표

모든 값은 3 회 평균(± 는 표본 표준편차)이다. * 는 bs < 매핑 단위 조합이다.

표 37. 쓰기 대역폭 MiB/s, 60 s 평균 — 순차 · wbuffix

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 2,090 ±8.4 | 2,232 ±0.5 | 2,233 ±0.0 |
| 16K | 334.9 ±0.5 * | 2,233 ±0.0 | 2,232 ±0.2 |
| 32K | 209.5 ±0.0 * | 369.0 ±0.0 * | 2,232 ±0.2 |

표 38. 쓰기 대역폭 MiB/s, 60 s 평균 — 순차 · merge

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 2,087 ±3.2 | 2,233 ±0.1 | 2,233 ±0.0 |
| 16K | 2,094 ±5.3 * | 2,233 ±0.1 | 2,233 ±0.1 |
| 32K | 2,095 ±0.8 * | 2,232 ±0.0 * | 2,232 ±0.0 |

표 39. IOPS, 60 s 평균 — 순차 · wbuffix

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 534,956 | 142,874 | 71,447 |
| 16K | 85,733 * | 142,881 | 71,439 |
| 32K | 53,635 * | 23,619 * | 71,429 |

표 40. IOPS, 60 s 평균 — 순차 · merge

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 534,186 | 142,891 | 71,448 |
| 16K | 536,158 * | 142,884 | 71,442 |
| 32K | 536,221 * | 142,864 * | 71,432 |

표 41. 평균 완료 지연 µs — 순차 · wbuffix

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 58.9 | 222.5 | 446.0 |
| 16K | 372.2 * | 222.5 | 446.0 |
| 32K | 595.5 * | 1,353 * | 446.1 |

표 42. 평균 완료 지연 µs — 순차 · merge

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 58.9 | 222.5 | 446.0 |
| 16K | 58.7 * | 222.5 | 446.0 |
| 32K | 58.7 * | 222.6 * | 446.1 |

표 43. p99 완료 지연 µs — 순차 · wbuffix

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 63 | 227 | 453 |
| 16K | 3,643 * | 226 | 450 |
| 32K | 2,256 * | 8,356 * | 450 |

표 44. p99 완료 지연 µs — 순차 · merge

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 63 | 228 | 451 |
| 16K | 62 * | 227 | 453 |
| 32K | 62 * | 226 * | 451 |

표 45. 전체 쓰기 증폭 WAF_total (NAND 바이트 ÷ 호스트 바이트) — 순차 · wbuffix

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 1.000 | 1.000 | 1.000 |
| 16K | 5.404 * | 1.000 | 1.000 |
| 32K | 9.155 * | 3.405 * | 1.000 |

표 46. 전체 쓰기 증폭 WAF_total (NAND 바이트 ÷ 호스트 바이트) — 순차 · merge

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 1.000 | 1.000 | 1.000 |
| 16K | 1.000 * | 1.000 | 1.000 |
| 32K | 1.000 * | 1.000 * | 1.000 |

표 47. GC 이후 구간 평균 대역폭 MiB/s — 순차 · wbuffix

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 2,084 | 2,233 | 2,233 |
| 16K | 312.5 * | 2,233 | 2,233 |
| 32K | 202.6 * | 294.3 * | 2,232 |

표 48. GC 이후 구간 평균 대역폭 MiB/s — 순차 · merge

| 매핑 \ bs | 4K | 16K | 32K |
| --- | --- | --- | --- |
| 4K | 2,081 | 2,233 | 2,233 |
| 16K | 2,089 * | 2,233 | 2,233 |
| 32K | 2,089 * | 2,232 * | 2,232 |

표 49. 순차 쓰기 — 원래 모델(wbuffix)과 병합 모델(merge)

| 매핑 | bs | bs<매핑 | wbuffix MiB/s | merge MiB/s | 차이 | WAF_total wbuffix | WAF_total merge |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K |  | 2,090 | 2,087 | -0.1 % | 1.000 | 1.000 |
| 4K | 16K |  | 2,232 | 2,233 | +0.0 % | 1.000 | 1.000 |
| 4K | 32K |  | 2,233 | 2,233 | +0.0 % | 1.000 | 1.000 |
| 16K | 4K | 예 | 334.9 | 2,094 | +525.4 % | 5.404 | 1.000 |
| 16K | 16K |  | 2,233 | 2,233 | +0.0 % | 1.000 | 1.000 |
| 16K | 32K |  | 2,232 | 2,233 | +0.0 % | 1.000 | 1.000 |
| 32K | 4K | 예 | 209.5 | 2,095 | +899.8 % | 9.155 | 1.000 |
| 32K | 16K | 예 | 369.0 | 2,232 | +504.9 % | 3.405 | 1.000 |
| 32K | 32K |  | 2,232 | 2,232 | +0.0 % | 1.000 | 1.000 |

표 50. merge 모델의 병합 통계 (4 파티션 합, 3 회 평균)

| 매핑 | bs | open | merge | full | evict | direct | still_open |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K | 0 | 0 | 0 | 0 | 32,051,722 | 0 |
| 4K | 16K | 0 | 0 | 0 | 0 | 34,294,367 | 0 |
| 4K | 32K | 0 | 0 | 0 | 0 | 34,295,725 | 0 |
| 16K | 4K | 8,042,597 | 24,127,413 | 8,042,426 | 169 | 0 | 1 |
| 16K | 16K | 0 | 0 | 0 | 0 | 8,573,167 | 0 |
| 16K | 32K | 0 | 0 | 0 | 0 | 8,573,208 | 0 |
| 32K | 4K | 4,021,747 | 28,152,078 | 4,021,709 | 37 | 0 | 1 |
| 32K | 16K | 4,285,997 | 4,285,986 | 4,285,986 | 11 | 0 | 0 |
| 32K | 32K | 0 | 0 | 0 | 0 | 4,286,009 | 0 |

블록 계층 확인: 54 회차 모두 fio 동안 블록 계층이 합친 요청은 최대 0 건이고, 장치가 받은 평균 요청 크기와 bs 의 차이는 최대 0.000 KiB 다.

## 8.5 그림

그림 11. 순차 쓰기 대역폭 — wbuffix / merge (회색 = bs < 매핑 단위)

그림 12. 순차 쓰기 WAF_total — wbuffix / merge

그림 13. bs 에 따른 순차 쓰기 대역폭 (모델별 패널)

그림 14. 0.5 s 평균 순차 쓰기 대역폭 시계열 — wbuffix (점선 = 첫 GC)

그림 15. 0.5 s 평균 순차 쓰기 대역폭 시계열 — merge (점선 = 첫 GC)

## 8.6 랜덤 쓰기와 비교

표 51. 60 s 평균 대역폭과 WAF_total — 랜덤(main3x3_20261008 · wbuffix_nodrop) / 순차(seq3x3_20261008)

| 매핑 | bs | bs<매핑 | 랜덤 MiB/s | 순차 wbuffix MiB/s | 순차 merge MiB/s | 랜덤 WAF | 순차 wbuffix WAF | 순차 merge WAF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4K | 4K |  | 416.0 | 2,090 | 2,087 | 3.12 | 1.00 | 1.00 |
| 4K | 16K |  | 481.0 | 2,232 | 2,233 | 3.55 | 1.00 | 1.00 |
| 4K | 32K |  | 482.3 | 2,233 | 2,233 | 3.54 | 1.00 | 1.00 |
| 16K | 4K | 예 | 129.7 | 334.9 | 2,094 | 8.82 | 5.40 | 1.00 |
| 16K | 16K |  | 420.4 | 2,233 | 2,233 | 3.12 | 1.00 | 1.00 |
| 16K | 32K |  | 441.6 | 2,232 | 2,233 | 3.32 | 1.00 | 1.00 |
| 32K | 4K | 예 | 68.3 | 209.5 | 2,095 | 17.08 | 9.15 | 1.00 |
| 32K | 16K | 예 | 219.6 | 369.0 | 2,232 | 5.96 | 3.40 | 1.00 |
| 32K | 32K |  | 424.6 | 2,232 | 2,232 | 3.12 | 1.00 | 1.00 |

## 8.7 반복 간 편차

표 52. 순차 쓰기 대역폭의 반복 간 변동계수

| 모델 | 평균 변동계수(CV) | 최대 CV | 최대 CV 조합 (매핑/bs) |
| --- | --- | --- | --- |
| wbuffix | 0.07 % | 0.40 % | 4K / 4K |
| merge | 0.05 % | 0.25 % | 16K / 4K |

## 8.8 관찰

순차 쓰기에서 bs ≥ 매핑 단위인 조합은 두 모델 모두 60 s 평균 2,087–2,233 MiB/s 다. bs 16K·32K 는 2,232.2–2,232.8 MiB/s 로, NAND 프로그램 한계(16 die 합 2,233 MiB/s)의 99.96 % 이상이다. bs 4K 는 매핑 4K(두 모델)와 병합 모델의 매핑 16K·32K 에서 약 535 K IOPS, 2,087–2,095 MiB/s(한계의 93.4–93.8 %)에서 멈춘다(요청 수 처리 쪽 한계로 보인다 — 추정). 이 조합들에서 두 모델의 차이는 최대 0.14 % 로, 반복 편차(변동계수 최대 0.40 %) 수준이다. 두 모델이 같은 코드 경로를 쓰기 때문이다.

원래 모델(wbuffix)에서 bs < 매핑 단위인 순차 쓰기는 크게 느려진다. 16K/4K 334.9, 32K/4K 209.5, 32K/16K 369.0 MiB/s 로, 같은 bs 의 매핑 4K 대비 −84.0 %, −90.0 %, −83.5 % 다. NVMeVirt 는 같은 매핑 단위로 이어지는 작은 쓰기를 합치지 않고 쓰기마다 매핑 단위 페이지를 새로 쓰기 때문이다. GC 이전 구간도 NAND 한계 × bs/매핑의 99.4–99.8 %(556.1, 278.5, 1,109.9 MiB/s)에 머문다.

쓰기 버퍼 병합 모델(merge)에서는 이 불이익이 사라진다. 16K/4K 2,094.4, 32K/4K 2,094.6, 32K/16K 2,232.2 MiB/s 로, 같은 bs 의 매핑 4K(2,086.7 / 2,086.7 / 2,232.7) 대비 +0.37 %, +0.38 %, −0.02 % 다. wbuffix 보다는 6.25 배, 10.00 배, 6.05 배 빠르다. 즉 순차 쓰기는 쓰기 버퍼가 작은 쓰기를 합치기만 하면 매핑 단위를 16K·32K 로 키워도 대역폭 손실이 없다.

WAF_total 은 병합 모델에서 9 조합 모두 1.000 이다. 원래 모델은 bs ≥ 매핑에서 1.000, bs < 매핑에서 16K/4K 5.404, 32K/4K 9.155, 32K/16K 3.405 다. 원래 모델의 값은 부분 쓰기 증폭(매핑/bs = 4·8·2 배)과 GC 증폭(1.351, 1.144, 1.702)의 곱이다. GC 증폭은 매핑 단위마다 마지막 판 하나만 유효하게 남기 때문에 생긴다. 16K/4K·32K/4K 는 단순 계산 1/(1 − bs/매핑) = 1.333·1.143 과 거의 같고, 32K/16K 는 단순 계산 2.000 보다 작다.

랜덤 쓰기와 비교하면 같은 조합에서 순차 쓰기(merge)가 bs ≥ 매핑에서 4.6–5.3 배, bs < 매핑에서 10.2–30.7 배 빠르다. 랜덤 쓰기(주 데이터셋 nodrop)는 bs ≥ 매핑에서 GC 복사 때문에 416–482 MiB/s(WAF 3.1–3.5)다. 랜덤 쓰기의 bs < 매핑 손실(−54–84 %)은 다음 쓰기가 다른 매핑 단위로 가서 합칠 것이 없으므로 쓰기 버퍼 병합으로도 줄지 않는다(8.3 절 C: merge −0.7 %). 반면 순차 쓰기의 손실은 병합으로 없어진다.

병합 통계(3 회 평균)에서 열린 단위 하나당 합류 횟수는 16K/4K 3.000, 32K/4K 7.000, 32K/16K 1.000 으로 매핑/bs − 1 과 정확히 같다. 덜 찬 채 쓴 단위(evict)는 회차당 169·37·11 개로, 연 단위의 0.0003–0.002 % 뿐이다. 장치 끝에서 처음으로 돌아갈 때 생기고, 4K 에서 더 많은 것은 드물게 요청 순서가 바뀌기 때문으로 보인다(추정).

순차 쓰기에서는 GC 가 거의 비용 없이 일어난다. bs ≥ 매핑과 병합 모델에서 GC 는 회차당 victim line 약 14,100–15,200 개를 지우지만 옮긴 페이지는 0 이다. 앞 바퀴의 line 이 통째로 무효가 되기 때문이다. 첫 GC 는 5.69–5.93 s 에 시작한다. bs 16K·32K 는 GC 전후 대역폭이 같고(약 2,233 MiB/s), bs 4K 는 GC 이후 약 3 % 낮다(2,142–2,152 → 2,081–2,089 MiB/s). 60 s 동안 122–131 GiB, 장치를 약 11 바퀴 쓴다.

평균 완료 지연은 iodepth 32 ÷ IOPS 로 정해진다(bs 4K 약 59 µs, 16K 222.5 µs, 32K 446 µs). p99 는 bs ≥ 매핑과 병합 모델에서 453 µs 이하다. 원래 모델의 bs < 매핑은 16K/4K 3,643 µs, 32K/4K 2,256 µs, 32K/16K 8,356 µs 다. 랜덤 쓰기의 p99(13–18 ms)보다는 짧다.

반복 간 대역폭 편차는 매우 작다. 변동계수 평균은 0.067 %(wbuffix)·0.051 %(merge)이고, 최대는 0.40 %(wbuffix 4K/4K)다.

해석 시 주의할 점이 있다. 병합 모델은 이 연구에서 추가한 것이다. 파티션마다 열린 단위가 1 개이고, 덜 찬 단위의 RMW 읽기는 없으며, 하나의 순차 흐름을 가정한다. 따라서 이 결과는 병합이 잘 될 때의 상한에 가깝다. 여러 흐름이 섞이거나 쓰기 버퍼가 작으면 실제 SSD 의 이득은 이보다 작을 수 있다. 원래 NVMeVirt 모델(wbuffix)의 결과는 병합이 전혀 없는 SSD 의 하한으로 읽는다. 모든 순차 쓰기 결과는 블록 계층 병합을 끈 상태(nomerges=2)에서 쟀다. 커널 기본값이면 순차 요청이 블록 계층에서 합쳐져 bs 축이 의미를 잃는다(8.3 절 A).

# 9. 추가 실험 — 랜덤 쓰기 bs 8K·64K (randbs_20261008)

## 9.1 목적과 설계

주 데이터셋의 랜덤 쓰기(bs 4K·16K·32K)에 bs 8K 와 64K 를 더해, 매핑 단위 4K·16K·32K 에서 bs 를 5 단계로 본다(사용자 요청, 22:08 KST). 사용자 선택에 따라 매핑 단위는 그대로 두고 bs 만 늘렸다. 새 조합(매핑 3 × bs 2)만 측정했고, bs 4K·16K·32K 는 주 데이터셋 nodrop 회차를 그대로 쓴다. 두 데이터셋은 같은 wbuffix 모듈 파일(SHA-256 같음)·같은 fio 설정·같은 절차이며, 블록 계층 설정도 같다(커널 기본값).

표 53. 랜덤 쓰기 bs 8K·64K 실험 설계

| 구분 | 값 |
| --- | --- |
| 새로 잰 조합 | 매핑 단위 4K·16K·32K × fio bs 8K·64K × 3 회 = 18 회 (randbs_20261008/wbuffix/) |
| 재사용한 조합 | 매핑 4K·16K·32K × bs 4K·16K·32K × 3 회 = 27 회 (main3x3_20261008/wbuffix_nodrop/, 6 절) |
| 합친 보기 | rand3x5_20261008/wbuffix/ — 두 폴더의 회차를 상대 심볼릭 링크로 모은 것(exp/link_runs.py, SOURCES.txt). analyze.py·plot.py 가 보통 데이터셋처럼 읽는다 |
| fio | rw=randwrite, libaio, direct=1, iodepth 32, numjobs 1, 60 s, ramp_time 0, randrepeat=1 (주 데이터셋과 같음) |
| 모델·캐시·블록 계층 | wbuffix, 페이지 캐시 그대로(nodrop), nomerges 커널 기본값 (주 데이터셋과 같음) |
| 순서 | 반복 1→3, 매핑 4K→32K, bs 8K→64K (run_all_rand_bs.sh) |

블록 계층 확인: 새 18 회차에서 fio 동안 블록 계층이 합친 요청은 최대 1,817 건이고, 장치가 받은 평균 요청 크기와 bs 의 차이는 최대 0.004 KiB 다.

## 9.2 결과 표 (매핑 3 × bs 5)

모든 값은 3 회 평균(± 는 표본 표준편차)이다. † 열(bs 8K·64K)이 이번에 잰 값이고, 나머지 열은 주 데이터셋 nodrop 회차(6.1 절 표와 같은 값)다. * 는 bs < 매핑 단위다.

표 54. 쓰기 대역폭 MiB/s, 60 s 평균 — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 416.0 ±1.6 | 437.0 ±1.2 | 481.0 ±0.1 | 482.3 ±0.2 | 486.3 ±0.0 |
| 16K | 129.7 ±0.0 * | 217.3 ±0.1 * | 420.4 ±1.3 | 441.6 ±1.3 | 486.4 ±0.1 |
| 32K | 68.3 ±0.0 * | 131.9 ±0.2 * | 219.6 ±0.4 * | 424.6 ±0.2 | 447.1 ±0.0 |

표 55. IOPS, 60 s 평균 — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 106,495 | 55,938 | 30,781 | 15,433 | 7,781 |
| 16K | 33,209 * | 27,815 * | 26,905 | 14,131 | 7,782 |
| 32K | 17,482 * | 16,890 * | 14,052 * | 13,589 | 7,154 |

표 56. 평균 완료 지연 µs — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 299.5 | 570.8 | 1,038 | 2,072 | 4,110 |
| 16K | 962.5 * | 1,149 * | 1,188 | 2,263 | 4,109 |
| 32K | 1,829 * | 1,893 * | 2,276 * | 2,353 | 4,470 |

표 57. p99 완료 지연 µs — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 13,347 | 16,581 | 17,695 | 18,219 | 19,005 |
| 16K | 14,047 * | 16,537 * | 16,843 | 18,219 | 19,005 |
| 32K | 14,483 * | 15,051 * | 17,607 * | 17,695 | 19,005 |

표 58. 전체 쓰기 증폭 WAF_total — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 3.12 | 3.32 | 3.55 | 3.54 | 3.53 |
| 16K | 8.82 * | 5.90 * | 3.12 | 3.32 | 3.53 |
| 32K | 17.08 * | 8.97 * | 5.96 * | 3.12 | 3.31 |

표 59. 첫 GC 시각 s — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 6.32 | 5.74 | 5.69 | 5.69 | 5.69 |
| 16K | 5.91 * | 5.90 * | 5.92 | 5.84 | 5.69 |
| 32K | 6.06 * | 6.07 * | 6.06 * | 6.10 | 5.97 |

표 60. GC 이전 구간 평균 대역폭 MiB/s — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 2,001 | 2,211 | 2,218 | 2,218 | 2,218 |
| 16K | 535.6 * | 1,072 * | 2,143 | 2,173 | 2,218 |
| 32K | 256.1 * | 512.6 * | 1,027 * | 2,033 | 2,124 |

표 61. GC 이후 구간 평균 대역폭 MiB/s — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 240.1 | 258.1 | 305.8 | 307.2 | 311.7 |
| 16K | 88.8 * | 131.1 * | 246.6 | 267.0 | 311.8 |
| 32K | 47.4 * | 89.7 * | 129.9 * | 246.0 | 278.0 |

표 62. 마지막 20 s 평균 대역폭 MiB/s — 랜덤 · 매핑 3 × bs 5

| 매핑 \ bs | 4K | 8K † | 16K | 32K | 64K † |
| --- | --- | --- | --- | --- | --- |
| 4K | 322.0 | 330.1 | 244.8 | 247.2 | 254.5 |
| 16K | 58.2 * | 80.9 * | 323.4 | 321.4 | 254.7 |
| 32K | 32.6 * | 60.5 * | 82.2 * | 322.8 | 320.0 |

## 9.3 그림

그림 16. bs 에 따른 랜덤 쓰기 대역폭 (매핑 단위별, 평균 ± 표준편차)

그림 17. 매핑 단위 × bs 평균 대역폭

그림 18. 전체 쓰기 증폭 WAF_total

그림 19. 평균 완료 지연 (로그 축)

그림 20. 0.5 s 평균 랜덤 쓰기 대역폭 시계열 (3 회 겹침, 점선 = 첫 GC)

## 9.4 반복 간 편차

표 63. 대역폭의 반복 간 변동계수

| 범위 | 평균 변동계수(CV) | 최대 CV | 최대 CV 조합 (매핑/bs) |
| --- | --- | --- | --- |
| 새로 잰 bs 8K·64K | 0.08 % | 0.28 % | 4K / 8K |
| 합친 보기 전체 | 0.12 % | 0.37 % | 4K / 4K |

## 9.5 관찰

bs 8K 에서 대역폭은 매핑 4K 437.0, 16K 217.3, 32K 131.9 MiB/s 다. 매핑 4K 대비 −50.3 %, −69.8 % 다. bs 64K 에서는 486.3, 486.4, 447.1 MiB/s 로, 매핑 16K 는 4K 와 같고(+0.02 %) 32K 는 −8.1 % 다.

결과는 bs 의 절대 크기보다 bs/매핑 비율로 거의 정해진다. 비율이 같은 조합끼리는 대역폭이 3 % 안에서 같고 WAF_total 도 거의 같다(표기는 다른 곳과 같이 매핑/bs). 비율 1(4K/4K, 16K/16K, 32K/32K)은 416.0·420.4·424.6 MiB/s, WAF 3.12 다. 비율 2(4K/8K, 16K/32K, 32K/64K)는 437.0·441.6·447.1 MiB/s, WAF 3.31–3.32 다. 비율 4(4K/16K, 16K/64K)는 481.0·486.4 MiB/s 다. 비율 1/2(16K/8K, 32K/16K)은 217.3·219.6 MiB/s, WAF 5.90·5.96 이고, 비율 1/4(16K/4K, 32K/8K)은 129.7·131.9 MiB/s, WAF 8.82·8.97 이다. flash page 는 모든 조합에서 32 KiB 로 같다.

bs ≥ 매핑인 10 조합은 416.0–486.4 MiB/s, WAF_total 3.12–3.55 다. 같은 bs 에서 매핑 4K 대비 차이는 −12.6 %(bs 16K, 매핑 16K)에서 +0.02 %(bs 64K, 매핑 16K) 사이다. 매핑 4K 는 bs 를 4K → 64K 로 키우면 416.0 → 486.3 MiB/s(+16.9 %)로 오른다.

bs < 매핑인 5 조합의 대역폭은 같은 매핑에서 bs = 매핑일 때의 0.16–0.52 배로, 조합마다 bs/매핑(0.125–0.5)보다 조금 크다. WAF_total 은 부분 쓰기 증폭(매핑/bs = 2·4·8 배)과 GC 증폭(2.1–3.0)의 곱이다. 16K/8K 는 5.90 = 2 × 2.95, 32K/8K 는 8.97 = 4 × 2.243 이다.

GC 이전 구간은 NAND 프로그램 한계로 정해진다. bs ≥ 매핑에서는 2,001–2,218 MiB/s 이고, bs < 매핑에서는 한계 × bs/매핑의 92–96 % 다(예: 16K/8K 1,072.0, 32K/8K 512.6 MiB/s). 첫 GC 는 모든 조합에서 5.69–6.32 s 에 시작한다. GC 이후 구간 평균은 bs ≥ 매핑에서 240–312 MiB/s, bs < 매핑에서 47–131 MiB/s 다.

평균 완료 지연은 iodepth 32 ÷ IOPS 로 정해진다. bs 64K 는 4.1–4.5 ms 다. p99 는 bs 가 커질수록 늘어, bs 4K 에서 13.3–14.5 ms, bs 64K 에서 19.0 ms(세 매핑 모두)다.

반복 간 대역폭 편차는 작다. 새로 잰 18 회의 6 조합은 변동계수 평균 0.083 %, 최대 0.280 %(4K/8K)다. 주 데이터셋 회차를 포함한 15 조합은 평균 0.123 %, 최대 0.375 %(4K/4K)다.

블록 계층 병합은 무시할 수 있다(커널 기본값, 주 데이터셋과 같은 설정). 새 회차에서 합쳐진 요청은 bs 8K 의 매핑 4K 에서 가장 많았는데, 회차당 약 1,800 건으로 요청의 0.054 % 다. 나머지 회차는 0–361 건(0.022 % 이하)이고, 장치가 받은 평균 요청 크기와 bs 의 차이는 최대 0.004 KiB 다.

해석: 랜덤 쓰기에서 매핑 단위를 키울 때의 손실은 호스트 쓰기가 매핑 단위보다 작을 때만 생기고, 그 크기는 bs/매핑 비율이 정한다. 쓰기 크기가 매핑 단위 이상이면 매핑 16K·32K 도 매핑 4K 와 비슷한 성능(최대 −12.6 %)을 낸다. 다만 NVMeVirt 는 부분 쓰기의 RMW 읽기를 모델링하지 않으므로, bs < 매핑의 불이익은 실제 SSD 보다 작게 나올 수 있다(5.5 절).

# 10. 실험 후 상태와 정리

실험 전(env_before)과 끝난 뒤(env_after_*) 환경 스냅샷을 비교했다. 날짜·부하·여유 메모리처럼 늘 바뀌는 값은 빼고 달라진 줄만 적었다.

표 64. 실험 전후 환경 차이

| 스냅샷 | 파일 | 달라진 줄 (- 전 / + 후) |
| --- | --- | --- |
| main3x3_20261008/env_after_wbuffix | 08_nvmevirt_git.txt | -1d6cd036893f06bfe8aa8f6bb4327239bc4b720f +a93ef76bbf990eafd1d0a5a9484a27a1b709ed9d -1d6cd03 exp: final 3x3 design with OS page-cache comparison +a93ef76 exp: final 3x3 design with OS page-cache comparison + M EXPERIMENT_LOG_FOR_CLAUDE.md + M exp/analyze.py … |
| main3x3_20261008/env_after_wbuffix | 09_block.txt | -/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 121.44 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 +/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 121.47 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 |
| main_20261008/env_after_stop | 03_cpu.txt | -CPU(s) scaling MHz: 58% +CPU(s) scaling MHz: 33% |
| main_20261008/env_after_stop | 08_nvmevirt_git.txt | -5769378d46821c45595585c932522e4f76538fc6 +2a462a3a518915e37fa313716bc9721801501e28 +2a462a3 exp: keep going when a run fails; plot tool; report generators -f84fec2 admin: Support queues in device memory + M EXPERIMENT_LOG_FOR_CLAUDE.md + M exp/report/make_report.py … |
| main_20261008/env_after_stop | 09_block.txt | -/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 101.15 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 +/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 121.44 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 |
| seq3x3_20261008/env_after_merge | 08_nvmevirt_git.txt | + M EXPERIMENT_LOG_FOR_CLAUDE.md + M README.md + M exp/report/make_md_results.py + M exp/report/make_report.py |
| seq3x3_20261008/env_after_merge | 09_block.txt | -/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 122.10 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 +/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 122.11 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 |
| seq3x3_20261008/env_after_wbuffix | 08_nvmevirt_git.txt | + M EXPERIMENT_LOG_FOR_CLAUDE.md + M README.md + M exp/report/make_md_results.py + M exp/report/make_report.py |
| seq3x3_20261008/env_after_wbuffix | 09_block.txt | -/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 122.10 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 +/dev/nvme0n1 /dev/ng0n1 S5GYNF0RA00765J Samsung SSD 980 PRO 500GB 0x1 122.11 GB / 500.11 GB 512 B + 0 B 3B2QGXA7 |
| randbs_20261008/env_after_wbuffix | 08_nvmevirt_git.txt | + M EXPERIMENT_LOG_FOR_CLAUDE.md + M exp/build_modules.sh + M exp/make_gallery.py + M exp/report/findings_seq_ko.txt + M exp/report/make_report.py + M exp/run_all_seq.sh |

표 65. 실험 전·중·후 설정 상태

| 항목 | 실험 전 | 실험 중 | 실험 후 |
| --- | --- | --- | --- |
| GRUB / 커널 명령줄 | memmap=12G$12G isolcpus=3-5 | 변경 없음 | 변경 없음 |
| nvmev 모듈 | 적재 안 됨 | 회차마다 적재·내림 | 적재 안 됨 (마지막 rmmod) |
| /dev/nvme1n1 | 없음 | 회차마다 생겼다 사라짐 | 없음 |
| /etc/sudoers.d/nvmevirt-exp | 없음 | 설치 (3.3 절) · 18:33 제거 후 순차 쓰기 실험용으로 20:17 다시 설치 · 랜덤 bs 8K·64K 실험용으로 22:11 다시 설치 | 제거함 (18:33 KST, sudo rm) · 순차 쓰기 실험 후 21:34 KST 다시 제거 · 22:11 다시 설치, 랜덤 bs 8K·64K 실험 후 23:08 KST 제거 |
| 블록 장치 nomerges | 0 (커널 기본) | 순차 쓰기 회차마다 insmod 직후 2 | 장치가 rmmod 로 사라져 설정도 없어짐 |
| CPU governor / 터보 | powersave / 켬 | 변경 없음 | 변경 없음 |
| NVMeVirt 저장소 위치 | – | /home/dccearth/jsw/KSC2026/nvmevirt | /home/dccearth/jsw/nvmevirt 로 이동 (사용자 지시) |
| 원본 모델(base)로 잰 결과 | – | 첫 설계에서 21 회(14:21–14:47), 스모크 테스트 1 회 | 23:33 사용자 요청으로 23:38 부터 문서와 저장소 최신 상태에서 뺌: 원자료 폴더, 그 수치·표·그림, 감사 원본 JSON. 이전 커밋과 태그 ksc2026-final·ksc2026-v2 에는 남아 있다. GC_STATS 영향 확인(5.3 절, bs = 매핑 단위)의 6 회는 남겼다 |
| 이전 실험 폴더 /home/dccearth/jsw/exp | 있음 (이전 iodepth 시험 등) | 사용 안 함 | 삭제 (사용자 지시; 수치는 인계 기록에 보존) |
| NVMeVirt 소스·결과·문서 | 원본 61c90f7 | e598e75 (3.4 절), 순차 쓰기 66446ea, 랜덤 bs 8K·64K a86fc87 | GitHub main 과 태그 ksc2026-v3 로 push (결과·문서·생성기 포함) |
| 예약 메모리 내용 | – | rmmod/insmod 는 FTL 상태만 초기화 (저장 데이터는 지우지 않음) | 이전 회차 데이터가 남아 있음 (쓰기 전용 실험이라 무관) |

커밋 재작성(모두 첫 push 전, 코드·스크립트는 그대로이고 인계 기록 파일 한 줄만 다름): ① 16:04 KST, 커밋 1d6cd03 에 인계 기록 파일의 한 줄(sudo 비밀번호와 서버 IP 앞부분 문자열)이 들어가 있어 그 줄만 고쳐 a93ef76 으로 다시 만들었다. ② 18:3x KST, 인계 기록 머리말에 들어간 사용자의 Claude 계정 이메일을 빼려고 2a462a3 → 949ae38, a93ef76 → e598e75 로 다시 만들었다. 따라서 원자료에 적힌 커밋은 다음과 같이 읽는다: 주 데이터셋 처음 4 회차(map4k_bs4k_r1·map4k_bs16k_r1 의 nodrop·drop)의 meta.txt 와 wbuffix_*/git_head.txt·env_before 의 1d6cd03, 나머지 50 회차의 a93ef76 = e598e75 / 보조 데이터셋 wbuffix 53 회차의 2a462a3 = 949ae38.

# 실험 후 정리 (서버) — 실제로 실행한 명령

lsmod | grep nvmev || echo "nvmev not loaded"

sudo rm /etc/sudoers.d/nvmevirt-exp      # 10 절 표의 시각 (18:33)

# 순차 쓰기 실험(8 절): 20:17 사용자가 다시 설치(3.3 절 install 명령), 실험 후 사용자가 다시 제거

sudo rm /etc/sudoers.d/nvmevirt-exp      # 21:34 KST

# 랜덤 쓰기 bs 8K·64K 실험(9 절): 22:11 사용자가 다시 설치, 실험 후 사용자가 다시 제거

sudo rm /etc/sudoers.d/nvmevirt-exp      # 23:08 KST

mv /home/dccearth/jsw/KSC2026/nvmevirt /home/dccearth/jsw/nvmevirt

rm -rf /home/dccearth/jsw/exp

# 부록 A. 스크립트 전문

## A. exp/common.sh

#!/usr/bin/env bash

# KSC2026 NVMeVirt mapping-unit experiment — common settings and helpers.

# Sourced by build_modules.sh, run_experiment.sh and collect_env.sh.

# Every value can be overridden from the environment (e.g. REPS=1 bash run_experiment.sh ...).

EXP_DIR="$(cd "$(dirname "${BASH_SOURCE[0]}")" && pwd)"

REPO_DIR="$(cd "$EXP_DIR/.." && pwd)"

NVMEV_SRC="$REPO_DIR/nvmevirt"

# ---- NVMeVirt module parameters (insmod) ----

MEMMAP_START="${MEMMAP_START:-12G}"   # GRUB_CMDLINE_LINUX: memmap=12G$12G  (reserve 12 GiB starting at 12 GiB)

MEMMAP_SIZE="${MEMMAP_SIZE:-12G}"

CPUS="${CPUS:-3,4,5}"                 # first = dispatcher, the rest = I/O workers (GRUB: isolcpus=3-5)

# ---- experiment matrix ----

MAPS="${MAPS:-4k 8k 16k 32k 64k 128k}"   # FTL mapping unit (make MAPPING_UNIT=...)

BSS="${BSS:-4k 8k 16k 32k 64k 128k}"     # fio block size

REPS="${REPS:-3}"                        # repetitions per (mapping unit, bs)

# ---- fio ----

WORKLOAD="${WORKLOAD:-randwrite}"   # job template jobs/<WORKLOAD>.fio.in: randwrite (rw=randwrite) | seqwrite (rw=write)

IODEPTH="${IODEPTH:-32}"

RUNTIME="${RUNTIME:-60}"      # seconds, time_based

RAMP="${RAMP:-0}"             # ramp_time 0: keep the pre-GC part of every run

LOG_MSEC="${LOG_MSEC:-500}"   # bw/iops/lat time-series averaging window (ms)

SETTLE_SEC="${SETTLE_SEC:-5}" # idle seconds between insmod and fio

# ---- OS page cache before each run ----

# nodrop: nothing / drop: sync; echo 3 > /proc/sys/vm/drop_caches right after insmod (before SETTLE_SEC)

# "nodrop drop": both, back to back for every (map, bs, rep); the order alternates with rep (odd: nodrop first)

CACHE_MODES="${CACHE_MODES:-nodrop}"

# ---- block layer request merging for the NVMeVirt namespace ----

# empty: leave the kernel default (0 = merging allowed); 2: echo 2 > /sys/block/<dev>/queue/nomerges after insmod,

# so the device sees exactly the fio bs (adjacent sequential requests are not merged into larger ones)

NOMERGES="${NOMERGES:-}"

# Expected NVMeVirt logical capacity: (12 GiB - 1 MiB) * 100 / 107 (OP 7 %), with some slack

DEV_MIN_BYTES=$(( 11 * 1024**3 ))

DEV_MAX_BYTES=$(( 12 * 1024**3 ))

log() { echo "[$(date '+%F %T')] $*"; }

die() { echo "[$(date '+%F %T')] ERROR: $*" >&2; exit 1; }

to_bytes() {   # 4k -> 4096, 1m -> 1048576

local v="${1,,}"

case "$v" in

*k) echo $(( ${v%k} * 1024 )) ;;

*m) echo $(( ${v%m} * 1024 * 1024 )) ;;

*) echo "$v" ;;

esac

}

# Build variants -> extra make arguments (all variants: conventional SSD, BLKS_PER_PLN 384, MAPPING_UNIT per module)

#   wbuffix  the model used for every result: write buffer holds whole mapping units (WBUF_FIX=1) + GC statistics

#            logging (observational only; does not change timing)

#   base     upstream write-buffer accounting + GC statistics. It over-releases the write buffer when bs < mapping

#            unit, so it is NOT used for results; kept only for the GC_STATS check at bs = mapping unit (pre_gcstats_ab)

#   plain    upstream write-buffer accounting without GC statistics (the other half of that check)

#   merge    wbuffix + write-buffer merge: writes smaller than the mapping unit are collected in one open

#            mapping unit per partition and written to flash once it is full (or replaced)

variant_make_args() {

case "$1" in

base) echo "GC_STATS=1" ;;

wbuffix) echo "GC_STATS=1 WBUF_FIX=1" ;;

merge) echo "GC_STATS=1 WBUF_FIX=1 WBUF_MERGE=1" ;;

plain) echo "" ;;

*) die "unknown VARIANT '$1'" ;;

esac

}

## A. exp/build_modules.sh

#!/usr/bin/env bash

# Build nvmev.ko once per FTL mapping unit:

#   exp/modules/<VARIANT>/nvmev_map<MAP>.ko  (+ build_map<MAP>.log, SHA256SUMS, build_info.txt)

# usage: bash build_modules.sh [VARIANT]      (VARIANT: wbuffix (default) | merge | base | plain, see common.sh)

set -euo pipefail

source "$(dirname "$0")/common.sh"

VARIANT="${1:-wbuffix}"

OUT="$EXP_DIR/modules/$VARIANT"

EXTRA_MAKE_ARGS="$(variant_make_args "$VARIANT")"

read -r -a MAP_LIST <<< "$MAPS"

mkdir -p "$OUT"

{

echo "date:            $(date -Is)"

echo "variant:         $VARIANT (extra make args: '${EXTRA_MAKE_ARGS}')"

echo "kernel:          $(uname -r)"

echo "compiler:        $(gcc --version | head -1)"

echo "kernel built by: $(sed -E 's/.*\((x86_64-linux-gnu-gcc[^)]*\)).*/\1/' /proc/version)"

echo "make:            $(make --version | head -1)"

echo "git HEAD:        $(git -C "$REPO_DIR" rev-parse HEAD)"

echo "git status (nvmevirt/): $(git -C "$REPO_DIR" status --porcelain -- nvmevirt | wc -l) modified file(s)"

echo

} > "$OUT/build_info.txt"

: > "$OUT/SHA256SUMS"

for m in "${MAP_LIST[@]}"; do

b=$(to_bytes "$m")

log "build nvmev.ko  MAPPING_UNIT=$b ($m)  variant=$VARIANT"

make -C "$NVMEV_SRC" clean > "$OUT/build_map$m.log" 2>&1

# shellcheck disable=SC2086

make -C "$NVMEV_SRC" MAPPING_UNIT="$b" $EXTRA_MAKE_ARGS >> "$OUT/build_map$m.log" 2>&1 \

|| die "build failed — see $OUT/build_map$m.log"

cp "$NVMEV_SRC/nvmev.ko" "$OUT/nvmev_map$m.ko"

(cd "$OUT" && sha256sum "nvmev_map$m.ko" >> SHA256SUMS)

{

echo "## nvmev_map$m.ko  (make MAPPING_UNIT=$b $EXTRA_MAKE_ARGS)"

modinfo "$OUT/nvmev_map$m.ko" | grep -E '^(srcversion|vermagic):'

} >> "$OUT/build_info.txt"

done

make -C "$NVMEV_SRC" clean > /dev/null 2>&1

log "done -> $OUT"

cat "$OUT/SHA256SUMS"

## A. exp/run_experiment.sh

#!/usr/bin/env bash

# KSC2026: NVMeVirt FTL mapping unit x fio block size write experiment (random or sequential, WORKLOAD in common.sh).

#

# Every run:  (rmmod nvmev if loaded) -> insmod nvmev_map<MAP>.ko (fresh FTL state = formatted device)

#             -> fio <WORKLOAD> bs=<BS>, iodepth 32, 60 s, ramp_time 0 (pre-GC part included) -> rmmod nvmev

# Order:      for rep in 1..REPS; for MAP in MAPS; for BS in BSS; for cache mode in CACHE_MODES (order alternates by rep)

#

# usage:  bash run_experiment.sh <EXP_NAME> [VARIANT]

#   - runs as the normal user; privileged steps use 'sudo -n' (/etc/sudoers.d/nvmevirt-exp)

#   - modules must be built first:  bash build_modules.sh [VARIANT]

#   - results: exp/results/<EXP_NAME>/<VARIANT>/map<MAP>_bs<BS>_r<REP>/   (CACHE_MODES=nodrop, default)

#              exp/results/<EXP_NAME>/<VARIANT>_<nodrop|drop>/...          (any other CACHE_MODES)

#   - re-running the same command resumes: runs with a DONE or FAILED marker are skipped

#   - a run whose fio fails (I/O errors, watchdog) gets a FAILED marker and the experiment continues

#   - ONLY_BS_LT_MAP=1 runs only the (MAP, BS) pairs with BS < MAP

set -euo pipefail

source "$(dirname "$0")/common.sh"

EXP="${1:?usage: bash run_experiment.sh <EXP_NAME> [VARIANT]}"

VARIANT="${2:-wbuffix}"

ONLY_BS_LT_MAP="${ONLY_BS_LT_MAP:-0}"

OUT="$EXP_DIR/results/$EXP"

read -r -a CM_LIST <<< "$CACHE_MODES"

vout_of() {   # result directory for cache mode $1

if [[ "$CACHE_MODES" == nodrop ]]; then echo "$OUT/$VARIANT"; else echo "$OUT/${VARIANT}_$1"; fi

}

VOUTS=()

for cm in "${CM_LIST[@]}"; do

[[ "$cm" == nodrop || "$cm" == drop ]] || { echo "CACHE_MODES: unknown mode '$cm'" >&2; exit 1; }

VOUTS+=("$(vout_of "$cm")")

done

MOD_DIR="$EXP_DIR/modules/$VARIANT"

JOB_TEMPLATE="$EXP_DIR/jobs/$WORKLOAD.fio.in"

[[ -f "$JOB_TEMPLATE" ]] || { echo "WORKLOAD: no job template $JOB_TEMPLATE" >&2; exit 1; }

[[ -z "$NOMERGES" || "$NOMERGES" =~ ^[012]$ ]] || { echo "NOMERGES must be empty, 0, 1 or 2" >&2; exit 1; }

UPSTREAM_COMMIT=61c90f7758cbd9545b4a4727e89377bf88eab060

FIO_GRACE="${FIO_GRACE:-180}"   # seconds allowed beyond RUNTIME before fio is stopped (nvme timeout 30 s + abort + reset)

read -r -a MAP_LIST <<< "$MAPS"

read -r -a BS_LIST <<< "$BSS"

ME="$(id -un)"

DEV=""

# ------------------------------------------------------------------ helpers

kmark() {   # write a marker line into the kernel log (same clock as NVMeVirt's messages)

echo "KSC2026-MARK $1" | sudo -n tee /dev/kmsg > /dev/null

}

dmesg_after() {   # kernel messages from marker $1 (inclusive) to the end

sudo -n dmesg | awk -v m="KSC2026-MARK $1" 'index($0, m) { found = 1 } found { print }'

}

find_nvmev_dev() {   # prints nvmeXnY of the NVMeVirt namespace (model CSL_Virt_MN_01), or nothing

local m found=()

for m in /sys/block/nvme*n*/device/model; do

[[ -e "$m" ]] || continue

if grep -q '^CSL_Virt' "$m"; then found+=("$(basename "$(dirname "$(dirname "$m")")")"); fi

done

(( ${#found[@]} <= 1 )) || die "more than one NVMeVirt namespace: ${found[*]}"

echo "${found[0]:-}"

}

check_target_dev() {   # refuse anything that is not exactly the NVMeVirt namespace; prints its size

local d="$1" size root_src

[[ "$(cat "/sys/block/$d/device/model" 2>/dev/null)" == CSL_Virt* ]] || die "/dev/$d is not an NVMeVirt device"

size=$(( $(cat "/sys/block/$d/size") * 512 ))

(( size >= DEV_MIN_BYTES && size <= DEV_MAX_BYTES )) || die "/dev/$d size $size B is outside the expected range"

if compgen -G "/sys/block/$d/${d}p*" > /dev/null; then die "/dev/$d has partitions — refusing"; fi

if lsblk -nro MOUNTPOINT "/dev/$d" | grep -q .; then die "/dev/$d is mounted — refusing"; fi

root_src="$(findmnt -n -o SOURCE /)"

[[ "$root_src" != "/dev/$d"* ]] || die "/dev/$d holds the root file system — refusing"

echo "$size"

}

unload_nvmev() {

grep -qs '^nvmev ' /proc/modules || return 0

local d; d="$(find_nvmev_dev)"

if [[ -n "$d" ]] && lsblk -nro MOUNTPOINT "/dev/$d" | grep -q .; then die "/dev/$d is mounted"; fi

sudo -n rmmod nvmev

for _ in $(seq 30); do grep -qs '^nvmev ' /proc/modules || break; sleep 1; done

if grep -qs '^nvmev ' /proc/modules; then die "rmmod nvmev failed"; fi

sleep 2

}

load_nvmev() {   # $1 = mapping unit (4k ...), $2 = run dir  -> sets DEV

local map="$1" rdir="$2" ko="$MOD_DIR/nvmev_map$1.ko" d="" tag

tag="$(basename "$rdir") insmod $(date +%s%N)"

kmark "$tag"

sudo -n insmod "$ko" memmap_start="$MEMMAP_START" memmap_size="$MEMMAP_SIZE" cpus="$CPUS"

for _ in $(seq 60); do

d="$(find_nvmev_dev)"

[[ -n "$d" && -b "/dev/$d" ]] && break

sleep 0.5

done

[[ -n "$d" && -b "/dev/$d" ]] || die "NVMeVirt block device did not appear after insmod (check dmesg)"

sleep 1

dmesg_after "$tag" > "$rdir/dmesg_load.txt"

grep -q "KSC2026: mapping unit=$(to_bytes "$map") B" "$rdir/dmesg_load.txt" \

|| die "dmesg does not confirm mapping unit $(to_bytes "$map") B — see $rdir/dmesg_load.txt"

DEV="/dev/$d"

}

queue_line() {   # block-layer settings of the NVMeVirt namespace

local q="/sys/block/$1/queue"

echo "nomerges=$(cat "$q/nomerges") scheduler=$(cat "$q/scheduler" | xargs) max_sectors_kb=$(cat "$q/max_sectors_kb") write_cache=$(cat "$q/write_cache" | xargs)"

}

blk_wr_stat() {   # /sys/block/<dev>/stat fields 5-7: write I/Os, write merges, write sectors

awk '{ print $5, $6, $7 }' "/sys/block/$1/stat"

}

meminfo_line() { awk '/^(MemFree|Buffers|Cached|Dirty|Writeback):/ { printf "%s%s kB ", $1, $2 }' /proc/meminfo; }

run_one() {   # $1 = mapping unit, $2 = bs, $3 = repetition, $4 = cache mode (nodrop | drop)

local map="$1" bs="$2" r="$3" cm="${4:-nodrop}" rdir size rc=0 tag_fio tag_un n_chm n_warn fio_pid waited=0 jerr

local wr_ios0 wr_mrg0 wr_sec0 wr_ios1 wr_mrg1 wr_sec1

rdir="$(vout_of "$cm")/map${map}_bs${bs}_r${r}"

if [[ -f "$rdir/DONE" ]]; then log "skip (already done): $(basename "$rdir")"; return 0; fi

if [[ -f "$rdir/FAILED" ]]; then log "skip (failed earlier, see FAILED): $(basename "$rdir")"; return 0; fi

rm -rf "$rdir"

mkdir -p "$rdir"

log "=== [$((++RUN_IDX))/$N_RUNS] variant=$VARIANT cache=$cm map=$map bs=$bs rep=$r ==="

unload_nvmev                     # rmmod: make sure no previous FTL state is left

# Follow the kernel log for the whole run: NVMeVirt's '[chmodel_request] Need to increase array size'

# error flood (bs < mapping unit without WBUF_FIX) can overwrite the 256 KiB ring buffer. Those lines are

# counted on the fly (first 20 kept) so kernel.log stays small.

sudo -n dmesg -W 2>&1 > >(awk -v cnt="$rdir/chmodel_msgs.txt" '

/\[chmodel_request\]/ { n++; if (n <= 20) print; next }

{ print; fflush() }

END { print n + 0 > cnt }' > "$rdir/kernel.log") &

KLOG_PID=$!

sleep 0.5

load_nvmev "$map" "$rdir"        # insmod: fresh (empty) device

size=$(check_target_dev "${DEV#/dev/}")

if [[ -n "$NOMERGES" ]]; then

echo "$NOMERGES" | sudo -n tee "/sys/block/${DEV#/dev/}/queue/nomerges" > /dev/null \

|| die "could not set /sys/block/${DEV#/dev/}/queue/nomerges (sudoers rule?)"

fi

# OS page cache: the NVMeVirt device is used with O_DIRECT, so this only changes the host's memory state

{ echo "cache_mode: $cm"; echo "before: $(meminfo_line)"; } > "$rdir/cache.txt"

if [[ "$cm" == drop ]]; then

sync

echo 3 | sudo -n tee /proc/sys/vm/drop_caches > /dev/null

echo "dropped: sync; echo 3 > /proc/sys/vm/drop_caches at $(date -Is)" >> "$rdir/cache.txt"

fi

echo "after:  $(meminfo_line)" >> "$rdir/cache.txt"

sed -e "s|@DEV@|$DEV|g" -e "s|@BS@|$bs|g" -e "s|@IODEPTH@|$IODEPTH|g" \

-e "s|@RUNTIME@|$RUNTIME|g" -e "s|@RAMP@|$RAMP|g" -e "s|@LOG_MSEC@|$LOG_MSEC|g" \

-e "s|@LOG_PREFIX@|$rdir/fio|g" "$JOB_TEMPLATE" > "$rdir/job.fio"

{

echo "variant:      $VARIANT"

echo "workload:     $WORKLOAD ($(grep -m1 '^rw=' "$rdir/job.fio"))"

echo "cache_mode:   $cm"

echo "map_unit:     $map ($(to_bytes "$map") B)"

echo "bs:           $bs ($(to_bytes "$bs") B)"

echo "rep:          $r"

echo "device:       $DEV  size=$size B  model=$(cat "/sys/block/${DEV#/dev/}/device/model" | xargs)"

echo "module:       $(cd "$MOD_DIR" && sha256sum "nvmev_map$map.ko")"

echo "insmod:       insmod nvmev_map$map.ko memmap_start=$MEMMAP_START memmap_size=$MEMMAP_SIZE cpus=$CPUS"

echo "fio:          fio --output-format=json --output=fio.json job.fio"

echo "blk_queue:    $(queue_line "${DEV#/dev/}")"

echo "loadavg:      $(cat /proc/loadavg)"

echo "git_head:     $(git -C "$REPO_DIR" rev-parse HEAD)"

echo "start:        $(date -Is)"

} > "$rdir/meta.txt"

sleep "$SETTLE_SEC"

echo "pre_fio: $(meminfo_line)" >> "$rdir/cache.txt"

tag_fio="$(basename "$rdir") fio-start $(date +%s%N)"

read -r wr_ios0 wr_mrg0 wr_sec0 <<< "$(blk_wr_stat "${DEV#/dev/}")"

kmark "$tag_fio"                 # GC onset is measured from this kernel-log timestamp

sudo -n fio --output-format=json --output="$rdir/fio.json" "$rdir/job.fio" > "$rdir/fio_stdout.txt" 2>&1 &

fio_pid=$!

while kill -0 "$fio_pid" 2> /dev/null; do   # watchdog: fio must end within RUNTIME + FIO_GRACE

sleep 1

waited=$((waited + 1))

if (( waited == RUNTIME + FIO_GRACE )); then

echo "watchdog: SIGTERM to fio after ${waited} s" >> "$rdir/fio_stdout.txt"

kill -TERM "$fio_pid" 2> /dev/null || true

elif (( waited == RUNTIME + FIO_GRACE + 60 )); then

echo "watchdog: SIGKILL to fio after ${waited} s" >> "$rdir/fio_stdout.txt"

kill -KILL "$fio_pid" 2> /dev/null || true

fi

done

wait "$fio_pid" || rc=$?

kmark "$(basename "$rdir") fio-end"

read -r wr_ios1 wr_mrg1 wr_sec1 <<< "$(blk_wr_stat "${DEV#/dev/}")"

tag_un="$(basename "$rdir") rmmod $(date +%s%N)"

kmark "$tag_un"

unload_nvmev                     # rmmod right after the run (GC statistics are printed here)

sleep 0.5

kill "$KLOG_PID" 2> /dev/null || true

wait "$KLOG_PID" 2> /dev/null || true

KLOG_PID=""

for _ in $(seq 120); do [[ -s "$rdir/chmodel_msgs.txt" ]] && break; sleep 0.5; done   # awk flushes at EOF

sudo -n chown -R "$ME:$ME" "$rdir"

# split the followed kernel log: run section (without channel-model overflow lines) / rmmod section

awk -v a="KSC2026-MARK $tag_fio" -v b="KSC2026-MARK $tag_un" -v run="$rdir/dmesg_run.txt" -v un="$rdir/dmesg_unload.txt" '

index($0, a) { s = 1 }

index($0, b) { s = 2 }

s == 1 && !/\[chmodel_request\]/ { print > run }

s == 2 { print > un }' "$rdir/kernel.log"

touch "$rdir/dmesg_run.txt" "$rdir/dmesg_unload.txt"

n_chm=$(cat "$rdir/chmodel_msgs.txt" 2> /dev/null || echo NA)

n_warn=$(grep -v -e KSC2026 -e '\[chmodel_request\]' "$rdir/kernel.log" \

| grep -ciE 'WARNING|almost full|timeout|reset|Oops|BUG|I/O error|Disk read failed' || true)

{

echo "end:          $(date -Is)"

echo "blk_writes:   ios=$((wr_ios1 - wr_ios0)) merges=$((wr_mrg1 - wr_mrg0)) sectors=$((wr_sec1 - wr_sec0))   (block layer, /sys/block/<dev>/stat during fio)"

echo "fio_exit:     $rc"

echo "chmodel_msgs: $n_chm   (NVMeVirt '[chmodel_request]' errors seen by dmesg -W: NAND backlog beyond the channel-model window)"

echo "kernel_warn:  $n_warn   (kernel.log lines matching WARNING|almost full|timeout|reset|Oops|BUG|I/O error|Disk read failed, excluding chmodel/KSC2026 lines)"

} >> "$rdir/meta.txt"

jerr=$(python3 -c 'import json,sys; print(json.load(open(sys.argv[1]))["jobs"][0]["error"])' "$rdir/fio.json" 2> /dev/null || echo NA)

if [[ $rc -ne 0 || "$jerr" != 0 ]]; then

{

echo "fio_exit=$rc fio_json_error=$jerr kernel_warn=$n_warn chmodel_msgs=$n_chm"

grep -v -e '\[chmodel_request\]' "$rdir/kernel.log" | grep -iE 'almost full|timeout|reset|disable|I/O error' | head -20

} > "$rdir/FAILED"

log "FAILED (fio exit $rc, json error $jerr) — kept as $rdir/FAILED, continuing"

return 0

fi

if grep -v KSC2026 "$rdir/dmesg_run.txt" | grep -qiE 'error|bug|oops|warn'; then

log "WARNING: kernel messages during the run — see $rdir/dmesg_run.txt"

fi

if [[ "$n_chm" != 0 ]]; then log "NOTE: $n_chm channel-model overflow messages (first 20 in kernel.log)"; fi

if [[ "$n_warn" != 0 ]]; then log "NOTE: $n_warn kernel warning line(s) — see $rdir/kernel.log"; fi

python3 - "$rdir/fio.json" <<'PY'

import json, sys

w = json.load(open(sys.argv[1]))["jobs"][0]["write"]

print(f"    -> {w['bw'] / 1024:.1f} MiB/s, {w['iops']:.0f} IOPS, clat mean {w['clat_ns']['mean'] / 1000:.1f} us")

PY

touch "$rdir/DONE"

}

# ------------------------------------------------------------------ preflight

PRIV=(/usr/sbin/insmod /usr/sbin/rmmod /usr/bin/fio /usr/bin/dmesg "/usr/bin/tee /dev/kmsg")

[[ " ${CM_LIST[*]} " == *" drop "* ]] && PRIV+=("/usr/bin/tee /proc/sys/vm/drop_caches")

for c in "${PRIV[@]}"; do

# shellcheck disable=SC2086

sudo -n -l $c > /dev/null 2>&1 || die "'sudo -n $c' is not allowed — install /etc/sudoers.d/nvmevirt-exp first"

done

grep -qF "memmap=${MEMMAP_SIZE}\$${MEMMAP_START}" /proc/cmdline \

|| die "kernel command line lacks memmap=${MEMMAP_SIZE}\$${MEMMAP_START}"

for m in "${MAP_LIST[@]}"; do

[[ -f "$MOD_DIR/nvmev_map$m.ko" ]] || die "missing $MOD_DIR/nvmev_map$m.ko — run: bash build_modules.sh $VARIANT"

done

(cd "$MOD_DIR" && sha256sum -c --quiet SHA256SUMS) || die "module checksum mismatch in $MOD_DIR"

PAIRS=()

for m in "${MAP_LIST[@]}"; do

for bs in "${BS_LIST[@]}"; do

if [[ "$ONLY_BS_LT_MAP" == 1 ]] && (( $(to_bytes "$bs") >= $(to_bytes "$m") )); then continue; fi

PAIRS+=("$m $bs")

done

done

N_RUNS=$(( REPS * ${#PAIRS[@]} * ${#CM_LIST[@]} ))

RUN_IDX=0

mkdir -p "${VOUTS[@]}"

exec > >(tee -a "${VOUTS[@]/%//run.log}") 2>&1

KLOG_PID=""

trap '[[ -n "$KLOG_PID" ]] && kill "$KLOG_PID" 2> /dev/null; sudo -n chown -R "$ME:$ME" "$OUT" 2> /dev/null || true' EXIT

log "experiment=$EXP variant=$VARIANT maps=[$MAPS] bss=[$BSS] reps=$REPS cache_modes=[$CACHE_MODES] only_bs_lt_map=$ONLY_BS_LT_MAP"

log "fio: libaio $WORKLOAD iodepth=$IODEPTH runtime=${RUNTIME}s ramp_time=${RAMP}s log=${LOG_MSEC}ms; insmod memmap_start=$MEMMAP_START memmap_size=$MEMMAP_SIZE cpus=$CPUS"

log "block layer: nomerges=${NOMERGES:-kernel default}"

log "$N_RUNS runs, about $(( N_RUNS * (RUNTIME + SETTLE_SEC + 8) / 60 )) min"

[[ -d "$OUT/env_before" ]] || bash "$EXP_DIR/collect_env.sh" "$OUT/env_before"

MOVE_COMMIT=$(git -C "$REPO_DIR" log --format=%H --grep='^Move NVMeVirt sources into nvmevirt/' -1)

for VOUT in "${VOUTS[@]}"; do

cp "$MOD_DIR/SHA256SUMS" "$VOUT/modules_SHA256SUMS"

cp "$MOD_DIR/build_info.txt" "$VOUT/modules_build_info.txt"

cp "$JOB_TEMPLATE" "$VOUT/"

echo "$(date -Is) $(git -C "$REPO_DIR" rev-parse HEAD)" >> "$VOUT/git_head.txt"   # one line per invocation

git -C "$REPO_DIR" diff "$MOVE_COMMIT" HEAD -- nvmevirt > "$VOUT/nvmevirt_vs_upstream.diff"   # upstream + pure rename -> HEAD

git -C "$REPO_DIR" diff -- nvmevirt > "$VOUT/nvmevirt_uncommitted.diff"

done

for r in $(seq "$REPS"); do

for p in "${PAIRS[@]}"; do

read -r m bs <<< "$p"

order=("${CM_LIST[@]}")

if (( r % 2 == 0 )); then   # even repetitions: reversed order, so neither mode always runs first

order=()

for (( i = ${#CM_LIST[@]} - 1; i >= 0; i-- )); do order+=("${CM_LIST[i]}"); done

fi

for cm in "${order[@]}"; do run_one "$m" "$bs" "$r" "$cm"; done

done

done

unload_nvmev

bash "$EXP_DIR/collect_env.sh" "$OUT/env_after_$VARIANT"

log "finished: ${VOUTS[*]}"

## A. exp/collect_env.sh

#!/usr/bin/env bash

# Snapshot of the server and software environment (taken before and after the experiment).

# usage: bash collect_env.sh <out dir>

set -uo pipefail

source "$(dirname "$0")/common.sh"

D="${1:?usage: bash collect_env.sh <out dir>}"

mkdir -p "$D"

# a new snapshot replaces an old one in the same directory (cap appends). Before 2026-10-08 21:3x KST the files were

# appended to, so results/seq3x3_20261008/env_after_* hold three snapshots in a row (one per run_experiment.sh call)

rm -f "$D"/[0-9][0-9]_*.txt

cap() {   # cap <file> <command...>: save the command line and its output

local f="$1"; shift

{ echo "\$ $*"; "$@" 2>&1; echo; } >> "$D/$f"

}

cap 00_date.txt            date -Is

cap 00_date.txt            hostname

cap 01_os.txt              uname -a

cap 01_os.txt              cat /etc/os-release

cap 01_os.txt              cat /proc/version

cap 02_boot.txt            cat /proc/cmdline

cap 02_boot.txt            grep -E '^GRUB_CMDLINE' /etc/default/grub

cap 02_boot.txt            cat /sys/devices/system/cpu/isolated

cap 03_cpu.txt             lscpu

cap 03_cpu.txt             nproc

cap 04_cpufreq.txt         cat /sys/devices/system/cpu/intel_pstate/status /sys/devices/system/cpu/intel_pstate/no_turbo

cap 04_cpufreq.txt         grep -H . /sys/devices/system/cpu/cpu[0-9]*/cpufreq/scaling_driver \

/sys/devices/system/cpu/cpu[0-9]*/cpufreq/scaling_governor \

/sys/devices/system/cpu/cpu[0-9]*/cpufreq/energy_performance_preference \

/sys/devices/system/cpu/cpu[0-9]*/cpufreq/scaling_min_freq \

/sys/devices/system/cpu/cpu[0-9]*/cpufreq/scaling_max_freq

cap 05_memory.txt          free -b

cap 05_memory.txt          grep -E 'MemTotal|MemFree|MemAvailable|Hugepages|AnonHugePages' /proc/meminfo

cap 05_memory.txt          cat /sys/kernel/mm/transparent_hugepage/enabled

cap 05_memory.txt          sudo -n cat /proc/iomem

cap 05_memory.txt          bash -c "sudo -n dmesg | grep -iE 'e820|memmap|user-defined' | head -60"

cap 06_platform.txt        grep -H . /sys/class/dmi/id/sys_vendor /sys/class/dmi/id/product_name \

/sys/class/dmi/id/board_vendor /sys/class/dmi/id/board_name /sys/class/dmi/id/bios_version /sys/class/dmi/id/bios_date

cap 07_versions.txt        fio --version

cap 07_versions.txt        gcc --version

cap 07_versions.txt        make --version

cap 07_versions.txt        python3 --version

cap 07_versions.txt        git --version

cap 07_versions.txt        nvme version

cap 07_versions.txt        dmesg --version

cap 07_versions.txt        bash --version

cap 07_versions.txt        dpkg-query -W fio libaio1t64 nvme-cli make gcc-13 "linux-headers-$(uname -r)" "linux-image-$(uname -r)"

cap 08_nvmevirt_git.txt    git -C "$REPO_DIR" rev-parse HEAD

cap 08_nvmevirt_git.txt    git -C "$REPO_DIR" log --oneline -15

cap 08_nvmevirt_git.txt    git -C "$REPO_DIR" status --porcelain

cap 08_nvmevirt_git.txt    git -C "$REPO_DIR" remote -v

cap 09_block.txt           lsblk -o NAME,SIZE,MODEL,TYPE,MOUNTPOINT

cap 09_block.txt           bash -c 'ls -l /dev/nvme*'

cap 09_block.txt           sudo -n nvme list

cap 10_modules.txt         bash -c "lsmod | grep -E '^(Module|nvmev)'"

cap 10_modules.txt         cat /proc/loadavg

cap 11_kernel_sysctl.txt   sysctl kernel.dmesg_restrict kernel.kptr_restrict kernel.numa_balancing vm.swappiness

echo "environment snapshot -> $D"

## A. exp/jobs/randwrite.fio.in

; KSC2026 NVMeVirt mapping-unit experiment — fresh-device random write.

; Template: run_experiment.sh replaces @VARS@ and saves the concrete job file as <run dir>/job.fio

[global]

filename=@DEV@

ioengine=libaio

direct=1

rw=randwrite

bs=@BS@

iodepth=@IODEPTH@

numjobs=1

time_based=1

runtime=@RUNTIME@

ramp_time=@RAMP@

group_reporting=1

; same random offset sequence in every run (fio default, written out for clarity)

randrepeat=1

percentile_list=50:90:95:99:99.9:99.99

; time series (averaged over @LOG_MSEC@ ms) to see the pre-GC and post-GC parts of the run

write_bw_log=@LOG_PREFIX@

write_iops_log=@LOG_PREFIX@

write_lat_log=@LOG_PREFIX@

log_avg_msec=@LOG_MSEC@

[randwrite]

## A. exp/jobs/seqwrite.fio.in

; KSC2026 NVMeVirt mapping-unit experiment — fresh-device sequential write.

; Template: run_experiment.sh replaces @VARS@ and saves the concrete job file as <run dir>/job.fio

[global]

filename=@DEV@

ioengine=libaio

direct=1

rw=write

; time_based: after the last block of the device fio wraps around to offset 0 and keeps writing sequentially

bs=@BS@

iodepth=@IODEPTH@

numjobs=1

time_based=1

runtime=@RUNTIME@

ramp_time=@RAMP@

group_reporting=1

percentile_list=50:90:95:99:99.9:99.99

; time series (averaged over @LOG_MSEC@ ms) to see the pre-GC and post-GC parts of the run

write_bw_log=@LOG_PREFIX@

write_iops_log=@LOG_PREFIX@

write_lat_log=@LOG_PREFIX@

log_avg_msec=@LOG_MSEC@

[seqwrite]

## A. exp/run_all.sh

#!/usr/bin/env bash

# KSC2026 (final design, results/main3x3_*): mapping unit 4k/16k/32k x fio bs 4k/16k/32k x 3 reps,

# wbuffix model, OS page cache dropped (drop) or not (nodrop) before every run, the two back to back.

# usage: bash run_all.sh <EXP_NAME>        (run inside tmux; re-running resumes)

set -euo pipefail

cd "$(dirname "$0")"

EXP="${1:?usage: bash run_all.sh <EXP_NAME>}"

export MAPS="4k 16k 32k" BSS="4k 16k 32k" REPS=3 CACHE_MODES="nodrop drop"

ls modules/wbuffix/nvmev_map{4k,16k,32k}.ko > /dev/null 2>&1 || bash build_modules.sh wbuffix   # .ko files are not in git

bash run_experiment.sh "$EXP" wbuffix

./.venv/bin/python analyze.py "results/$EXP"

## A. exp/run_all_6x6.sh

#!/usr/bin/env bash

# KSC2026 (first design, used for results/main_20261008): build -> wbuffix 6x6x3 -> analysis.

# (Until 2026-10-08 this script also ran the variant "base" first. base has the upstream write-buffer over-release

#  when bs < mapping unit; its results were removed at the user's request and it is no longer run.)

# usage: bash run_all_6x6.sh <EXP_NAME>        (run inside tmux; re-running resumes)

set -euo pipefail

cd "$(dirname "$0")"

EXP="${1:?usage: bash run_all_6x6.sh <EXP_NAME>}"

bash build_modules.sh wbuffix

bash run_experiment.sh "$EXP" wbuffix

./.venv/bin/python analyze.py "results/$EXP"

## A. exp/run_all_seq.sh

#!/usr/bin/env bash

# KSC2026 sequential-write experiment (results/seq3x3_*): mapping unit 4k/16k/32k x fio bs 4k/16k/32k x 3 reps,

# fio rw=write, page cache not dropped, two models:

#   wbuffix  the model of the random-write experiment (same module binaries as results/main3x3_*)

#   merge    wbuffix + write-buffer merge of writes smaller than the mapping unit (WBUF_MERGE=1)

# The two models alternate by repetition (rep 1 wbuffix, rep 1 merge, rep 2 wbuffix, ...).

# usage: bash run_all_seq.sh <EXP_NAME>        (run inside tmux; re-running resumes)

set -euo pipefail

cd "$(dirname "$0")"

EXP="${1:?usage: bash run_all_seq.sh <EXP_NAME>}"

export MAPS="4k 16k 32k" BSS="4k 16k 32k" CACHE_MODES=nodrop WORKLOAD=seqwrite

export NOMERGES="${NOMERGES:-2}"   # the device must see the fio bs: no block-layer merging of adjacent requests

for v in wbuffix merge; do   # .ko files are not in git: build what is missing

ls modules/$v/nvmev_map{4k,16k,32k}.ko > /dev/null 2>&1 || bash build_modules.sh $v

done

for r in 1 2 3; do

for v in wbuffix merge; do

# runs with a DONE marker are skipped, so this adds repetition r (the "N runs" / "[i/N]" in the log count

# repetitions 1..r, so each call ends at about [9/N] — expected)

REPS=$r bash run_experiment.sh "$EXP" "$v"

done

done

./.venv/bin/python analyze.py "results/$EXP"

## A. exp/run_all_rand_bs.sh

#!/usr/bin/env bash

# KSC2026 random-write extension (results/randbs_*): fio bs 8k and 64k on mapping units 4k/16k/32k x 3 reps,

# wbuffix model, page cache not dropped, block-layer settings as in results/main3x3_* (kernel default).

# The 3x3 combinations (bs 4k/16k/32k) are not measured again: link_runs.py puts the new runs next to the

# nodrop runs of the primary data set in a combined view (results/rand3x5_*: mapping 3 x bs 5).

# usage: bash run_all_rand_bs.sh <EXP_NAME> [PRIMARY_EXP] [VIEW_NAME]     (run inside tmux; re-running resumes)

set -euo pipefail

cd "$(dirname "$0")"

EXP="${1:?usage: bash run_all_rand_bs.sh <EXP_NAME> [PRIMARY_EXP] [VIEW_NAME]}"

PRIMARY="${2:-main3x3_20261008}"

VIEW="${3:-rand3x5_${EXP##*_}}"

export MAPS="4k 16k 32k" BSS="8k 64k" REPS=3 CACHE_MODES=nodrop WORKLOAD=randwrite NOMERGES=

ls modules/wbuffix/nvmev_map{4k,16k,32k}.ko > /dev/null 2>&1 || bash build_modules.sh wbuffix   # .ko files are not in git

bash run_experiment.sh "$EXP" wbuffix

./.venv/bin/python analyze.py "results/$EXP"

./.venv/bin/python link_runs.py "results/$VIEW" "results/$PRIMARY/wbuffix_nodrop" "results/$EXP/wbuffix"

./.venv/bin/python analyze.py "results/$VIEW"

## A. exp/link_runs.py

#!/usr/bin/env python3

"""Build a combined view of run directories from several data sets (relative symlinks, no copies).

usage: python3 link_runs.py results/<VIEW> <run-dir parent> [<run-dir parent> ...] [--variant NAME]

Every <parent>/map<M>_bs<B>_r<R>/ that has a DONE marker is linked as results/<VIEW>/<NAME>/map<M>_bs<B>_r<R>

(NAME defaults to wbuffix). analyze.py, plot.py and make_md_results.py then treat the view like any data set.

The same run name must not come from two parents; nothing is linked when that happens. SOURCES.txt in the view

lists every link that exists under the view (all variants), so it stays complete when the tool is called again.

"""

import os

import re

import sys

from pathlib import Path

RUN_RE = re.compile(r"^map\d+k_bs\d+k_r\d+$")

args = sys.argv[1:]

name = "wbuffix"

if "--variant" in args:

i = args.index("--variant")

name = args[i + 1]

del args[i:i + 2]

if len(args) < 2:

sys.exit(__doc__)

view = Path(args[0]).resolve()

dst = view / name

# 1. collect and check everything before touching the view

seen = {}

for parent in map(lambda a: Path(a).resolve(), args[1:]):

if not parent.is_dir():

sys.exit(f"{parent} is not a directory")

for run in sorted(parent.iterdir()):

if not RUN_RE.match(run.name) or not (run / "DONE").exists():

continue

if run.name in seen and seen[run.name] != run:

sys.exit(f"{run.name} comes from both {seen[run.name]} and {run} — nothing linked")

seen[run.name] = run

if not seen:

sys.exit("no finished runs (map*_bs*_r* with DONE) under the given directories")

for run_name, run in seen.items():

link, rel = dst / run_name, os.path.relpath(run, dst)

if link.is_symlink():

if os.readlink(link) != rel:

sys.exit(f"{link} exists and points elsewhere ({os.readlink(link)}) — nothing linked")

elif link.exists():

sys.exit(f"{link} exists and is not a symlink — nothing linked")

# 2. link

dst.mkdir(parents=True, exist_ok=True)

for run_name, run in seen.items():

link = dst / run_name

if not link.is_symlink():

link.symlink_to(os.path.relpath(run, dst), target_is_directory=True)

# 3. SOURCES.txt from what is really there

lines = []

for link in sorted(p for p in view.glob("*/map*_bs*_r*") if p.is_symlink()):

target = (link.parent / os.readlink(link)).resolve()

note = "" if (target / "DONE").exists() else "   (WARNING: target has no DONE marker)"

lines.append(f"{link.parent.name}/{link.name} -> {os.path.relpath(target, view)}{note}")

(view / "SOURCES.txt").write_text("# combined view made by exp/link_runs.py (relative symlinks)\n" + "\n".join(lines) + "\n")

print(f"{view}: {len(seen)} runs linked into {name}/")

## A. exp/analyze.py

#!/usr/bin/env python3

"""KSC2026 NVMeVirt mapping-unit experiment: collect fio results into CSV tables and figures.

usage:  python3 analyze.py results/<EXP_NAME>

reads   results/<EXP>/<variant>/map<MAP>_bs<BS>_r<REP>/{fio.json, fio_bw.1.log, meta.txt, dmesg_*.txt}

writes  results/<EXP>/analysis/summary_runs.csv     one row per run

results/<EXP>/analysis/summary_agg.csv      mean / std / min / max over repetitions

results/<EXP>/analysis/fig_*.png

"""

import csv

import json

import re

import statistics

import sys

from pathlib import Path

import matplotlib

matplotlib.use("Agg")

import matplotlib.pyplot as plt  # noqa: E402

import numpy as np  # noqa: E402

SIZES = ["4k", "8k", "16k", "32k", "64k", "128k"]   # replaced in main() by the sizes present in the results

MAPS, BSS = list(SIZES), list(SIZES)   # mapping units (rows) and fio bs (columns) present; set in main()

WORKLOAD = "Random-write"   # "Sequential-write" when the runs' meta.txt says workload: seqwrite (set in main())

# categorical slots (fixed order) and chart chrome — light mode

SERIES = ["#2a78d6", "#eb6834", "#1baf7a", "#eda100", "#e87ba4", "#008300", "#4a3aa7", "#e34948"]

INK, INK2, MUTED, GRID, AXIS, SURFACE = "#0b0b0b", "#52514e", "#898781", "#e1e0d9", "#c3c2b7", "#fcfcfb"

SEQ_BLUE = ["#cde2fb", "#9ec5f4", "#6da7ec", "#3987e5", "#256abf", "#184f95", "#0d366b"]

RUN_RE = re.compile(r"^map(?P<map>\d+k)_bs(?P<bs>\d+k)_r(?P<rep>\d+)$")

DMESG_TS = re.compile(r"^\[\s*(\d+\.\d+)\]")

EARLY_S, LATE_S = 10.0, 20.0   # early window = first 10 s, late window = last 20 s

def kib(s):

return int(s[:-1])

def meta_value(meta, key):

m = re.search(rf"^{key}:\s*(.*)$", meta, re.M)

return m.group(1).strip() if m else ""

def read_log(path):

"""fio log line: time(ms), value, direction, bs, offset[, prio]."""

t, v = [], []

if not path.exists():

return np.array(t), np.array(v)

for line in path.read_text().splitlines():

p = line.split(",")

if len(p) >= 2:

t.append(int(p[0]) / 1000.0)

v.append(float(p[1]))

return np.array(t), np.array(v)

def gc_info(rdir):

"""KSC2026 GC instrumentation in dmesg: first GC time (relative to the fio-start marker), counters at rmmod."""

out = {}

run = rdir / "dmesg_run.txt"

if run.exists():

fio_start, firsts = None, []

for line in run.read_text().splitlines():

m = DMESG_TS.match(line)

if not m:

continue

if "KSC2026-MARK" in line and "fio-start" in line and fio_start is None:

fio_start = float(m.group(1))

elif "KSC2026: first GC" in line:

firsts.append(float(m.group(1)))

if fio_start is not None and firsts:

out["gc_onset_s"] = min(firsts) - fio_start

out["gc_onset_last_part_s"] = max(firsts) - fio_start

unload = rdir / "dmesg_unload.txt"

if unload.exists():

tot = {"host_pgs": 0, "gc_pgs": 0, "gc_cnt": 0}

seen = False

for line in unload.read_text().splitlines():

m = re.search(r"KSC2026: stats part=\d+ host_pgs=(\d+) gc_pgs=(\d+) gc_cnt=(\d+)", line)

if m:

seen = True

tot["host_pgs"] += int(m.group(1))

tot["gc_pgs"] += int(m.group(2))

tot["gc_cnt"] += int(m.group(3))

if seen:

out.update(tot)

mg = {"mg_open": 0, "mg_merge": 0, "mg_full": 0, "mg_evict": 0, "mg_direct": 0, "mg_still_open": 0}

seen = False

for line in unload.read_text().splitlines():   # merge variant (WBUF_MERGE=1) only

m = re.search(r"KSC2026: merge part=\d+ open=(\d+) merge=(\d+) full=(\d+) evict=(\d+) direct=(\d+) still_open=(\d+)", line)

if m:

seen = True

for k, v in zip(mg, m.groups()):

mg[k] += int(v)

if seen:

out.update(mg)

return out

def kernel_warnings(rdir):

"""Kernel log lines that look like problems, other than NVMeVirt's own channel-model messages."""

log = rdir / "kernel.log"

if not log.exists():

return 0

pat = re.compile(r"WARNING|almost full|timeout|reset|Oops|BUG|Disk read failed|I/O error", re.I)

return sum(1 for ln in log.read_text(errors="replace").splitlines()

if "[chmodel_request]" not in ln and "KSC2026" not in ln and pat.search(ln))

def collect(exp_dir):

rows, series = [], {}

for vdir in sorted(p for p in exp_dir.iterdir() if p.is_dir() and p.name not in ("analysis", "plots") and not p.name.startswith("env_")):

for rdir in sorted(vdir.iterdir()):

m = RUN_RE.match(rdir.name)

if not m or not (rdir / "DONE").exists():

continue

j = json.loads((rdir / "fio.json").read_text())

w = j["jobs"][0]["write"]

meta = (rdir / "meta.txt").read_text()

dev_size = int(re.search(r"size=(\d+) B", meta).group(1))

pct = w["clat_ns"].get("percentile", {})

t, bw = read_log(rdir / "fio_bw.1.log")

bw = bw / 1024.0  # KiB/s -> MiB/s

dur = t[-1] if len(t) else 0.0

row = {

"variant": vdir.name, "map": m["map"], "bs": m["bs"], "rep": int(m["rep"]),

"bw_MiBps": w["bw"] / 1024.0,

"iops": w["iops"],

"written_GiB": w["io_bytes"] / 2**30,

"fill_ratio": w["io_bytes"] / dev_size,

"clat_mean_us": w["clat_ns"]["mean"] / 1e3,

"clat_p50_us": pct.get("50.000000", float("nan")) / 1e3,

"clat_p99_us": pct.get("99.000000", float("nan")) / 1e3,

"clat_p999_us": pct.get("99.900000", float("nan")) / 1e3,

"lat_mean_us": w["lat_ns"]["mean"] / 1e3,

"slat_mean_us": w["slat_ns"]["mean"] / 1e3,

"runtime_s": w["runtime"] / 1e3,

"bw_first10s_MiBps": float(bw[t <= EARLY_S].mean()) if len(t) else float("nan"),

"bw_last20s_MiBps": float(bw[t > dur - LATE_S].mean()) if len(t) else float("nan"),

"dev_bytes": dev_size,

"chmodel_msgs": int(meta_value(meta, "chmodel_msgs").split()[0]) if meta_value(meta, "chmodel_msgs") else 0,

"kernel_warn": kernel_warnings(rdir),

}

bm = re.search(r"^blk_writes:\s*ios=(\d+) merges=(\d+) sectors=(\d+)", meta, re.M)

if bm:   # block-layer write requests during fio (recorded from the sequential-write experiment on)

row["blk_wr_ios"], row["blk_wr_merges"] = int(bm.group(1)), int(bm.group(2))

row["blk_avg_req_KiB"] = int(bm.group(3)) * 512 / 1024 / int(bm.group(1)) if int(bm.group(1)) else float("nan")

g = gc_info(rdir)

if "gc_onset_s" in g:

on = g["gc_onset_s"]

row["gc_onset_s"] = on

row["gc_onset_last_part_s"] = g["gc_onset_last_part_s"]

row["bw_pre_gc_MiBps"] = float(bw[t <= on].mean()) if (t <= on).any() else float("nan")

row["bw_post_gc_MiBps"] = float(bw[t > on].mean()) if (t > on).any() else float("nan")

if "host_pgs" in g and g["host_pgs"]:

pg = kib(m["map"]) * 1024

row["gc_cnt"] = g["gc_cnt"]

row["ftl_host_pgs"] = g["host_pgs"]

row["ftl_gc_pgs"] = g["gc_pgs"]

row["waf_gc"] = (g["host_pgs"] + g["gc_pgs"]) / g["host_pgs"]

row["waf_total"] = (g["host_pgs"] + g["gc_pgs"]) * pg / w["io_bytes"]

for k in ("mg_open", "mg_merge", "mg_full", "mg_evict", "mg_direct", "mg_still_open"):

if k in g:

row[k] = g[k]

rows.append(row)

series[(vdir.name, m["map"], m["bs"], int(m["rep"]))] = (t, bw, row.get("gc_onset_s"))

return rows, series

def collect_failed(exp_dir):

"""Runs with a FAILED marker (fio I/O errors / watchdog): kept out of the aggregates, reported separately."""

out = []

for vdir in sorted(p for p in exp_dir.iterdir() if p.is_dir() and p.name not in ("analysis", "plots") and not p.name.startswith("env_")):

for rdir in sorted(vdir.iterdir()):

m = RUN_RE.match(rdir.name)

if not m or not (rdir / "FAILED").exists() or (rdir / "DONE").exists():

continue

row = {"variant": vdir.name, "map": m["map"], "bs": m["bs"], "rep": int(m["rep"])}

first = (rdir / "FAILED").read_text(errors="replace").splitlines()[0]

for k in ("fio_exit", "fio_json_error"):

mm = re.search(rf"{k}=(\S+)", first)

row[k] = mm.group(1) if mm else ""

try:

w = json.loads((rdir / "fio.json").read_text())["jobs"][0]["write"]

row.update({"runtime_s": w["runtime"] / 1e3, "bw_MiBps": w["bw"] / 1024.0, "iops": w["iops"],

"written_GiB": w["io_bytes"] / 2**30})

except (OSError, ValueError, KeyError, IndexError):

pass

meta = (rdir / "meta.txt").read_text() if (rdir / "meta.txt").exists() else ""

cm = meta_value(meta, "chmodel_msgs")

row["chmodel_msgs"] = int(cm.split()[0]) if cm and cm.split()[0].isdigit() else ""

t0, marks = None, {}

log = (rdir / "kernel.log").read_text(errors="replace") if (rdir / "kernel.log").exists() else ""

for line in log.splitlines():

mt = DMESG_TS.match(line)

if not mt:

continue

ts = float(mt.group(1))

if "fio-start" in line and t0 is None:

t0 = ts

for key, pat in (("t_gc_s", "KSC2026: first GC"), ("t_queue_full_warn_s", "__allocate_work_queue_entry"),

("t_nvme_timeout_s", "timeout, aborting"), ("t_reset_s", "reset controller"),

("t_disable_s", "disable controller"), ("t_first_io_error_s", "I/O error, dev")):

if pat in line and key not in marks:

marks[key] = ts

for key, ts in marks.items():

row[key] = round(ts - t0, 3) if t0 is not None else ""

out.append(row)

return out

def aggregate(rows):

keys = [k for k in rows[0] if k not in ("variant", "map", "bs", "rep", "dev_bytes")]

keys += sorted({k for r in rows for k in r} - set(keys) - {"variant", "map", "bs", "rep", "dev_bytes"})

groups = {}

for r in rows:

groups.setdefault((r["variant"], r["map"], r["bs"]), []).append(r)

agg = []

for (v, mp, bs), rs in sorted(groups.items(), key=lambda x: (x[0][0], kib(x[0][1]), kib(x[0][2]))):

a = {"variant": v, "map": mp, "bs": bs, "n": len(rs)}

for k in keys:

vals = [r[k] for r in rs if k in r and r[k] == r[k]]

if not vals:

continue

a[f"{k}_mean"] = statistics.fmean(vals)

a[f"{k}_std"] = statistics.stdev(vals) if len(vals) > 1 else 0.0

a[f"{k}_min"] = min(vals)

a[f"{k}_max"] = max(vals)

agg.append(a)

return agg

def write_csv(path, rows):

cols = []

for r in rows:

cols += [k for k in r if k not in cols]

with path.open("w", newline="") as f:

wr = csv.DictWriter(f, fieldnames=cols)

wr.writeheader()

for r in rows:

wr.writerow({k: (repr(v) if isinstance(v, float) else v) for k, v in r.items()})   # full precision

def style(ax):

ax.set_facecolor(SURFACE)

for s in ("top", "right"):

ax.spines[s].set_visible(False)

for s in ("left", "bottom"):

ax.spines[s].set_color(AXIS)

ax.tick_params(colors=INK2, labelsize=9)

ax.grid(axis="y", color=GRID, linewidth=0.8)

ax.set_axisbelow(True)

def line_vs_bs(agg, variant, metric, ylabel, title, path, logy=False):

fig, ax = plt.subplots(figsize=(7.2, 4.2), facecolor=SURFACE)

style(ax)

x = np.arange(len(BSS))

for i, mp in enumerate(MAPS):

pts = [(BSS.index(a["bs"]), a[f"{metric}_mean"], a[f"{metric}_std"])

for a in agg if a["variant"] == variant and a["map"] == mp and f"{metric}_mean" in a]

if not pts:

continue

xs, ys, es = zip(*sorted(pts))

ax.errorbar(xs, ys, yerr=es, color=SERIES[i], linewidth=2, marker="o", markersize=5,

capsize=3, label=f"map {mp.upper()}")

ax.set_xticks(x, [s.upper() for s in BSS])

ax.set_xlabel("fio block size", color=INK2)

ax.set_ylabel(ylabel, color=INK2)

if logy:

ax.set_yscale("log")

ax.set_title(title, color=INK, fontsize=11, loc="left")

ax.legend(frameon=False, fontsize=8, ncol=3, labelcolor=INK2)

fig.tight_layout()

fig.savefig(path, dpi=200)

plt.close(fig)

def heatmap(agg, variant, metric, label, title, path):

grid = np.full((len(MAPS), len(BSS)), np.nan)

for a in agg:

if a["variant"] == variant and f"{metric}_mean" in a:

grid[MAPS.index(a["map"]), BSS.index(a["bs"])] = a[f"{metric}_mean"]

if np.isnan(grid).all():

return

fig, ax = plt.subplots(figsize=(6.6, 4.8), facecolor=SURFACE)

cmap = matplotlib.colors.LinearSegmentedColormap.from_list("seqblue", SEQ_BLUE)

im = ax.imshow(grid, cmap=cmap, aspect="auto", origin="lower")

vmax = np.nanmax(grid)

for i in range(len(MAPS)):

for k in range(len(BSS)):

if not np.isnan(grid[i, k]):

ax.text(k, i, f"{grid[i, k]:.0f}", ha="center", va="center", fontsize=8,

color="#ffffff" if grid[i, k] > 0.55 * vmax else INK)

ax.set_xticks(range(len(BSS)), [s.upper() for s in BSS])

ax.set_yticks(range(len(MAPS)), [s.upper() for s in MAPS])

ax.set_xlabel("fio block size", color=INK2)

ax.set_ylabel("FTL mapping unit", color=INK2)

ax.tick_params(colors=INK2, labelsize=9)

cb = fig.colorbar(im, ax=ax)

cb.set_label(label, color=INK2)

ax.set_title(title, color=INK, fontsize=11, loc="left")

fig.tight_layout()

fig.savefig(path, dpi=200)

plt.close(fig)

def timeseries_grid(series, variant, path):

fig, axes = plt.subplots(len(MAPS), len(BSS), figsize=(2.7 * len(BSS) + 2, 2.2 * len(MAPS) + 1.0), sharex=True,

facecolor=SURFACE, squeeze=False)

ymax = {}

for (v, mp, bs, r), (t, bw, on) in series.items():

if v == variant and len(bw):

ymax[mp] = max(ymax.get(mp, 0), float(bw.max()))

for i, mp in enumerate(MAPS):

for k, bs in enumerate(BSS):

ax = axes[i, k]

style(ax)

ax.tick_params(labelsize=7)

for r in (1, 2, 3):

key = (variant, mp, bs, r)

if key not in series:

continue

t, bw, on = series[key]

ax.plot(t, bw, color=SERIES[r - 1], linewidth=1.0, label=f"rep {r}")

if on is not None:

ax.axvline(on, color=SERIES[r - 1], linestyle="--", linewidth=0.8)

if mp in ymax:

ax.set_ylim(0, ymax[mp] * 1.08)

if i == 0:

ax.set_title(f"bs {bs.upper()}", color=INK, fontsize=10)

if k == 0:

ax.set_ylabel(f"map {mp.upper()}\nMiB/s", color=INK2, fontsize=9)

if i == len(MAPS) - 1:

ax.set_xlabel("time (s)", color=INK2, fontsize=9)

handles, labels = axes[0, 0].get_legend_handles_labels()

if handles:

fig.legend(handles, labels, loc="upper right", frameon=False, fontsize=9, ncol=3)

fig.suptitle(f"Write bandwidth over time ({variant}; dashed line = first GC, when recorded)",

color=INK, fontsize=12, x=0.01, ha="left")

fig.tight_layout(rect=(0, 0, 1, 0.97))

fig.savefig(path, dpi=150)

plt.close(fig)

def variant_compare(agg, path):

"""One panel per mapping unit: every variant (or cache mode) against bs; shaded = bs < mapping unit."""

variants = sorted({a["variant"] for a in agg})

if len(variants) < 2:

return

maps = [mp for mp in MAPS if any(a["map"] == mp for a in agg)]

fig, axes = plt.subplots(1, len(maps), figsize=(3.4 * len(maps) + 1, 3.8), facecolor=SURFACE, squeeze=False)

for ax, mp in zip(axes[0], maps):

style(ax)

for vi, v in enumerate(variants):

pts = [(BSS.index(a["bs"]), a["bw_MiBps_mean"], a["bw_MiBps_std"])

for a in agg if a["variant"] == v and a["map"] == mp]

if pts:

xs, ys, es = zip(*sorted(pts))

ax.errorbar(xs, ys, yerr=es, color=SERIES[vi], linewidth=2, marker="o", markersize=5,

capsize=3, label=v)

ax.axvspan(-0.5, sum(1 for b in BSS if kib(b) < kib(mp)) - 0.5, color="#f0efec", zorder=0)

ax.set_xticks(range(len(BSS)), [s.upper() for s in BSS], fontsize=8)

ax.set_title(f"map {mp.upper()}", color=INK, fontsize=10, loc="left")

ax.set_xlabel("fio block size", color=INK2, fontsize=9)

axes[0][0].set_ylabel("write bandwidth (MiB/s)", color=INK2)

axes[0][0].legend(frameon=False, fontsize=8, labelcolor=INK2)

fig.suptitle("Variants compared, mean ± std (shaded: bs < mapping unit)", color=INK, fontsize=11, x=0.01, ha="left")

fig.tight_layout(rect=(0, 0, 1, 0.92))

fig.savefig(path, dpi=200)

plt.close(fig)

def _betacf(a, b, x):

qab, qap, qam = a + b, a + 1.0, a - 1.0

c, d = 1.0, 1.0 - qab * x / qap

d = 1.0 / (d if abs(d) > 1e-300 else 1e-300)

h = d

for m in range(1, 300):

m2 = 2 * m

aa = m * (b - m) * x / ((qam + m2) * (a + m2))

d = 1.0 + aa * d

d = 1.0 / (d if abs(d) > 1e-300 else 1e-300)

c = 1.0 + aa / c if abs(c) > 1e-300 else 1e-300

h *= d * c

aa = -(a + m) * (qab + m) * x / ((a + m2) * (qap + m2))

d = 1.0 + aa * d

d = 1.0 / (d if abs(d) > 1e-300 else 1e-300)

c = 1.0 + aa / c if abs(c) > 1e-300 else 1e-300

de = d * c

h *= de

if abs(de - 1.0) < 1e-12:

break

return h

def t_two_sided_p(t, df):

"""Two-sided p-value of Student's t (regularized incomplete beta), no scipy needed."""

import math

if df <= 0 or t != t:

return float("nan")

x = df / (df + t * t)

a, b = df / 2.0, 0.5

lbeta = math.lgamma(a) + math.lgamma(b) - math.lgamma(a + b)

front = math.exp(a * math.log(x) + b * math.log(1 - x) - lbeta) if 0 < x < 1 else (1.0 if x >= 1 else 0.0)

if x == 0 or x == 1:

return 0.0 if x == 0 else 1.0

if x < (a + 1) / (a + b + 2):

ib = front * _betacf(a, b, x) / a

else:

ib = 1 - front * _betacf(b, a, 1 - x) / b

return ib

def t_quantile_975(df):

lo, hi = 0.0, 50.0

for _ in range(100):

mid = (lo + hi) / 2

if t_two_sided_p(mid, df) > 0.05:

lo = mid

else:

hi = mid

return (lo + hi) / 2

def cache_compare(rows, out):

"""Paired comparison of <v>_drop vs <v>_nodrop (same map, bs, rep). Writes cache_compare.csv / cache_pairs.csv."""

import math

by = {(r["variant"], r["map"], r["bs"], r["rep"]): r for r in rows}

bases = sorted({r["variant"][: -len("_nodrop")] for r in rows if r["variant"].endswith("_nodrop")})

metrics = ["bw_MiBps", "iops", "clat_mean_us", "clat_p99_us", "gc_onset_s", "bw_pre_gc_MiBps", "bw_post_gc_MiBps", "waf_total"]

pair_rows, summ = [], []

for b in bases:

keys = sorted({(r["map"], r["bs"]) for r in rows if r["variant"] == f"{b}_nodrop"}, key=lambda k: (kib(k[0]), kib(k[1])))

allrel = {m: [] for m in metrics}

for mp, bs in keys:

for m in metrics:

diffs, rels, nd, dr = [], [], [], []

for rep in (1, 2, 3, 4, 5):

a, c = by.get((f"{b}_nodrop", mp, bs, rep)), by.get((f"{b}_drop", mp, bs, rep))

if not a or not c or m not in a or m not in c:

continue

nd.append(a[m]); dr.append(c[m]); diffs.append(c[m] - a[m])

rel = (c[m] - a[m]) / a[m] * 100 if a[m] else float("nan")

rels.append(rel); allrel[m].append(rel)

if m == "bw_MiBps":

pair_rows.append({"variant": b, "map": mp, "bs": bs, "rep": rep, "nodrop_bw_MiBps": a[m], "drop_bw_MiBps": c[m], "diff_pct": rel})

if len(diffs) < 2:

continue

md, sd = statistics.fmean(diffs), statistics.stdev(diffs)

t = md / (sd / math.sqrt(len(diffs))) if sd > 0 else float("inf") if md else 0.0

summ.append({"variant": b, "map": mp, "bs": bs, "metric": m, "n_pairs": len(diffs),

"nodrop_mean": statistics.fmean(nd), "drop_mean": statistics.fmean(dr),

"mean_diff": md, "mean_diff_pct": statistics.fmean(rels), "sd_diff": sd,

"paired_t": t, "p_two_sided": t_two_sided_p(t, len(diffs) - 1) if sd > 0 else float("nan")})

for m in metrics:

v = [x for x in allrel[m] if x == x]

if len(v) >= 2:

mean, sd = statistics.fmean(v), statistics.stdev(v)

half = t_quantile_975(len(v) - 1) * sd / math.sqrt(len(v))

t = mean / (sd / math.sqrt(len(v))) if sd > 0 else float("nan")

summ.append({"variant": b, "map": "ALL", "bs": "ALL", "metric": m, "n_pairs": len(v),

"mean_diff_pct": mean, "sd_diff": sd, "ci95_low_pct": mean - half, "ci95_high_pct": mean + half,

"paired_t": t, "p_two_sided": t_two_sided_p(t, len(v) - 1)})

if summ:

out.mkdir(parents=True, exist_ok=True)

write_csv(out / "cache_compare.csv", summ)

write_csv(out / "cache_pairs.csv", pair_rows)

return summ

def main():

if len(sys.argv) != 2:

sys.exit(__doc__)

exp_dir = Path(sys.argv[1]).resolve()

out = exp_dir / "analysis"

out.mkdir(exist_ok=True)

rows, series = collect(exp_dir)

if not rows:

sys.exit(f"no finished runs under {exp_dir}")

rows.sort(key=lambda r: (r["variant"], kib(r["map"]), kib(r["bs"]), r["rep"]))

global SIZES, MAPS, BSS, WORKLOAD

SIZES = sorted({r["map"] for r in rows} | {r["bs"] for r in rows}, key=kib)

MAPS = sorted({r["map"] for r in rows}, key=kib)

BSS = sorted({r["bs"] for r in rows}, key=kib)

if any(re.search(r"^workload:\s*seqwrite", p.read_text(), re.M) for p in exp_dir.glob("*/map*/meta.txt")):

WORKLOAD = "Sequential-write"

agg = aggregate(rows)

write_csv(out / "summary_runs.csv", rows)

write_csv(out / "summary_agg.csv", agg)

failed = collect_failed(exp_dir)

if failed:

write_csv(out / "failed_runs.csv", failed)

for v in sorted({r["variant"] for r in rows}):

line_vs_bs(agg, v, "bw_MiBps", "write bandwidth (MiB/s)",

f"{WORKLOAD} bandwidth, 60 s from a fresh device ({v}; mean ± std, n=3)",

out / f"fig_bw_vs_bs_{v}.png")

line_vs_bs(agg, v, "clat_mean_us", "mean completion latency (µs)",

f"Mean completion latency ({v}; mean ± std, n=3)", out / f"fig_clat_mean_vs_bs_{v}.png", logy=True)

line_vs_bs(agg, v, "clat_p99_us", "p99 completion latency (µs)",

f"p99 completion latency ({v}; mean ± std, n=3)", out / f"fig_clat_p99_vs_bs_{v}.png", logy=True)

line_vs_bs(agg, v, "bw_last20s_MiBps", "write bandwidth, last 20 s (MiB/s)",

f"Bandwidth in the last 20 s of the run ({v})", out / f"fig_bw_last20s_vs_bs_{v}.png")

heatmap(agg, v, "bw_MiBps", "MiB/s", f"Mean write bandwidth, MiB/s ({v})", out / f"fig_bw_heatmap_{v}.png")

heatmap(agg, v, "waf_total", "NAND bytes / host bytes",

f"Total write amplification ({v})", out / f"fig_waf_heatmap_{v}.png")

timeseries_grid(series, v, out / f"fig_timeseries_{v}.png")

variant_compare(agg, out / "fig_variant_compare.png")

cache_compare(rows, out)

print(f"{len(rows)} runs, {len(failed)} failed -> {out}")

if __name__ == "__main__":

main()

## A. exp/plot.py

#!/usr/bin/env python3

"""KSC2026 NVMeVirt mapping-unit experiment — plotting tool (reads the results directly, works on partial data).

usage (from exp/, with ./.venv/bin/python):

python plot.py list                                   # finished runs, available metrics

python plot.py bs      --metric bw_MiBps              # metric vs fio bs, one line per mapping unit (panel per variant)

python plot.py map     --metric iops --logy           # metric vs mapping unit, one line per bs

python plot.py heatmap --metric waf_total --variant wbuffix

python plot.py ts      --maps 128k --bss 4k,128k      # 0.5 s time series (bw|iops|clat|lat), dashed = first GC

python plot.py compare --metric bw_MiBps              # variants compared (e.g. wbuffix vs merge), one panel per mapping unit

python plot.py all                                    # standard set -> results/<EXP>/plots/

common options:

--exp results/main_20261008   experiment directory (default: newest results/main_*)

--variant wbuffix,merge       variants to include

--maps 4k,8k  --bss 4k,128k   restrict mapping units / block sizes

--reps 1,2,3                  repetitions (ts)

--logy                        logarithmic y axis

--out file.png                output file (default: results/<EXP>/plots/<auto>.png)

--show                        also open a window (needs a display)

"""

import argparse

import sys

from pathlib import Path

import matplotlib

HERE = Path(__file__).resolve().parent

sys.path.insert(0, str(HERE))

import analyze as A  # noqa: E402  (collect / aggregate / palette)

SIZES = list(A.SIZES)   # narrowed in main() to the sizes present in the results

MAPS, BSS = list(SIZES), list(SIZES)   # mapping units / fio bs present (they differ in a combined view)

METRICS = {

"bw_MiBps": "write bandwidth, 60 s mean (MiB/s)",

"iops": "IOPS, 60 s mean",

"clat_mean_us": "mean completion latency (us)",

"clat_p50_us": "p50 completion latency (us)",

"clat_p99_us": "p99 completion latency (us)",

"clat_p999_us": "p99.9 completion latency (us)",

"lat_mean_us": "mean total latency (us)",

"bw_first10s_MiBps": "bandwidth, first 10 s (MiB/s)",

"bw_last20s_MiBps": "bandwidth, last 20 s (MiB/s)",

"gc_onset_s": "first GC after fio start (s)",

"bw_pre_gc_MiBps": "bandwidth before first GC (MiB/s)",

"bw_post_gc_MiBps": "bandwidth after first GC (MiB/s)",

"waf_gc": "WAF (GC only, mapping-unit pages)",

"waf_total": "WAF total (NAND bytes / host bytes)",

"gc_cnt": "GC victim lines (all partitions)",

"written_GiB": "host data written in 60 s (GiB)",

"fill_ratio": "host bytes written / device size",

"chmodel_msgs": "NVMeVirt channel-model overflow messages",

}

TS_LOGS = {"bw": ("fio_bw.1.log", 1 / 1024, "write bandwidth (MiB/s)"),

"iops": ("fio_iops.1.log", 1, "IOPS"),

"clat": ("fio_clat.1.log", 1 / 1000, "completion latency (us)"),

"lat": ("fio_lat.1.log", 1 / 1000, "total latency (us)")}

def parse_args():

ap = argparse.ArgumentParser(description=__doc__, formatter_class=argparse.RawDescriptionHelpFormatter)

ap.add_argument("cmd", choices=["list", "bs", "map", "heatmap", "ts", "compare", "all"])

ap.add_argument("--exp")

ap.add_argument("--variant", default="all", help="comma list of result subdirectories (default: all present)")

ap.add_argument("--metric", default="bw_MiBps")

ap.add_argument("--ts-metric", default="bw", choices=list(TS_LOGS))

ap.add_argument("--maps", default="")

ap.add_argument("--bss", default="")

ap.add_argument("--reps", default="1,2,3")

ap.add_argument("--logy", action="store_true")

ap.add_argument("--out")

ap.add_argument("--show", action="store_true")

a = ap.parse_args()

if a.show:   # analyze.py selected Agg at import; switch to an interactive backend when a window is wanted

import matplotlib.pyplot as plt

try:

plt.switch_backend("TkAgg")

except Exception as e:  # noqa: BLE001

print(f"--show unavailable ({e}); saving files only")

a.show = False

return a

def exp_dir(arg):

if arg:

return Path(arg).resolve()

cands = sorted((HERE / "results").glob("main_*"))

if not cands:

sys.exit("no results/main_* directory; pass --exp")

return cands[-1]

def split(s):

return [x.strip() for x in s.split(",") if x.strip()]

def finish(fig, out, show):

import matplotlib.pyplot as plt

out.parent.mkdir(parents=True, exist_ok=True)

fig.savefig(out, dpi=170, facecolor=A.SURFACE)

print(f"wrote {out}")

if show:

plt.show()

plt.close(fig)

def agg_lookup(agg):

return {(a["variant"], a["map"], a["bs"]): a for a in agg}

def plot_lines(agg, variants, metric, x_axis, maps, bss, logy, out, show):

import matplotlib.pyplot as plt

look = agg_lookup(agg)

fig, axes = plt.subplots(1, len(variants), figsize=(6.4 * len(variants), 4.4), facecolor=A.SURFACE, squeeze=False,

sharey=True)

xs_lab = bss if x_axis == "bs" else maps

series = maps if x_axis == "bs" else bss

for ax, v in zip(axes[0], variants):

A.style(ax)

for i, s in enumerate(series):

pts = []

for k, x in enumerate(xs_lab):

key = (v, s, x) if x_axis == "bs" else (v, x, s)

a = look.get(key)

if a and f"{metric}_mean" in a:

pts.append((k, a[f"{metric}_mean"], a.get(f"{metric}_std", 0.0)))

if pts:

xk, ys, es = zip(*pts)

lab = f"map {s.upper()}" if x_axis == "bs" else f"bs {s.upper()}"

ax.errorbar(xk, ys, yerr=es, color=A.SERIES[SIZES.index(s) % len(A.SERIES)], linewidth=2, marker="o",

markersize=5, capsize=3, label=lab)

ax.set_xticks(range(len(xs_lab)), [x.upper() for x in xs_lab])

ax.set_xlabel("fio block size" if x_axis == "bs" else "FTL mapping unit", color=A.INK2)

ax.set_title(v, color=A.INK, fontsize=11, loc="left")

if logy:

ax.set_yscale("log")

axes[0][0].set_ylabel(METRICS.get(metric, metric), color=A.INK2)

axes[0][-1].legend(frameon=False, fontsize=8, labelcolor=A.INK2, ncol=2)

fig.suptitle(f"{METRICS.get(metric, metric)} — mean ± std over repetitions", color=A.INK, x=0.01, ha="left", fontsize=11)

fig.tight_layout(rect=(0, 0, 1, 0.94))

finish(fig, out, show)

def plot_heatmaps(agg, variants, metric, out, show):

import matplotlib.pyplot as plt

import numpy as np

look = agg_lookup(agg)

fig, axes = plt.subplots(1, len(variants), figsize=(6.2 * len(variants), 4.8), facecolor=A.SURFACE, squeeze=False)

cmap = matplotlib.colors.LinearSegmentedColormap.from_list("seqblue", A.SEQ_BLUE)

grids = []

for v in variants:

g = np.full((len(MAPS), len(BSS)), np.nan)

for i, mp in enumerate(MAPS):

for k, bs in enumerate(BSS):

a = look.get((v, mp, bs))

if a and f"{metric}_mean" in a:

g[i, k] = a[f"{metric}_mean"]

grids.append(g)

vmax = np.nanmax([np.nanmax(g) for g in grids if not np.isnan(g).all()] or [1])

for ax, v, g in zip(axes[0], variants, grids):

im = ax.imshow(g, cmap=cmap, aspect="auto", origin="lower", vmin=0, vmax=vmax)

for i in range(len(MAPS)):

for k in range(len(BSS)):

if not np.isnan(g[i, k]):

val = g[i, k]

txt = f"{val:.0f}" if val >= 100 else f"{val:.2f}" if val < 10 else f"{val:.1f}"

ax.text(k, i, txt, ha="center", va="center", fontsize=8, color="#ffffff" if val > 0.55 * vmax else A.INK)

ax.set_xticks(range(len(BSS)), [s.upper() for s in BSS])

ax.set_yticks(range(len(MAPS)), [s.upper() for s in MAPS])

ax.set_xlabel("fio block size", color=A.INK2)

ax.set_ylabel("FTL mapping unit", color=A.INK2)

ax.set_title(v, color=A.INK, fontsize=11, loc="left")

ax.tick_params(colors=A.INK2, labelsize=9)

cb = fig.colorbar(im, ax=list(axes[0]))

cb.set_label(METRICS.get(metric, metric), color=A.INK2)

fig.suptitle(METRICS.get(metric, metric), color=A.INK, x=0.01, ha="left", fontsize=11)

finish(fig, out, show)

def read_ts(rdir, kind):

fname, scale, _ = TS_LOGS[kind]

t, v = A.read_log(rdir / fname)

return t, v * scale

def plot_ts(exp, variants, maps, bss, reps, kind, logy, out, show):

import matplotlib.pyplot as plt

fig, axes = plt.subplots(len(maps), len(bss), figsize=(4.2 * len(bss) + 1, 2.8 * len(maps) + 1), squeeze=False,

facecolor=A.SURFACE, sharex=True)

color_i = 0

handles = {}

for i, mp in enumerate(maps):

for k, bs in enumerate(bss):

ax = axes[i][k]

A.style(ax)

ax.tick_params(labelsize=7)

for vi, v in enumerate(variants):

for r in reps:

rdir = exp / v / f"map{mp}_bs{bs}_r{r}"

if not (rdir / "DONE").exists():

continue

t, y = read_ts(rdir, kind)

idx = (vi * len(reps) + reps.index(r)) % len(A.SERIES)

h, = ax.plot(t, y, color=A.SERIES[idx], linewidth=1.0)

handles[f"{v} rep{r}"] = h

on = A.gc_info(rdir).get("gc_onset_s")

if on is not None:

ax.axvline(on, color=A.SERIES[idx], linestyle="--", linewidth=0.8)

if logy:

ax.set_yscale("log")

ax.set_title(f"map {mp.upper()} / bs {bs.upper()}", color=A.INK, fontsize=9, loc="left")

if k == 0:

ax.set_ylabel(TS_LOGS[kind][2], color=A.INK2, fontsize=8)

if i == len(maps) - 1:

ax.set_xlabel("time since fio start (s)", color=A.INK2, fontsize=8)

color_i += 1

if handles:

fig.legend(list(handles.values()), list(handles.keys()), loc="upper right", frameon=False, fontsize=8,

ncol=min(6, len(handles)))

fig.suptitle(f"{TS_LOGS[kind][2]} over time (0.5 s average); dashed = first GC", color=A.INK, x=0.01, ha="left",

fontsize=11)

fig.tight_layout(rect=(0, 0, 1, 0.94))

finish(fig, out, show)

def plot_compare(agg, metric, maps, logy, out, show):

import matplotlib.pyplot as plt

look = agg_lookup(agg)

variants = sorted({a["variant"] for a in agg})

fig, axes = plt.subplots(1, len(maps), figsize=(3.3 * len(maps) + 1, 3.8), facecolor=A.SURFACE, squeeze=False)

for ax, mp in zip(axes[0], maps):

A.style(ax)

for vi, v in enumerate(variants):

pts = [(k, look[(v, mp, bs)][f"{metric}_mean"], look[(v, mp, bs)].get(f"{metric}_std", 0.0))

for k, bs in enumerate(BSS) if (v, mp, bs) in look and f"{metric}_mean" in look[(v, mp, bs)]]

if pts:

xk, ys, es = zip(*pts)

ax.errorbar(xk, ys, yerr=es, color=A.SERIES[vi], linewidth=2, marker="o", markersize=4, capsize=3, label=v)

ax.axvspan(-0.5, sum(1 for b in BSS if A.kib(b) < A.kib(mp)) - 0.5, color="#f0efec", zorder=0)   # bs < mapping unit

ax.set_xticks(range(len(BSS)), [s.upper() for s in BSS], fontsize=7)

ax.set_title(f"map {mp.upper()}", color=A.INK, fontsize=10, loc="left")

if logy:

ax.set_yscale("log")

axes[0][0].set_ylabel(METRICS.get(metric, metric), color=A.INK2)

axes[0][0].legend(frameon=False, fontsize=8, labelcolor=A.INK2)

fig.suptitle(f"{METRICS.get(metric, metric)}: variants compared (shaded = bs < mapping unit)", color=A.INK, x=0.01,

ha="left", fontsize=11)

fig.tight_layout(rect=(0, 0, 1, 0.9))

finish(fig, out, show)

def main():

a = parse_args()

exp = exp_dir(a.exp)

rows, _ = A.collect(exp)

if not rows:

sys.exit(f"no finished runs under {exp}")

agg = A.aggregate(rows)

global SIZES, MAPS, BSS

SIZES[:] = sorted({r["map"] for r in rows} | {r["bs"] for r in rows}, key=A.kib)

MAPS[:] = sorted({r["map"] for r in rows}, key=A.kib)

BSS[:] = sorted({r["bs"] for r in rows}, key=A.kib)

A.SIZES, A.MAPS, A.BSS = SIZES, MAPS, BSS

present = sorted({r["variant"] for r in rows})

variants = [v for v in split(a.variant) if v in present] if a.variant != "all" else present

if not variants:

variants = present

maps, bss, reps = split(a.maps) or list(MAPS), split(a.bss) or list(BSS), [int(x) for x in split(a.reps)]

pdir = exp / "plots"

out = Path(a.out) if a.out else None

if a.cmd == "list":

print(f"experiment: {exp}")

for v in sorted({r["variant"] for r in rows}):

done = [r for r in rows if r["variant"] == v]

print(f"  {v}: {len(done)} finished runs")

print("metrics:", ", ".join(METRICS))

print("time-series kinds (ts --ts-metric):", ", ".join(TS_LOGS))

return

if a.cmd == "bs":

plot_lines(agg, variants, a.metric, "bs", maps, bss, a.logy, out or pdir / f"bs_{a.metric}.png", a.show)

elif a.cmd == "map":

plot_lines(agg, variants, a.metric, "map", maps, bss, a.logy, out or pdir / f"map_{a.metric}.png", a.show)

elif a.cmd == "heatmap":

plot_heatmaps(agg, variants, a.metric, out or pdir / f"heatmap_{a.metric}.png", a.show)

elif a.cmd == "ts":

plot_ts(exp, variants, maps, bss, reps, a.ts_metric, a.logy,

out or pdir / f"ts_{a.ts_metric}_{'-'.join(maps)}_{'-'.join(bss)}.png", a.show)

elif a.cmd == "compare":

plot_compare(agg, a.metric, maps, a.logy, out or pdir / f"compare_{a.metric}.png", a.show)

elif a.cmd == "all":

for m, logy in (("bw_MiBps", False), ("iops", True), ("clat_mean_us", True), ("clat_p99_us", True),

("bw_pre_gc_MiBps", False), ("bw_post_gc_MiBps", False), ("gc_onset_s", False), ("waf_total", True)):

plot_lines(agg, variants, m, "bs", MAPS, BSS, logy, pdir / f"bs_{m}.png", False)

plot_lines(agg, variants, m, "map", MAPS, BSS, logy, pdir / f"map_{m}.png", False)

for m in ("bw_MiBps", "waf_total", "gc_onset_s", "bw_post_gc_MiBps"):

plot_heatmaps(agg, variants, m, pdir / f"heatmap_{m}.png", False)

for kind in ("bw", "clat"):

plot_ts(exp, variants, MAPS, BSS, [1], kind, kind == "clat", pdir / f"ts_{kind}_all_rep1.png", False)

for m in ("bw_MiBps", "clat_mean_us", "waf_total"):

plot_compare(agg, m, MAPS, m != "bw_MiBps", pdir / f"compare_{m}.png", False)

if __name__ == "__main__":

main()

## A. exp/make_gallery.py

#!/usr/bin/env python3

"""Make a self-contained HTML gallery (images embedded) of every graph of one experiment.

usage: python3 make_gallery.py results/<EXP> [out.html]

Reads results/<EXP>/analysis/fig_*.png (analyze.py) and results/<EXP>/plots/*.png (plot.py all).

"""

import base64

import html

import sys

from pathlib import Path

EXP = Path(sys.argv[1]).resolve()

OUT = Path(sys.argv[2]) if len(sys.argv) > 2 else EXP / f"gallery_{EXP.name}.html"

# (group title, [(file pattern relative to EXP, caption)])

GROUPS = [

("1. 대역폭 — 핵심", [

("plots/bs_bw_MiBps.png", "60 s 평균 쓰기 대역폭. x = fio bs, 선 = 매핑 단위, 패널 = 변형(캐시 조건), 오차막대 = 표준편차"),

("plots/heatmap_bw_MiBps.png", "매핑 단위 × bs 평균 대역폭 열지도(변형별 패널)"),

("plots/map_bw_MiBps.png", "같은 값을 매핑 단위 축으로: 선 = bs"),

("plots/compare_bw_MiBps.png", "변형 비교(주 데이터셋: drop vs nodrop, 순차: wbuffix vs merge; 변형이 하나뿐인 데이터셋은 그 변형만). 회색 = bs < 매핑 단위"),

("plots/bs_iops.png", "IOPS (로그 축)"),

("plots/map_iops.png", "IOPS, 매핑 단위 축 (로그 축)"),

]),

("2. GC 전후 나눠 보기", [

("plots/bs_gc_onset_s.png", "fio 시작부터 첫 GC 까지의 시간"),

("plots/map_gc_onset_s.png", "첫 GC 시각, 매핑 단위 축"),

("plots/heatmap_gc_onset_s.png", "첫 GC 시각 열지도"),

("plots/bs_bw_pre_gc_MiBps.png", "GC 이전 구간 평균 대역폭 — NAND 프로그램 한계(× bs/매핑)를 보여 준다"),

("plots/map_bw_pre_gc_MiBps.png", "GC 이전 대역폭, 매핑 단위 축"),

("plots/bs_bw_post_gc_MiBps.png", "GC 이후 구간 평균 대역폭"),

("plots/map_bw_post_gc_MiBps.png", "GC 이후 대역폭, 매핑 단위 축"),

("plots/heatmap_bw_post_gc_MiBps.png", "GC 이후 대역폭 열지도"),

("analysis/fig_bw_last20s_vs_bs_*.png", "마지막 20 s(40–60 s) 평균 대역폭 — GC 이후 정상 상태에 가까운 구간"),

]),

("3. 쓰기 증폭 (WAF)", [

("plots/bs_waf_total.png", "전체 쓰기 증폭 WAF_total = NAND 바이트 ÷ 호스트 바이트 (로그 축)"),

("plots/map_waf_total.png", "WAF_total, 매핑 단위 축"),

("plots/heatmap_waf_total.png", "WAF_total 열지도"),

("plots/compare_waf_total.png", "WAF_total 변형 비교"),

("analysis/fig_waf_heatmap_*.png", "WAF_total 열지도 (변형별 파일)"),

]),

("4. 지연", [

("plots/bs_clat_mean_us.png", "평균 완료 지연 (로그 축)"),

("plots/map_clat_mean_us.png", "평균 완료 지연, 매핑 단위 축"),

("plots/compare_clat_mean_us.png", "평균 완료 지연 변형 비교"),

("plots/bs_clat_p99_us.png", "p99 완료 지연 (로그 축)"),

("plots/map_clat_p99_us.png", "p99 완료 지연, 매핑 단위 축"),

]),

("5. 시간에 따른 변화 (0.5 s 평균)", [

("analysis/fig_timeseries_*.png", "조합별 대역폭 시계열 — 3 회차를 겹침, 점선 = 첫 GC"),

("plots/ts_bw_all_rep1.png", "1 회차 대역폭 시계열 — 변형을 한 그림에"),

("plots/ts_clat_all_rep1.png", "1 회차 완료 지연 시계열 (로그 축)"),

]),

("6. analyze.py 기본 그림 (변형별 파일)", [

("analysis/fig_bw_vs_bs_*.png", "대역폭 vs bs"),

("analysis/fig_bw_heatmap_*.png", "대역폭 열지도"),

("analysis/fig_clat_mean_vs_bs_*.png", "평균 완료 지연 vs bs"),

("analysis/fig_clat_p99_vs_bs_*.png", "p99 완료 지연 vs bs"),

("analysis/fig_variant_compare.png", "변형 비교 (매핑 단위별 패널)"),

]),

]

TITLE = {"main3x3_20261008": "주 데이터셋 — 매핑 4K·16K·32K × bs 4K·16K·32K × 3 회 × 페이지 캐시 drop/no-drop (wbuffix)",

"main_20261008": "보조 데이터셋 — 첫 설계 6×6 중 측정한 부분 (wbuffix 53 회)",

"seq3x3_20261008": "순차 쓰기 추가 실험 — 매핑 4K·16K·32K × bs 4K·16K·32K × 3 회 × 모델 wbuffix / merge (쓰기 버퍼 병합)",

"randbs_20261008": "랜덤 쓰기 bs 8K·64K 추가 실험 — 매핑 4K·16K·32K × bs 8K·64K × 3 회 (wbuffix, 페이지 캐시 그대로)",

"rand3x5_20261008": "랜덤 쓰기 매핑 3 × bs 5 — 주 데이터셋 nodrop(bs 4K·16K·32K) + randbs_20261008(bs 8K·64K) 를 합친 보기 (wbuffix)"}.get(EXP.name, EXP.name)

def img(path):

return "data:image/png;base64," + base64.b64encode(path.read_bytes()).decode()

used, sections, toc = set(), [], []

for gi, (gtitle, items) in enumerate(GROUPS):

cards = []

for pat, cap in items:

for f in sorted(EXP.glob(pat)):

if f in used:

continue

used.add(f)

name = f.relative_to(EXP)

cards.append(f'<figure><img loading="lazy" src="{img(f)}" alt="{html.escape(cap)}"><figcaption><b>{html.escape(cap)}</b>'

f'<span>{html.escape(str(name))}</span></figcaption></figure>')

if cards:

sections.append(f'<section id="g{gi}"><h2>{html.escape(gtitle)}</h2>{"".join(cards)}</section>')

toc.append(f'<a href="#g{gi}">{html.escape(gtitle)}</a>')

rest = [f for f in sorted(EXP.glob("analysis/*.png")) + sorted(EXP.glob("plots/*.png")) if f not in used]

if rest:

sections.append('<section id="rest"><h2>기타</h2>' + "".join(

f'<figure><img loading="lazy" src="{img(f)}"><figcaption><span>{html.escape(str(f.relative_to(EXP)))}</span></figcaption></figure>' for f in rest) + "</section>")

toc.append('<a href="#rest">기타</a>')

page = f"""<!doctype html><html lang="ko"><head><meta charset="utf-8"><meta name="viewport" content="width=device-width,initial-scale=1">

<title>KSC2026 그래프 — {html.escape(EXP.name)}</title>

<style>

:root {{ --bg:#f9f9f7; --card:#fcfcfb; --ink:#0b0b0b; --ink2:#52514e; --line:#e1e0d9; }}

body {{ margin:0; background:var(--bg); color:var(--ink); font:14px/1.5 system-ui,-apple-system,"Segoe UI","Malgun Gothic",sans-serif; padding:0 16px 48px; }}

header {{ max-width:1200px; margin:0 auto; padding:24px 0 8px; }}

h1 {{ font-size:20px; margin:0 0 4px; }} h2 {{ font-size:17px; margin:32px 0 12px; border-bottom:1px solid var(--line); padding-bottom:6px; }}

p.sub {{ color:var(--ink2); margin:0 0 12px; }}

nav {{ display:flex; flex-wrap:wrap; gap:8px; }} nav a {{ color:var(--ink2); text-decoration:none; border:1px solid var(--line); border-radius:999px; padding:2px 10px; background:var(--card); }}

main {{ max-width:1200px; margin:0 auto; }}

section {{ display:grid; grid-template-columns:repeat(auto-fill,minmax(520px,1fr)); gap:16px; }}

section h2 {{ grid-column:1/-1; }}

figure {{ margin:0; background:var(--card); border:1px solid var(--line); border-radius:8px; padding:10px; }}

figure img {{ width:100%; height:auto; display:block; cursor:zoom-in; }}

figcaption {{ margin-top:6px; color:var(--ink2); font-size:13px; }} figcaption span {{ display:block; font-family:ui-monospace,Consolas,monospace; font-size:11px; color:#898781; }}

@media (max-width:600px) {{ section {{ grid-template-columns:1fr; }} }}

</style></head><body>

<header><h1>KSC2026 NVMeVirt 매핑 단위 실험 — 그래프 모음</h1>

<p class="sub">{html.escape(TITLE)} · 원본 폴더 exp/results/{html.escape(EXP.name)}/ · 그림 {len(used) + len(rest)} 장 · 이미지를 누르면 새 탭에서 크게 볼 수 있다</p>

<nav>{"".join(toc)}</nav></header>

<main>{"".join(sections)}</main>

<script>document.querySelectorAll("figure img").forEach(i=>i.addEventListener("click",()=>{{const w=window.open();if(w){{w.document.write('<img src="'+i.src+'" style="max-width:100%">');}}}}));</script>

</body></html>"""

OUT.write_text(page)

print(f"wrote {OUT} ({OUT.stat().st_size / 1e6:.1f} MB, {len(used) + len(rest)} images)")

## A. exp/requirements.txt

contourpy==1.4.0

cycler==0.12.1

fonttools==4.66.1

kiwisolver==1.5.1

lxml==6.1.3

matplotlib==3.11.2

numpy==2.5.3

packaging==26.3

pillow==12.3.0

pyparsing==3.3.3

python-dateutil==2.9.0.post0

python-docx==1.2.0

six==1.17.0

typing_extensions==4.16.0

## A. exp/.gitignore

# kernel modules are rebuilt from nvmevirt/ (checksums are kept in modules/<variant>/SHA256SUMS)

modules/*/*.ko

# python virtualenv for analysis

.venv/

__pycache__/

# one-page HTML galleries (make_gallery.py): large, the PNGs they contain are committed

results/*/gallery_*.html

부록의 스크립트는 문서를 만든 시점의 작업 트리 내용이다. 랜덤 쓰기 실험 때의 run_experiment.sh·common.sh·analyze.py 는 커밋 e598e75 의 것이며, 그 뒤 순차 쓰기용으로 WORKLOAD·NOMERGES·블록 계층 기록·merge 변형을 더했다(기본값은 이전 동작과 같다; git diff e598e75 66446ea -- exp). 이어서 랜덤 bs 8K·64K 실험용으로 run_all_rand_bs.sh·link_runs.py 를 더하고, analyze.py·plot.py 가 매핑 단위(행)와 bs(열)를 따로 다루게 했으며, collect_env.sh 가 같은 폴더의 옛 스냅샷을 지우고 새로 쓰게 했다(git diff 66446ea a86fc87 -- exp). 랜덤 bs 8K·64K 회차는 a86fc87 의 스크립트로 실행했다(run_experiment.sh·common.sh 는 66446ea 와 같다). 그 뒤의 커밋에서는 주석, run_all*.sh 의 모듈 확인 방식(.ko 파일 존재 검사), link_runs.py 의 오류 처리, make_gallery.py 를 고쳤다. 마지막으로 원본 모델(base) 결과를 빼면서(10 절) run_experiment.sh·build_modules.sh 의 기본 변형을 base 에서 wbuffix 로 바꾸고 run_all_6x6.sh 에서 base 빌드·실행 단계를 뺐다. run_all*.sh 와 4.3 절의 명령은 모두 변형을 인자로 주므로 측정 절차는 같다.

이 문서와 인계 기록을 만드는 exp/report/ 의 make_report.py · docx_helpers.py · make_md_results.py · findings_ko.txt · findings_seq_ko.txt · findings_randbs_ko.txt · make_handoff.sh 는 태그 ksc2026-v3 의 커밋에 있다(분량상 생략).

# 부록 B. NVMeVirt 변경 diff 전문 (원본 61c90f7 대비)

원본 61c90f7 의 파일을 그대로 nvmevirt/ 로 옮긴 커밋(d508610, 내용 변경 없음) 이후의 변경이다: git diff d508610 e598e75 -- nvmevirt

diff --git a/nvmevirt/Kbuild b/nvmevirt/Kbuild

index 11bb91b..ccba48b 100644

--- a/nvmevirt/Kbuild

+++ b/nvmevirt/Kbuild

@@ -1,6 +1,6 @@

# Select one of the targets to build

-CONFIG_NVMEVIRT_NVM := y

-#CONFIG_NVMEVIRT_SSD := y

+#CONFIG_NVMEVIRT_NVM := y

+CONFIG_NVMEVIRT_SSD := y

#CONFIG_NVMEVIRT_ZNS := y

#CONFIG_NVMEVIRT_KV := y

@@ -12,6 +12,15 @@ ccflags-$(CONFIG_NVMEVIRT_NVM) += -DBASE_SSD=INTEL_OPTANE

nvmev-$(CONFIG_NVMEVIRT_NVM) += simple_ftl.o

ccflags-$(CONFIG_NVMEVIRT_SSD) += -DBASE_SSD=SAMSUNG_970PRO

+# FTL mapping unit in bytes (KSC2026): make MAPPING_UNIT=16384  (default 4096 = upstream behaviour)

+MAPPING_UNIT ?= 4096

+ccflags-$(CONFIG_NVMEVIRT_SSD) += -DMAPPING_UNIT=$(MAPPING_UNIT)

+# KSC2026 optional switches (default 0 = upstream behaviour):

+#   WBUF_FIX=1  a write occupies whole mapping units in the write buffer (what is released after programming)

+#   GC_STATS=1  log the first GC and per-partition host/GC page counters (printed at rmmod)

+WBUF_FIX ?= 0

+GC_STATS ?= 0

+ccflags-$(CONFIG_NVMEVIRT_SSD) += -DKSC_WBUF_FIX=$(WBUF_FIX) -DKSC_GC_STATS=$(GC_STATS)

nvmev-$(CONFIG_NVMEVIRT_SSD) += ssd.o conv_ftl.o pqueue/pqueue.o channel_model.o

ccflags-$(CONFIG_NVMEVIRT_ZNS) += -DBASE_SSD=WD_ZN540

diff --git a/nvmevirt/conv_ftl.c b/nvmevirt/conv_ftl.c

index a0bea2b..24d4e77 100644

--- a/nvmevirt/conv_ftl.c

+++ b/nvmevirt/conv_ftl.c

@@ -392,6 +392,13 @@ void conv_init_namespace(struct nvmev_ns *ns, uint32_t id, uint64_t size, void *

ssd = kmalloc(sizeof(struct ssd), GFP_KERNEL);

ssd_init(ssd, &spp, cpu_nr_dispatcher);

conv_init_ftl(&conv_ftls[i], &cpp, ssd);

+#if KSC_GC_STATS

+        conv_ftls[i].ksc_part = i;

+        conv_ftls[i].ksc_gc_seen = false;

+        conv_ftls[i].ksc_host_pgs = 0;

+        conv_ftls[i].ksc_gc_pgs = 0;

+        conv_ftls[i].ksc_gc_cnt = 0;

+#endif

}

/* PCIe, Write buffer are shared by all instances*/

@@ -415,6 +422,7 @@ void conv_init_namespace(struct nvmev_ns *ns, uint32_t id, uint64_t size, void *

NVMEV_INFO("FTL physical space: %lld, logical space: %lld (physical/logical * 100 = %d)\n",

size, ns->size, cpp.pba_pcent);

+    NVMEV_INFO("KSC2026: WBUF_FIX=%d GC_STATS=%d\n", KSC_WBUF_FIX, KSC_GC_STATS);

return;

}

@@ -435,6 +443,14 @@ void conv_remove_namespace(struct nvmev_ns *ns)

conv_ftls[i].ssd->write_buffer = NULL;

}

+#if KSC_GC_STATS

+    for (i = 0; i < nr_parts; i++) {

+        NVMEV_INFO("KSC2026: stats part=%u host_pgs=%llu gc_pgs=%llu gc_cnt=%llu free_lines=%u\n", i,

+               conv_ftls[i].ksc_host_pgs, conv_ftls[i].ksc_gc_pgs, conv_ftls[i].ksc_gc_cnt,

+               conv_ftls[i].lm.free_line_cnt);

+    }

+#endif

+

for (i = 0; i < nr_parts; i++) {

conv_remove_ftl(&conv_ftls[i]);

ssd_remove(conv_ftls[i].ssd);

@@ -603,6 +619,9 @@ static uint64_t gc_write_page(struct conv_ftl *conv_ftl, struct ppa *old_ppa)

uint64_t lpn = get_rmap_ent(conv_ftl, old_ppa);

NVMEV_ASSERT(valid_lpn(conv_ftl, lpn));

+#if KSC_GC_STATS

+    conv_ftl->ksc_gc_pgs++;

+#endif

new_ppa = get_new_page(conv_ftl, GC_IO);

/* update maptbl */

set_maptbl_ent(conv_ftl, lpn, &new_ppa);

@@ -768,6 +787,15 @@ static int do_gc(struct conv_ftl *conv_ftl, bool force)

conv_ftl->lm.full_line_cnt, conv_ftl->lm.free_line_cnt);

conv_ftl->wfc.credits_to_refill = victim_line->ipc;

+#if KSC_GC_STATS

+    conv_ftl->ksc_gc_cnt++;

+    if (!conv_ftl->ksc_gc_seen) {

+        conv_ftl->ksc_gc_seen = true;

+        NVMEV_INFO("KSC2026: first GC part=%u victim line=%d vpc=%d ipc=%d free_lines=%u host_pgs=%llu\n",

+               conv_ftl->ksc_part, victim_line->id, victim_line->vpc, victim_line->ipc,

+               conv_ftl->lm.free_line_cnt, conv_ftl->ksc_host_pgs);

+    }

+#endif

/* copy back valid data */

for (flashpg = 0; flashpg < spp->flashpgs_per_blk; flashpg++) {

@@ -943,6 +971,7 @@ static bool conv_write(struct nvmev_ns *ns, struct nvmev_request *req, struct nv

uint64_t nsecs_latest;

uint64_t nsecs_xfer_completed;

uint32_t allocated_buf_size;

+    uint64_t wbuf_bytes;

struct nand_cmd swr = {

.type = USER_IO,

@@ -958,8 +987,15 @@ static bool conv_write(struct nvmev_ns *ns, struct nvmev_request *req, struct nv

return false;

}

-    allocated_buf_size = buffer_allocate(wbuf, LBA_TO_BYTE(nr_lba));

-    if (allocated_buf_size < LBA_TO_BYTE(nr_lba))

+#if KSC_WBUF_FIX

+    /* KSC2026: a write occupies whole mapping units in the write buffer; this is also what

+     * schedule_internal_operation() releases once the flash page is programmed */

+    wbuf_bytes = (end_lpn - start_lpn + 1) * spp->pgsz;

+#else

+    wbuf_bytes = LBA_TO_BYTE(nr_lba);

+#endif

+    allocated_buf_size = buffer_allocate(wbuf, wbuf_bytes);

+    if (allocated_buf_size < wbuf_bytes)

return false;

nsecs_latest =

@@ -993,6 +1029,9 @@ static bool conv_write(struct nvmev_ns *ns, struct nvmev_request *req, struct nv

set_rmap_ent(conv_ftl, local_lpn, &ppa);

mark_page_valid(conv_ftl, &ppa);

+#if KSC_GC_STATS

+        conv_ftl->ksc_host_pgs++;

+#endif

/* need to advance the write pointer here */

advance_write_pointer(conv_ftl, USER_IO);

diff --git a/nvmevirt/conv_ftl.h b/nvmevirt/conv_ftl.h

index 6d025f3..486eced 100644

--- a/nvmevirt/conv_ftl.h

+++ b/nvmevirt/conv_ftl.h

@@ -65,6 +65,14 @@ struct conv_ftl {

struct write_pointer gc_wp;

struct line_mgmt lm;

struct write_flow_control wfc;

+#if KSC_GC_STATS

+    /* KSC2026 statistics (only touched by the dispatcher thread) */

+    uint32_t ksc_part;

+    bool ksc_gc_seen;

+    uint64_t ksc_host_pgs; /* mapping-unit pages written by host commands */

+    uint64_t ksc_gc_pgs; /* mapping-unit pages copied by GC */

+    uint64_t ksc_gc_cnt; /* victim lines cleaned */

+#endif

};

void conv_init_namespace(struct nvmev_ns *ns, uint32_t id, uint64_t size, void *mapped_addr,

diff --git a/nvmevirt/ssd.c b/nvmevirt/ssd.c

index bc91f58..6d79057 100644

--- a/nvmevirt/ssd.c

+++ b/nvmevirt/ssd.c

@@ -70,7 +70,7 @@ void ssd_init_params(struct ssdparams *spp, uint64_t capacity, uint32_t nparts)

uint64_t blk_size, total_size;

spp->secsz = LBA_SIZE;

-    spp->secs_per_pg = 4096 / LBA_SIZE; // pg == 4KB

+    spp->secs_per_pg = MAPPING_UNIT / LBA_SIZE; // pg == mapping unit (KSC2026, upstream: 4KB)

spp->pgsz = spp->secsz * spp->secs_per_pg;

spp->nchs = NAND_CHANNELS;

@@ -169,6 +169,9 @@ void ssd_init_params(struct ssdparams *spp, uint64_t capacity, uint32_t nparts)

spp->tt_lines, BYTE_TO_MB(spp->pgs_per_blk * spp->pgsz),

BYTE_TO_KB(spp->pgs_per_blk * spp->pgsz), BYTE_TO_MB(spp->pgs_per_line * spp->pgsz),

BYTE_TO_KB(spp->pgs_per_line * spp->pgsz));

+    NVMEV_INFO("KSC2026: mapping unit=%u B, flash page=%u B, oneshot page=%u B, pgs_per_blk=%u, blks_per_pl=%u, write buffer=%u B\n",

+           spp->pgsz, FLASH_PAGE_SIZE, ONESHOT_PAGE_SIZE, spp->pgs_per_blk, spp->blks_per_pl,

+           GLOBAL_WB_SIZE);

}

static void ssd_init_nand_page(struct nand_page *pg, struct ssdparams *spp)

diff --git a/nvmevirt/ssd_config.h b/nvmevirt/ssd_config.h

index 2c661e8..b069e82 100644

--- a/nvmevirt/ssd_config.h

+++ b/nvmevirt/ssd_config.h

@@ -74,9 +74,20 @@ enum {

#define NAND_CHANNELS (8)

#define LUNS_PER_NAND_CH (2)

#define PLNS_PER_LUN (1)

+/* KSC2026: FTL mapping unit (bytes), set at build time with make MAPPING_UNIT=<bytes> */

+#ifndef MAPPING_UNIT

+#define MAPPING_UNIT (4096)

+#endif

+/* KSC2026: flash page stays 32 KiB (default); for mapping units larger than 32 KiB

+ * the flash page is set equal to the mapping unit so that FLASH_PAGE_SIZE % pgsz == 0 */

+#if (MAPPING_UNIT > 32768)

+#define FLASH_PAGE_SIZE (MAPPING_UNIT)

+#else

#define FLASH_PAGE_SIZE KB(32)

+#endif

#define ONESHOT_PAGE_SIZE (FLASH_PAGE_SIZE * 1)

-#define BLKS_PER_PLN (8192)

+/* KSC2026: 12 GiB storage / 4 partitions / (2 ch x 2 LUN x 1 plane) / 384 = 2 MiB block (default 8192) */

+#define BLKS_PER_PLN (384)

#define BLK_SIZE (0) /*BLKS_PER_PLN should not be 0 */

static_assert((ONESHOT_PAGE_SIZE % FLASH_PAGE_SIZE) == 0);

## B-2. 순차 쓰기 실험에서 더한 변경 (쓰기 버퍼 병합 모델, e598e75 → 66446ea)

git diff e598e75 66446ea -- nvmevirt. WBUF_MERGE=0 인 빌드(wbuffix 등)는 이 변경 전과 같은 코드가 된다(8.2 절).

diff --git a/nvmevirt/Kbuild b/nvmevirt/Kbuild

index ccba48b..1d5e2e6 100644

--- a/nvmevirt/Kbuild

+++ b/nvmevirt/Kbuild

@@ -18,9 +18,13 @@ ccflags-$(CONFIG_NVMEVIRT_SSD) += -DMAPPING_UNIT=$(MAPPING_UNIT)

# KSC2026 optional switches (default 0 = upstream behaviour):

#   WBUF_FIX=1  a write occupies whole mapping units in the write buffer (what is released after programming)

#   GC_STATS=1  log the first GC and per-partition host/GC page counters (printed at rmmod)

+#   WBUF_MERGE=1  (needs WBUF_FIX=1) writes smaller than the mapping unit are merged in the write buffer:

+#               one open mapping unit per partition, written to flash when full or replaced

WBUF_FIX ?= 0

GC_STATS ?= 0

+WBUF_MERGE ?= 0

ccflags-$(CONFIG_NVMEVIRT_SSD) += -DKSC_WBUF_FIX=$(WBUF_FIX) -DKSC_GC_STATS=$(GC_STATS)

+ccflags-$(CONFIG_NVMEVIRT_SSD) += -DKSC_WBUF_MERGE=$(WBUF_MERGE)

nvmev-$(CONFIG_NVMEVIRT_SSD) += ssd.o conv_ftl.o pqueue/pqueue.o channel_model.o

ccflags-$(CONFIG_NVMEVIRT_ZNS) += -DBASE_SSD=WD_ZN540

diff --git a/nvmevirt/conv_ftl.c b/nvmevirt/conv_ftl.c

index 24d4e77..cf2f3ed 100644

--- a/nvmevirt/conv_ftl.c

+++ b/nvmevirt/conv_ftl.c

@@ -398,6 +398,15 @@ void conv_init_namespace(struct nvmev_ns *ns, uint32_t id, uint64_t size, void *

conv_ftls[i].ksc_host_pgs = 0;

conv_ftls[i].ksc_gc_pgs = 0;

conv_ftls[i].ksc_gc_cnt = 0;

+#endif

+#if KSC_WBUF_MERGE

+        conv_ftls[i].ksc_open_lpn = INVALID_LPN;

+        bitmap_zero(conv_ftls[i].ksc_open_mask, MAPPING_UNIT / LBA_SIZE);

+        conv_ftls[i].ksc_mg_open = 0;

+        conv_ftls[i].ksc_mg_merge = 0;

+        conv_ftls[i].ksc_mg_full = 0;

+        conv_ftls[i].ksc_mg_evict = 0;

+        conv_ftls[i].ksc_mg_direct = 0;

#endif

}

@@ -423,6 +432,10 @@ void conv_init_namespace(struct nvmev_ns *ns, uint32_t id, uint64_t size, void *

NVMEV_INFO("FTL physical space: %lld, logical space: %lld (physical/logical * 100 = %d)\n",

size, ns->size, cpp.pba_pcent);

NVMEV_INFO("KSC2026: WBUF_FIX=%d GC_STATS=%d\n", KSC_WBUF_FIX, KSC_GC_STATS);

+#if KSC_WBUF_MERGE

+    NVMEV_INFO("KSC2026: WBUF_MERGE=1 (one open mapping unit per partition merges writes smaller than %u B)\n",

+           MAPPING_UNIT);

+#endif

return;

}

@@ -450,6 +463,14 @@ void conv_remove_namespace(struct nvmev_ns *ns)

conv_ftls[i].lm.free_line_cnt);

}

#endif

+#if KSC_WBUF_MERGE

+    for (i = 0; i < nr_parts; i++) {

+        NVMEV_INFO("KSC2026: merge part=%u open=%llu merge=%llu full=%llu evict=%llu direct=%llu still_open=%d\n",

+               i, conv_ftls[i].ksc_mg_open, conv_ftls[i].ksc_mg_merge, conv_ftls[i].ksc_mg_full,

+               conv_ftls[i].ksc_mg_evict, conv_ftls[i].ksc_mg_direct,

+               conv_ftls[i].ksc_open_lpn != INVALID_LPN);

+    }

+#endif

for (i = 0; i < nr_parts; i++) {

conv_remove_ftl(&conv_ftls[i]);

@@ -950,6 +971,164 @@ static bool conv_read(struct nvmev_ns *ns, struct nvmev_request *req, struct nvm

return true;

}

+#if KSC_WBUF_MERGE

+#if !KSC_WBUF_FIX

+#error "WBUF_MERGE=1 needs WBUF_FIX=1 (whole mapping units in the write buffer)"

+#endif

+/* KSC2026: write one mapping unit (global lpn) of host data to flash. Same steps as the body of the

+ * upstream conv_write() loop below; returns the program completion time, or 0 when the wordline is

+ * not full yet (nothing programmed). */

+static uint64_t ksc_write_lpn(struct conv_ftl *conv_ftls, uint32_t nr_parts, uint64_t lpn,

+                  struct nand_cmd *swr, uint32_t sq_id, struct buffer *wbuf)

+{

+    struct conv_ftl *conv_ftl = &conv_ftls[lpn % nr_parts];

+    struct ssdparams *spp = &conv_ftl->ssd->sp;

+    uint64_t local_lpn = lpn / nr_parts;

+    uint64_t nsecs_completed = 0;

+    struct ppa ppa;

+

+    ppa = get_maptbl_ent(conv_ftl, local_lpn);

+    if (mapped_ppa(&ppa)) {

+        mark_page_invalid(conv_ftl, &ppa);

+        set_rmap_ent(conv_ftl, INVALID_LPN, &ppa);

+    }

+

+    ppa = get_new_page(conv_ftl, USER_IO);

+    set_maptbl_ent(conv_ftl, local_lpn, &ppa);

+    set_rmap_ent(conv_ftl, local_lpn, &ppa);

+

+    mark_page_valid(conv_ftl, &ppa);

+#if KSC_GC_STATS

+    conv_ftl->ksc_host_pgs++;

+#endif

+

+    advance_write_pointer(conv_ftl, USER_IO);

+

+    if (last_pg_in_wordline(conv_ftl, &ppa)) {

+        swr->ppa = &ppa;

+        nsecs_completed = ssd_advance_nand(conv_ftl->ssd, swr);

+        schedule_internal_operation(sq_id, nsecs_completed, wbuf,

+                        spp->pgs_per_oneshotpg * spp->pgsz);

+    }

+

+    consume_write_credit(conv_ftl);

+    check_and_refill_write_credit(conv_ftl);

+

+    return nsecs_completed;

+}

+

+/* KSC2026 write path with write-buffer merge. Each partition has one open mapping unit:

+ *  - a write that covers only part of a mapping unit opens it (taking one mapping unit of write

+ *    buffer) or, if it is that partition's open unit already, joins it without taking buffer space;

+ *  - the open unit is written to flash when every sector of it has been written, or when another

+ *    partial write needs the partition's slot (then it is written partially filled; like the rest of

+ *    this model, the read of the old data that a real SSD would merge in is not modelled);

+ *  - a write that covers a whole mapping unit is written as in upstream (joining the open unit if

+ *    it is that unit, which completes it).

+ * As in upstream conv_write(), FUA (or write_early_completion == 0) waits only for the programs this

+ * command starts; neither FUA nor FLUSH forces an open unit (or a partly filled wordline) to flash.

+ * The controller reports no volatile write cache (VWC = 0), so the kernel sends neither of them. */

+static bool ksc_conv_write_merge(struct nvmev_ns *ns, struct nvmev_request *req,

+                 struct nvmev_result *ret)

+{

+    struct conv_ftl *conv_ftls = (struct conv_ftl *)ns->ftls;

+    struct ssdparams *spp = &conv_ftls[0].ssd->sp;

+    struct buffer *wbuf = conv_ftls[0].ssd->write_buffer;

+    struct nvme_command *cmd = req->cmd;

+    uint64_t lba = cmd->rw.slba;

+    uint64_t nr_lba = (cmd->rw.length + 1);

+    uint64_t secs_per_pg = spp->secs_per_pg;

+    uint64_t start_lpn = lba / secs_per_pg;

+    uint64_t end_lpn = (lba + nr_lba - 1) / secs_per_pg;

+    uint32_t nr_parts = ns->nr_parts;

+    uint64_t sim_open[SSD_PARTITIONS];

+    uint64_t lpn, wbuf_bytes = 0;

+    uint64_t nsecs_latest, nsecs_xfer_completed;

+    uint32_t i;

+

+    struct nand_cmd swr = {

+        .type = USER_IO,

+        .cmd = NAND_WRITE,

+        .interleave_pci_dma = false,

+        .xfer_size = spp->pgsz * spp->pgs_per_oneshotpg,

+    };

+

+    NVMEV_ASSERT(nr_parts <= SSD_PARTITIONS);

+    NVMEV_ASSERT(secs_per_pg == MAPPING_UNIT / LBA_SIZE);

+    if ((end_lpn / nr_parts) >= spp->tt_pgs) {

+        NVMEV_ERROR("%s: lpn passed FTL range (start_lpn=%lld > tt_pgs=%ld)\n", __func__,

+                start_lpn, spp->tt_pgs);

+        return false;

+    }

+

+    /* write buffer: one mapping unit for every unit this command opens or writes directly; replay

+     * the slot decisions of the loop below so that the amount matches exactly */

+    for (i = 0; i < nr_parts; i++)

+        sim_open[i] = conv_ftls[i].ksc_open_lpn;

+    for (lpn = start_lpn; lpn <= end_lpn; lpn++) {

+        uint64_t s = (lpn == start_lpn) ? lba % secs_per_pg : 0;

+        uint64_t e = (lpn == end_lpn) ? (lba + nr_lba - 1) % secs_per_pg : secs_per_pg - 1;

+        uint32_t p = lpn % nr_parts;

+

+        if (sim_open[p] == lpn)

+            continue;

+        wbuf_bytes += spp->pgsz;

+        if (!(s == 0 && e == secs_per_pg - 1))

+            sim_open[p] = lpn;

+    }

+    if (wbuf_bytes && buffer_allocate(wbuf, wbuf_bytes) < wbuf_bytes)

+        return false;

+

+    nsecs_latest = ssd_advance_write_buffer(conv_ftls[0].ssd, req->nsecs_start, LBA_TO_BYTE(nr_lba));

+    nsecs_xfer_completed = nsecs_latest;

+    swr.stime = nsecs_latest;

+

+    for (lpn = start_lpn; lpn <= end_lpn; lpn++) {

+        uint64_t s = (lpn == start_lpn) ? lba % secs_per_pg : 0;

+        uint64_t e = (lpn == end_lpn) ? (lba + nr_lba - 1) % secs_per_pg : secs_per_pg - 1;

+        struct conv_ftl *f = &conv_ftls[lpn % nr_parts];

+        uint64_t done = 0;

+

+        if (f->ksc_open_lpn == lpn) {

+            bitmap_set(f->ksc_open_mask, s, e - s + 1);

+            f->ksc_mg_merge++;

+            if (bitmap_full(f->ksc_open_mask, secs_per_pg)) {

+                f->ksc_open_lpn = INVALID_LPN;

+                f->ksc_mg_full++;

+                done = ksc_write_lpn(conv_ftls, nr_parts, lpn, &swr, req->sq_id, wbuf);

+            }

+        } else if (s == 0 && e == secs_per_pg - 1) {

+            f->ksc_mg_direct++;

+            done = ksc_write_lpn(conv_ftls, nr_parts, lpn, &swr, req->sq_id, wbuf);

+        } else {

+            if (f->ksc_open_lpn != INVALID_LPN) {

+                uint64_t old = f->ksc_open_lpn;

+

+                f->ksc_open_lpn = INVALID_LPN;

+                f->ksc_mg_evict++;

+                done = ksc_write_lpn(conv_ftls, nr_parts, old, &swr, req->sq_id, wbuf);

+            }

+            f->ksc_open_lpn = lpn;

+            bitmap_zero(f->ksc_open_mask, secs_per_pg);

+            bitmap_set(f->ksc_open_mask, s, e - s + 1);

+            f->ksc_mg_open++;

+        }

+        nsecs_latest = max(done, nsecs_latest);

+    }

+

+    if ((cmd->rw.control & NVME_RW_FUA) || (spp->write_early_completion == 0)) {

+        /* Wait all flash operations */

+        ret->nsecs_target = nsecs_latest;

+    } else {

+        /* Early completion */

+        ret->nsecs_target = nsecs_xfer_completed;

+    }

+    ret->status = NVME_SC_SUCCESS;

+

+    return true;

+}

+#endif

+

static bool conv_write(struct nvmev_ns *ns, struct nvmev_request *req, struct nvmev_result *ret)

{

struct conv_ftl *conv_ftls = (struct conv_ftl *)ns->ftls;

@@ -1090,8 +1269,13 @@ bool conv_proc_nvme_io_cmd(struct nvmev_ns *ns, struct nvmev_request *req, struc

switch (cmd->common.opcode) {

case nvme_cmd_write:

+#if KSC_WBUF_MERGE

+        if (!ksc_conv_write_merge(ns, req, ret))

+            return false;

+#else

if (!conv_write(ns, req, ret))

return false;

+#endif

break;

case nvme_cmd_read:

if (!conv_read(ns, req, ret))

diff --git a/nvmevirt/conv_ftl.h b/nvmevirt/conv_ftl.h

index 486eced..61a9693 100644

--- a/nvmevirt/conv_ftl.h

+++ b/nvmevirt/conv_ftl.h

@@ -4,6 +4,7 @@

#define _NVMEVIRT_CONV_FTL_H

#include <linux/types.h>

+#include <linux/bitmap.h>

#include "pqueue/pqueue.h"

#include "ssd_config.h"

#include "ssd.h"

@@ -73,6 +74,17 @@ struct conv_ftl {

uint64_t ksc_gc_pgs; /* mapping-unit pages copied by GC */

uint64_t ksc_gc_cnt; /* victim lines cleaned */

#endif

+#if KSC_WBUF_MERGE

+    /* KSC2026 write-buffer merge (only touched by the dispatcher thread): one open mapping unit per

+     * partition collects host writes smaller than the mapping unit until it is full or replaced */

+    uint64_t ksc_open_lpn; /* global lpn of the open mapping unit, INVALID_LPN if none */

+    DECLARE_BITMAP(ksc_open_mask, MAPPING_UNIT / LBA_SIZE); /* sectors of the open unit written so far */

+    uint64_t ksc_mg_open; /* units opened by a partial write */

+    uint64_t ksc_mg_merge; /* writes that joined the open unit */

+    uint64_t ksc_mg_full; /* open units written to flash because they became full */

+    uint64_t ksc_mg_evict; /* open units written to flash partially filled (replaced by another unit) */

+    uint64_t ksc_mg_direct; /* whole-unit writes that did not touch the open unit */

+#endif

};

void conv_init_namespace(struct nvmev_ns *ns, uint32_t id, uint64_t size, void *mapped_addr,

# 부록 C. 실험 전 환경 스냅샷 (env_before)

exp/results/main3x3_20261008/env_before/ 의 원본 출력을 그대로 옮겼다. 출력 안의 시각은 서버 시계 UTC 다(KST = UTC + 9 시간; 예: 09_block.txt 의 ls 시각 'Oct  7 12:22' = 10 월 7 일 21:22 KST, 00_date.txt 의 07:00:35+00:00 = 10 월 8 일 16:00:35 KST).

## C. 00_date.txt

$ date -Is

2026-10-08T07:00:35+00:00

$ hostname

dccearth

## C. 01_os.txt

$ uname -a

Linux dccearth 6.8.0-142-generic #142-Ubuntu SMP PREEMPT_DYNAMIC Wed Sep  2 14:24:27 UTC 2026 x86_64 x86_64 x86_64 GNU/Linux

$ cat /etc/os-release

PRETTY_NAME="Ubuntu 24.04.4 LTS"

NAME="Ubuntu"

VERSION_ID="24.04"

VERSION="24.04.4 LTS (Noble Numbat)"

VERSION_CODENAME=noble

ID=ubuntu

ID_LIKE=debian

HOME_URL="https://www.ubuntu.com/"

SUPPORT_URL="https://help.ubuntu.com/"

BUG_REPORT_URL="https://bugs.launchpad.net/ubuntu/"

PRIVACY_POLICY_URL="https://www.ubuntu.com/legal/terms-and-policies/privacy-policy"

UBUNTU_CODENAME=noble

LOGO=ubuntu-logo

$ cat /proc/version

Linux version 6.8.0-142-generic (buildd@lcy02-amd64-049) (x86_64-linux-gnu-gcc-13 (Ubuntu 13.3.0-6ubuntu2~24.04.1) 13.3.0, GNU ld (GNU Binutils for Ubuntu) 2.42) #142-Ubuntu SMP PREEMPT_DYNAMIC Wed Sep  2 14:24:27 UTC 2026

## C. 02_boot.txt

$ cat /proc/cmdline

BOOT_IMAGE=/boot/vmlinuz-6.8.0-142-generic root=UUID=bb1b37f9-0806-4044-a2bf-5ff988ecb286 ro memmap=12G$12G isolcpus=3-5

$ grep -E ^GRUB_CMDLINE /etc/default/grub

GRUB_CMDLINE_LINUX_DEFAULT=""

GRUB_CMDLINE_LINUX="memmap=12G\\\$12G isolcpus=3-5"

$ cat /sys/devices/system/cpu/isolated

3-5

## C. 03_cpu.txt

$ lscpu

Architecture:                            x86_64

CPU op-mode(s):                          32-bit, 64-bit

Address sizes:                           46 bits physical, 48 bits virtual

Byte Order:                              Little Endian

CPU(s):                                  6

On-line CPU(s) list:                     0-5

Vendor ID:                               GenuineIntel

Model name:                              13th Gen Intel(R) Core(TM) i5-13600K

CPU family:                              6

Model:                                   183

Thread(s) per core:                      1

Core(s) per socket:                      6

Socket(s):                               1

Stepping:                                1

CPU(s) scaling MHz:                      58%

CPU max MHz:                             5100.0000

CPU min MHz:                             800.0000

BogoMIPS:                                6988.80

Flags:                                   fpu vme de pse tsc msr pae mce cx8 apic sep mtrr pge mca cmov pat pse36 clflush dts acpi mmx fxsr sse sse2 ss ht tm pbe syscall nx pdpe1gb rdtscp lm constant_tsc art arch_perfmon pebs bts rep_good nopl xtopology nonstop_tsc cpuid aperfmperf tsc_known_freq pni pclmulqdq dtes64 monitor ds_cpl vmx smx est tm2 ssse3 sdbg fma cx16 xtpr pdcm pcid sse4_1 sse4_2 x2apic movbe popcnt tsc_deadline_timer aes xsave avx f16c rdrand lahf_lm abm 3dnowprefetch cpuid_fault epb ssbd ibrs ibpb stibp ibrs_enhanced tpr_shadow flexpriority ept vpid ept_ad fsgsbase tsc_adjust bmi1 avx2 smep bmi2 erms invpcid rdseed adx smap clflushopt clwb intel_pt sha_ni xsaveopt xsavec xgetbv1 xsaves split_lock_detect user_shstk avx_vnni dtherm ida arat pln pts hwp hwp_notify hwp_act_window hwp_epp hwp_pkg_req hfi vnmi umip pku ospke waitpkg gfni vaes vpclmulqdq tme rdpid movdiri movdir64b fsrm md_clear serialize pconfig arch_lbr ibt flush_l1d arch_capabilities ibpb_exit_to_user

Virtualization:                          VT-x

L1d cache:                               288 KiB (6 instances)

L1i cache:                               192 KiB (6 instances)

L2 cache:                                12 MiB (6 instances)

L3 cache:                                24 MiB (1 instance)

NUMA node(s):                            1

NUMA node0 CPU(s):                       0-5

Vulnerability Gather data sampling:      Not affected

Vulnerability Indirect target selection: Not affected

Vulnerability Itlb multihit:             Not affected

Vulnerability L1tf:                      Not affected

Vulnerability Mds:                       Not affected

Vulnerability Meltdown:                  Not affected

Vulnerability Mmio stale data:           Not affected

Vulnerability Reg file data sampling:    Mitigation; Clear Register File

Vulnerability Retbleed:                  Not affected

Vulnerability Spec rstack overflow:      Not affected

Vulnerability Spec store bypass:         Mitigation; Speculative Store Bypass disabled via prctl

Vulnerability Spectre v1:                Mitigation; usercopy/swapgs barriers and __user pointer sanitization

Vulnerability Spectre v2:                Mitigation; Enhanced / Automatic IBRS; IBPB conditional; PBRSB-eIBRS SW sequence; BHI BHI_DIS_S

Vulnerability Srbds:                     Not affected

Vulnerability Tsa:                       Not affected

Vulnerability Tsx async abort:           Not affected

Vulnerability Vmscape:                   Mitigation; IBPB before exit to userspace

$ nproc

3

## C. 04_cpufreq.txt

$ cat /sys/devices/system/cpu/intel_pstate/status /sys/devices/system/cpu/intel_pstate/no_turbo

active

0

$ grep -H . /sys/devices/system/cpu/cpu0/cpufreq/scaling_driver /sys/devices/system/cpu/cpu1/cpufreq/scaling_driver /sys/devices/system/cpu/cpu2/cpufreq/scaling_driver /sys/devices/system/cpu/cpu3/cpufreq/scaling_driver /sys/devices/system/cpu/cpu4/cpufreq/scaling_driver /sys/devices/system/cpu/cpu5/cpufreq/scaling_driver /sys/devices/system/cpu/cpu0/cpufreq/scaling_governor /sys/devices/system/cpu/cpu1/cpufreq/scaling_governor /sys/devices/system/cpu/cpu2/cpufreq/scaling_governor /sys/devices/system/cpu/cpu3/cpufreq/scaling_governor /sys/devices/system/cpu/cpu4/cpufreq/scaling_governor /sys/devices/system/cpu/cpu5/cpufreq/scaling_governor /sys/devices/system/cpu/cpu0/cpufreq/energy_performance_preference /sys/devices/system/cpu/cpu1/cpufreq/energy_performance_preference /sys/devices/system/cpu/cpu2/cpufreq/energy_performance_preference /sys/devices/system/cpu/cpu3/cpufreq/energy_performance_preference /sys/devices/system/cpu/cpu4/cpufreq/energy_performance_preference /sys/devices/system/cpu/cpu5/cpufreq/energy_performance_preference /sys/devices/system/cpu/cpu0/cpufreq/scaling_min_freq /sys/devices/system/cpu/cpu1/cpufreq/scaling_min_freq /sys/devices/system/cpu/cpu2/cpufreq/scaling_min_freq /sys/devices/system/cpu/cpu3/cpufreq/scaling_min_freq /sys/devices/system/cpu/cpu4/cpufreq/scaling_min_freq /sys/devices/system/cpu/cpu5/cpufreq/scaling_min_freq /sys/devices/system/cpu/cpu0/cpufreq/scaling_max_freq /sys/devices/system/cpu/cpu1/cpufreq/scaling_max_freq /sys/devices/system/cpu/cpu2/cpufreq/scaling_max_freq /sys/devices/system/cpu/cpu3/cpufreq/scaling_max_freq /sys/devices/system/cpu/cpu4/cpufreq/scaling_max_freq /sys/devices/system/cpu/cpu5/cpufreq/scaling_max_freq

/sys/devices/system/cpu/cpu0/cpufreq/scaling_driver:intel_pstate

/sys/devices/system/cpu/cpu1/cpufreq/scaling_driver:intel_pstate

/sys/devices/system/cpu/cpu2/cpufreq/scaling_driver:intel_pstate

/sys/devices/system/cpu/cpu3/cpufreq/scaling_driver:intel_pstate

/sys/devices/system/cpu/cpu4/cpufreq/scaling_driver:intel_pstate

/sys/devices/system/cpu/cpu5/cpufreq/scaling_driver:intel_pstate

/sys/devices/system/cpu/cpu0/cpufreq/scaling_governor:powersave

/sys/devices/system/cpu/cpu1/cpufreq/scaling_governor:powersave

/sys/devices/system/cpu/cpu2/cpufreq/scaling_governor:powersave

/sys/devices/system/cpu/cpu3/cpufreq/scaling_governor:powersave

/sys/devices/system/cpu/cpu4/cpufreq/scaling_governor:powersave

/sys/devices/system/cpu/cpu5/cpufreq/scaling_governor:powersave

/sys/devices/system/cpu/cpu0/cpufreq/energy_performance_preference:balance_performance

/sys/devices/system/cpu/cpu1/cpufreq/energy_performance_preference:balance_performance

/sys/devices/system/cpu/cpu2/cpufreq/energy_performance_preference:balance_performance

/sys/devices/system/cpu/cpu3/cpufreq/energy_performance_preference:balance_performance

/sys/devices/system/cpu/cpu4/cpufreq/energy_performance_preference:balance_performance

/sys/devices/system/cpu/cpu5/cpufreq/energy_performance_preference:balance_performance

/sys/devices/system/cpu/cpu0/cpufreq/scaling_min_freq:800000

/sys/devices/system/cpu/cpu1/cpufreq/scaling_min_freq:800000

/sys/devices/system/cpu/cpu2/cpufreq/scaling_min_freq:800000

/sys/devices/system/cpu/cpu3/cpufreq/scaling_min_freq:800000

/sys/devices/system/cpu/cpu4/cpufreq/scaling_min_freq:800000

/sys/devices/system/cpu/cpu5/cpufreq/scaling_min_freq:800000

/sys/devices/system/cpu/cpu0/cpufreq/scaling_max_freq:5100000

/sys/devices/system/cpu/cpu1/cpufreq/scaling_max_freq:5100000

/sys/devices/system/cpu/cpu2/cpufreq/scaling_max_freq:5100000

/sys/devices/system/cpu/cpu3/cpufreq/scaling_max_freq:5100000

/sys/devices/system/cpu/cpu4/cpufreq/scaling_max_freq:5100000

/sys/devices/system/cpu/cpu5/cpufreq/scaling_max_freq:5100000

## C. 05_memory.txt

$ free -b

total        used        free      shared  buff/cache   available

Mem:     12299948032   885612544 11315458048    12660736   380661760 11414335488

Swap:     8589930496      262144  8589668352

$ grep -E MemTotal|MemFree|MemAvailable|Hugepages|AnonHugePages /proc/meminfo

MemTotal:       12011668 kB

MemFree:        11050252 kB

MemAvailable:   11146812 kB

AnonHugePages:         0 kB

Hugepagesize:       2048 kB

$ cat /sys/kernel/mm/transparent_hugepage/enabled

always [madvise] never

$ sudo -n cat /proc/iomem

00000000-00000fff : Reserved

00001000-0009dfff : System RAM

0009e000-0009efff : Reserved

0009f000-0009ffff : System RAM

000a0000-000fffff : Reserved

000a0000-000bffff : PCI Bus 0000:00

000e0000-000fffff : PCI Bus 0000:00

000f0000-000fffff : System ROM

00100000-6ad8a017 : System RAM

01000000-025fffff : Kernel code

02600000-03416fff : Kernel rodata

03600000-03a5677f : Kernel data

03f56000-043fffff : Kernel bss

6ad8a018-6ad97857 : System RAM

6ad97858-6bfedfff : System RAM

6bfee000-6bfeefff : Reserved

6bfef000-71f90fff : System RAM

71f91000-7433cfff : Reserved

7433d000-74448fff : ACPI Tables

74449000-74648fff : ACPI Non-volatile Storage

74649000-75bfefff : Reserved

75bff000-75bfffff : System RAM

75c00000-79ffffff : Reserved

7a400000-7a7fffff : Reserved

7b000000-807fffff : Reserved

7c800000-807fffff : Graphics Stolen Memory

80800000-bfffffff : PCI Bus 0000:00

80800000-811fffff : PCI Bus 0000:02

81200000-812fffff : PCI Bus 0000:03

81200000-8120ffff : 0000:03:00.0

81200000-8120ffff : r8169

81210000-81213fff : 0000:03:00.0

81300000-813fffff : PCI Bus 0000:01

81300000-81303fff : 0000:01:00.0

81300000-81303fff : nvme

81400000-81401fff : 0000:00:17.0

81400000-81401fff : ahci

81402000-814027ff : 0000:00:17.0

81402000-814027ff : ahci

81403000-814030ff : 0000:00:17.0

81403000-814030ff : ahci

81404000-81404fff : 0000:00:1f.5

81404000-81404fff : 0000:00:1f.5 0000:00:1f.5

c0000000-cfffffff : PCI ECAM 0000 [bus 00-ff]

e0690000-e069ffff : INTC1085:00

e0690000-e069ffff : INTC1085:00 INTC1085:00

e06a0000-e06affff : INTC1085:00

e06a0000-e06affff : INTC1085:00 INTC1085:00

e06b0000-e06bffff : INTC1085:00

e06b0000-e06bffff : INTC1085:00 INTC1085:00

e06d0000-e06dffff : INTC1085:00

e06d0000-e06dffff : INTC1085:00 INTC1085:00

e06e0000-e06effff : INTC1085:00

e06e0000-e06effff : INTC1085:00 INTC1085:00

fe000000-fe010fff : Reserved

fec00000-fec00fff : Reserved

fec00000-fec003ff : IOAPIC 0

fed00000-fed00fff : Reserved

fed00000-fed003ff : HPET 0

fed00000-fed003ff : PNP0103:00

fed20000-fed7ffff : Reserved

fed40000-fed44fff : MSFT0101:00

fed40000-fed44fff : MSFT0101:00

fed90000-fed90fff : dmar0

fed91000-fed91fff : dmar1

feda0000-feda0fff : pnp 00:03

feda1000-feda1fff : pnp 00:03

fedc0000-fedc7fff : pnp 00:03

fee00000-fee00fff : Reserved

100000000-2ffffffff : System RAM

300000000-5ffffffff : Reserved

600000000-67f7fffff : System RAM

67f800000-67fffffff : RAM buffer

4000000000-7fffffffff : PCI Bus 0000:00

4000000000-400fffffff : 0000:00:02.0

4010000000-4016ffffff : 0000:00:02.0

4017000000-40171fffff : PCI Bus 0000:02

4020000000-40ffffffff : 0000:00:02.0

6000000000-6000ffffff : 0000:00:02.0

6001000000-60010fffff : 0000:00:1f.3

6001100000-600110ffff : 0000:00:14.0

6001100000-600110ffff : xhci-hcd

6001110000-6001117fff : 0000:00:0a.0

6001114000-6001114d9f : telem1

6001114da0-6001114edf : telem2

60011164d8-60011164e7 : intel_vsec.telemetry.0

60011164e8-60011164f7 : intel_vsec.telemetry.0

60011164f8-6001116507 : intel_vsec.telemetry.0

6001116508-6001116517 : intel_vsec.telemetry.0

6001120000-6001123fff : 0000:00:1f.3

6001120000-6001123fff : ICH HD audio

6001124000-6001127fff : 0000:00:14.2

6001128000-60011280ff : 0000:00:1f.4

6001129000-6001129fff : 0000:00:16.0

6001129000-6001129fff : mei_me

600112a000-600112afff : 0000:00:14.2

600112b000-600112bfff : 0000:00:08.0

$ bash -c sudo -n dmesg | grep -iE 'e820|memmap|user-defined' | head -60

## C. 06_platform.txt

$ grep -H . /sys/class/dmi/id/sys_vendor /sys/class/dmi/id/product_name /sys/class/dmi/id/board_vendor /sys/class/dmi/id/board_name /sys/class/dmi/id/bios_version /sys/class/dmi/id/bios_date

/sys/class/dmi/id/sys_vendor:Micro-Star International Co., Ltd.

/sys/class/dmi/id/product_name:MS-7E01

/sys/class/dmi/id/board_vendor:Micro-Star International Co., Ltd.

/sys/class/dmi/id/board_name:MAG B760M MORTAR (MS-7E01)

/sys/class/dmi/id/bios_version:M.30

/sys/class/dmi/id/bios_date:05/16/2023

## C. 07_versions.txt

$ fio --version

fio-3.36

$ gcc --version

gcc (Ubuntu 13.3.0-6ubuntu2~24.04.1) 13.3.0

Copyright (C) 2023 Free Software Foundation, Inc.

This is free software; see the source for copying conditions.  There is NO

warranty; not even for MERCHANTABILITY or FITNESS FOR A PARTICULAR PURPOSE.

$ make --version

GNU Make 4.3

Built for x86_64-pc-linux-gnu

Copyright (C) 1988-2020 Free Software Foundation, Inc.

License GPLv3+: GNU GPL version 3 or later <http://gnu.org/licenses/gpl.html>

This is free software: you are free to change and redistribute it.

There is NO WARRANTY, to the extent permitted by law.

$ python3 --version

Python 3.12.3

$ git --version

git version 2.43.0

$ nvme version

nvme version 2.8 (git 2.8)

libnvme version 1.8 (git 1.8)

$ dmesg --version

dmesg from util-linux 2.39.3

$ bash --version

GNU bash, version 5.2.21(1)-release (x86_64-pc-linux-gnu)

Copyright (C) 2022 Free Software Foundation, Inc.

License GPLv3+: GNU GPL version 3 or later <http://gnu.org/licenses/gpl.html>

This is free software; you are free to change and redistribute it.

There is NO WARRANTY, to the extent permitted by law.

$ dpkg-query -W fio libaio1t64 nvme-cli make gcc-13 linux-headers-6.8.0-142-generic linux-image-6.8.0-142-generic

fio    3.36-1ubuntu0.1

gcc-13    13.3.0-6ubuntu2~24.04.1

libaio1t64:amd64    0.3.113-6build1.1

linux-headers-6.8.0-142-generic    6.8.0-142.142

linux-image-6.8.0-142-generic    6.8.0-142.142

make    4.3-4.1build2

nvme-cli    2.8-1ubuntu0.1

## C. 08_nvmevirt_git.txt

$ git -C /home/dccearth/jsw/KSC2026/nvmevirt rev-parse HEAD

1d6cd036893f06bfe8aa8f6bb4327239bc4b720f

$ git -C /home/dccearth/jsw/KSC2026/nvmevirt log --oneline -15

1d6cd03 exp: final 3x3 design with OS page-cache comparison

2a462a3 exp: keep going when a run fails; plot tool; report generators

5769378 exp: scripts for the KSC2026 mapping-unit experiment

c02b9fd nvmevirt: KSC2026 mapping-unit configuration

d508610 Move NVMeVirt sources into nvmevirt/ subdirectory

61c90f7 Merge pull request #72 from duckhanson/fix/zns-append-return-slba

b48962a Merge pull request #71 from enkerewpo/fix

e808a49 zns: return allocated SLBA in CQE for Zone Append commands

d9b62a2 fix: add E820_TYPE_RESERVED_KERN check and fixed access out-of-bound bug

a6a4252 misc: fixed miscellaneous bugs

c8ef01d dma: support async DMA processing

00a9ee1 Merge pull request #62 from WithoutStress/main

c93e88c io: Support I/Os from peripheral devices

e26d4ef pci: fix queue phase initlization

519e19f io: fix lock in __fill_cq_result()

$ git -C /home/dccearth/jsw/KSC2026/nvmevirt status --porcelain

?? exp/modules/

?? exp/results/

$ git -C /home/dccearth/jsw/KSC2026/nvmevirt remote -v

origin    git@github.com:Sangwon8799/nvmevirt.git (fetch)

origin    git@github.com:Sangwon8799/nvmevirt.git (push)

upstream    https://github.com/snu-csl/nvmevirt.git (fetch)

upstream    https://github.com/snu-csl/nvmevirt.git (push)

## C. 09_block.txt

$ lsblk -o NAME,SIZE,MODEL,TYPE,MOUNTPOINT

NAME          SIZE MODEL                     TYPE MOUNTPOINT

nvme0n1     465.8G Samsung SSD 980 PRO 500GB disk

├─nvme0n1p1     1G                           part /boot/efi

└─nvme0n1p2 464.7G                           part /

$ bash -c ls -l /dev/nvme*

crw------- 1 root root 241,   0 Oct  7 12:22 /dev/nvme0

brw-rw---- 1 root disk 259,   0 Oct  7 12:22 /dev/nvme0n1

brw-rw---- 1 root disk 259,   1 Oct  7 12:22 /dev/nvme0n1p1

brw-rw---- 1 root disk 259,   2 Oct  7 12:22 /dev/nvme0n1p2

crw------- 1 root root  10, 261 Oct  7 12:22 /dev/nvme-fabrics

$ sudo -n nvme list

Node                  Generic               SN                   Model                                    Namespace  Usage                      Format           FW Rev

--------------------- --------------------- -------------------- ---------------------------------------- ---------- -------------------------- ---------------- --------

/dev/nvme0n1          /dev/ng0n1            S5GYNF0RA00765J      Samsung SSD 980 PRO 500GB                0x1        121.44  GB / 500.11  GB    512   B +  0 B   3B2QGXA7

## C. 10_modules.txt

$ bash -c lsmod | grep -E '^(Module|nvmev)'

Module                  Size  Used by

$ cat /proc/loadavg

0.80 1.18 1.96 1/244 51475

## C. 11_kernel_sysctl.txt

$ sysctl kernel.dmesg_restrict kernel.kptr_restrict kernel.numa_balancing vm.swappiness

kernel.dmesg_restrict = 1

kernel.kptr_restrict = 1

kernel.numa_balancing = 0

vm.swappiness = 60

# 부록 D. 회차별 원자료

표 66. 회차별 결과 — main3x3_20261008 (analysis/summary_runs.csv)

| 변형 | 매핑 | bs | 회 | MiB/s | IOPS | clat 평균 µs | clat p99 µs | 첫 GC s | WAF total | 채널모델 오류 줄 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wbuffix_drop | 4K | 4K | 1 | 415.9 | 106,482 | 299.5 | 13,435 | 6.34 | 3.12 | 0 |
| wbuffix_drop | 4K | 4K | 2 | 418.6 | 107,172 | 297.5 | 13,435 | 6.37 | 3.15 | 0 |
| wbuffix_drop | 4K | 4K | 3 | 418.2 | 107,065 | 297.9 | 13,304 | 6.31 | 3.14 | 0 |
| wbuffix_drop | 4K | 16K | 1 | 480.6 | 30,760 | 1,039 | 17,695 | 5.72 | 3.55 | 0 |
| wbuffix_drop | 4K | 16K | 2 | 481.0 | 30,782 | 1,038 | 17,695 | 5.73 | 3.55 | 0 |
| wbuffix_drop | 4K | 16K | 3 | 480.9 | 30,776 | 1,038 | 17,695 | 5.72 | 3.55 | 0 |
| wbuffix_drop | 4K | 32K | 1 | 482.3 | 15,433 | 2,072 | 18,219 | 5.73 | 3.54 | 0 |
| wbuffix_drop | 4K | 32K | 2 | 482.4 | 15,438 | 2,071 | 18,219 | 5.73 | 3.54 | 0 |
| wbuffix_drop | 4K | 32K | 3 | 482.2 | 15,430 | 2,072 | 18,219 | 5.73 | 3.54 | 0 |
| wbuffix_drop | 16K | 4K | 1 | 129.8 | 33,221 | 962.1 | 14,221 | 5.95 | 8.83 | 0 |
| wbuffix_drop | 16K | 4K | 2 | 130.2 | 33,332 | 958.9 | 14,090 | 5.95 | 8.87 | 0 |
| wbuffix_drop | 16K | 4K | 3 | 129.6 | 33,174 | 963.5 | 14,090 | 5.95 | 8.81 | 0 |
| wbuffix_drop | 16K | 16K | 1 | 420.4 | 26,903 | 1,188 | 16,712 | 5.95 | 3.12 | 0 |
| wbuffix_drop | 16K | 16K | 2 | 419.5 | 26,846 | 1,190 | 16,712 | 5.95 | 3.11 | 0 |
| wbuffix_drop | 16K | 16K | 3 | 421.2 | 26,955 | 1,186 | 16,712 | 5.96 | 3.13 | 0 |
| wbuffix_drop | 16K | 32K | 1 | 440.4 | 14,094 | 2,269 | 18,219 | 5.88 | 3.31 | 0 |
| wbuffix_drop | 16K | 32K | 2 | 440.2 | 14,087 | 2,270 | 18,219 | 5.87 | 3.30 | 0 |
| wbuffix_drop | 16K | 32K | 3 | 441.3 | 14,121 | 2,264 | 18,219 | 5.88 | 3.31 | 0 |
| wbuffix_drop | 32K | 4K | 1 | 68.3 | 17,479 | 1,830 | 14,352 | 6.09 | 17.07 | 0 |
| wbuffix_drop | 32K | 4K | 2 | 68.1 | 17,444 | 1,833 | 14,352 | 6.09 | 17.03 | 0 |
| wbuffix_drop | 32K | 4K | 3 | 68.4 | 17,498 | 1,828 | 14,483 | 6.09 | 17.09 | 0 |
| wbuffix_drop | 32K | 16K | 1 | 219.6 | 14,055 | 2,275 | 17,695 | 6.09 | 5.95 | 0 |
| wbuffix_drop | 32K | 16K | 2 | 219.6 | 14,053 | 2,276 | 17,433 | 6.10 | 5.96 | 0 |
| wbuffix_drop | 32K | 16K | 3 | 219.4 | 14,039 | 2,278 | 17,433 | 6.09 | 5.95 | 0 |
| wbuffix_drop | 32K | 32K | 1 | 423.8 | 13,561 | 2,358 | 17,695 | 6.12 | 3.11 | 0 |
| wbuffix_drop | 32K | 32K | 2 | 424.6 | 13,589 | 2,353 | 17,433 | 6.13 | 3.12 | 0 |
| wbuffix_drop | 32K | 32K | 3 | 424.4 | 13,580 | 2,355 | 17,433 | 6.12 | 3.11 | 0 |
| wbuffix_nodrop | 4K | 4K | 1 | 414.7 | 106,175 | 300.4 | 13,435 | 6.28 | 3.10 | 0 |
| wbuffix_nodrop | 4K | 4K | 2 | 415.5 | 106,369 | 299.8 | 13,304 | 6.34 | 3.11 | 0 |
| wbuffix_nodrop | 4K | 4K | 3 | 417.7 | 106,942 | 298.2 | 13,304 | 6.33 | 3.14 | 0 |
| wbuffix_nodrop | 4K | 16K | 1 | 481.0 | 30,784 | 1,038 | 17,695 | 5.70 | 3.55 | 0 |
| wbuffix_nodrop | 4K | 16K | 2 | 481.0 | 30,785 | 1,038 | 17,695 | 5.69 | 3.55 | 0 |
| wbuffix_nodrop | 4K | 16K | 3 | 480.9 | 30,775 | 1,038 | 17,695 | 5.69 | 3.55 | 0 |
| wbuffix_nodrop | 4K | 32K | 1 | 482.5 | 15,439 | 2,071 | 18,219 | 5.69 | 3.54 | 0 |
| wbuffix_nodrop | 4K | 32K | 2 | 482.2 | 15,432 | 2,072 | 18,219 | 5.69 | 3.54 | 0 |
| wbuffix_nodrop | 4K | 32K | 3 | 482.1 | 15,427 | 2,072 | 18,219 | 5.69 | 3.54 | 0 |
| wbuffix_nodrop | 16K | 4K | 1 | 129.7 | 33,205 | 962.6 | 14,090 | 5.91 | 8.82 | 0 |
| wbuffix_nodrop | 16K | 4K | 2 | 129.8 | 33,219 | 962.2 | 13,959 | 5.91 | 8.82 | 0 |
| wbuffix_nodrop | 16K | 4K | 3 | 129.7 | 33,203 | 962.7 | 14,090 | 5.91 | 8.82 | 0 |
| wbuffix_nodrop | 16K | 16K | 1 | 419.2 | 26,828 | 1,191 | 16,712 | 5.92 | 3.11 | 0 |
| wbuffix_nodrop | 16K | 16K | 2 | 420.2 | 26,894 | 1,188 | 16,908 | 5.92 | 3.12 | 0 |
| wbuffix_nodrop | 16K | 16K | 3 | 421.8 | 26,992 | 1,184 | 16,908 | 5.92 | 3.14 | 0 |
| wbuffix_nodrop | 16K | 32K | 1 | 440.2 | 14,085 | 2,270 | 18,219 | 5.84 | 3.31 | 0 |
| wbuffix_nodrop | 16K | 32K | 2 | 442.7 | 14,166 | 2,257 | 18,219 | 5.85 | 3.33 | 0 |
| wbuffix_nodrop | 16K | 32K | 3 | 441.9 | 14,141 | 2,261 | 18,219 | 5.84 | 3.32 | 0 |
| wbuffix_nodrop | 32K | 4K | 1 | 68.3 | 17,484 | 1,829 | 14,483 | 6.06 | 17.08 | 0 |
| wbuffix_nodrop | 32K | 4K | 2 | 68.2 | 17,469 | 1,831 | 14,483 | 6.06 | 17.06 | 0 |
| wbuffix_nodrop | 32K | 4K | 3 | 68.3 | 17,493 | 1,828 | 14,483 | 6.06 | 17.09 | 0 |
| wbuffix_nodrop | 32K | 16K | 1 | 219.1 | 14,025 | 2,280 | 17,695 | 6.06 | 5.94 | 0 |
| wbuffix_nodrop | 32K | 16K | 2 | 219.9 | 14,072 | 2,273 | 17,695 | 6.06 | 5.98 | 0 |
| wbuffix_nodrop | 32K | 16K | 3 | 219.7 | 14,060 | 2,274 | 17,433 | 6.06 | 5.96 | 0 |
| wbuffix_nodrop | 32K | 32K | 1 | 424.5 | 13,585 | 2,354 | 17,695 | 6.10 | 3.12 | 0 |
| wbuffix_nodrop | 32K | 32K | 2 | 424.8 | 13,595 | 2,352 | 17,695 | 6.10 | 3.12 | 0 |
| wbuffix_nodrop | 32K | 32K | 3 | 424.6 | 13,587 | 2,353 | 17,695 | 6.10 | 3.12 | 0 |

표 67. 회차별 결과 — main_20261008 (analysis/summary_runs.csv)

| 변형 | 매핑 | bs | 회 | MiB/s | IOPS | clat 평균 µs | clat p99 µs | 첫 GC s | WAF total | 채널모델 오류 줄 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wbuffix | 4K | 4K | 1 | 419.5 | 107,392 | 296.9 | 13,435 | 6.31 | 3.16 | 0 |
| wbuffix | 4K | 4K | 2 | 417.6 | 106,915 | 298.3 | 13,566 | 6.30 | 3.14 | 0 |
| wbuffix | 4K | 8K | 1 | 439.3 | 56,230 | 567.9 | 16,581 | 5.75 | 3.34 | 0 |
| wbuffix | 4K | 8K | 2 | 438.0 | 56,064 | 569.5 | 16,581 | 5.74 | 3.33 | 0 |
| wbuffix | 4K | 16K | 1 | 480.9 | 30,777 | 1,038 | 17,695 | 5.69 | 3.55 | 0 |
| wbuffix | 4K | 16K | 2 | 480.9 | 30,778 | 1,038 | 17,695 | 5.70 | 3.55 | 0 |
| wbuffix | 4K | 32K | 1 | 482.4 | 15,436 | 2,071 | 18,219 | 5.70 | 3.54 | 0 |
| wbuffix | 4K | 32K | 2 | 482.3 | 15,433 | 2,072 | 18,219 | 5.70 | 3.54 | 0 |
| wbuffix | 4K | 64K | 1 | 486.3 | 7,780 | 4,110 | 19,005 | 5.70 | 3.53 | 0 |
| wbuffix | 4K | 64K | 2 | 486.3 | 7,780 | 4,110 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 4K | 128K | 1 | 496.9 | 3,975 | 8,046 | 20,316 | 5.70 | 3.49 | 0 |
| wbuffix | 4K | 128K | 2 | 496.8 | 3,975 | 8,046 | 20,578 | 5.70 | 3.49 | 0 |
| wbuffix | 8K | 4K | 1 | 216.7 | 55,481 | 575.7 | 15,008 | 5.81 | 5.89 | 0 |
| wbuffix | 8K | 4K | 2 | 216.6 | 55,453 | 576.0 | 15,008 | 5.82 | 5.89 | 0 |
| wbuffix | 8K | 8K | 1 | 420.1 | 53,767 | 593.9 | 15,401 | 5.82 | 3.15 | 0 |
| wbuffix | 8K | 8K | 2 | 416.9 | 53,361 | 598.5 | 15,401 | 5.82 | 3.11 | 0 |
| wbuffix | 8K | 16K | 1 | 436.8 | 27,954 | 1,143 | 17,433 | 5.78 | 3.30 | 0 |
| wbuffix | 8K | 16K | 2 | 439.1 | 28,099 | 1,137 | 17,433 | 5.78 | 3.32 | 0 |
| wbuffix | 8K | 32K | 1 | 482.4 | 15,435 | 2,071 | 18,219 | 5.70 | 3.54 | 0 |
| wbuffix | 8K | 32K | 2 | 482.3 | 15,432 | 2,072 | 18,219 | 5.70 | 3.54 | 0 |
| wbuffix | 8K | 64K | 1 | 486.4 | 7,782 | 4,109 | 19,005 | 5.70 | 3.53 | 0 |
| wbuffix | 8K | 64K | 2 | 486.4 | 7,782 | 4,109 | 19,005 | 5.70 | 3.53 | 0 |
| wbuffix | 8K | 128K | 1 | 496.7 | 3,974 | 8,048 | 20,578 | 5.70 | 3.49 | 0 |
| wbuffix | 8K | 128K | 2 | 496.7 | 3,974 | 8,048 | 20,316 | 5.70 | 3.49 | 0 |
| wbuffix | 16K | 4K | 1 | 129.9 | 33,261 | 961.0 | 14,090 | 5.92 | 8.84 | 0 |
| wbuffix | 16K | 4K | 2 | 129.7 | 33,200 | 962.8 | 14,090 | 5.92 | 8.82 | 0 |
| wbuffix | 16K | 8K | 1 | 217.6 | 27,849 | 1,148 | 16,581 | 5.91 | 5.91 | 0 |
| wbuffix | 16K | 8K | 2 | 218.0 | 27,904 | 1,146 | 16,712 | 5.91 | 5.93 | 0 |
| wbuffix | 16K | 16K | 1 | 420.7 | 26,926 | 1,187 | 16,712 | 5.92 | 3.13 | 0 |
| wbuffix | 16K | 16K | 2 | 420.3 | 26,900 | 1,188 | 16,908 | 5.92 | 3.12 | 0 |
| wbuffix | 16K | 32K | 1 | 439.5 | 14,064 | 2,273 | 18,219 | 5.85 | 3.30 | 0 |
| wbuffix | 16K | 32K | 2 | 444.9 | 14,236 | 2,246 | 18,219 | 5.85 | 3.34 | 0 |
| wbuffix | 16K | 64K | 1 | 486.4 | 7,782 | 4,109 | 19,005 | 5.70 | 3.53 | 0 |
| wbuffix | 16K | 64K | 2 | 486.2 | 7,780 | 4,110 | 19,005 | 5.70 | 3.53 | 0 |
| wbuffix | 16K | 128K | 1 | 496.9 | 3,976 | 8,045 | 20,578 | 5.69 | 3.49 | 0 |
| wbuffix | 32K | 4K | 1 | 68.3 | 17,477 | 1,830 | 14,483 | 6.07 | 17.07 | 0 |
| wbuffix | 32K | 8K | 1 | 131.7 | 16,858 | 1,897 | 15,008 | 6.07 | 8.95 | 0 |
| wbuffix | 32K | 16K | 1 | 219.3 | 14,036 | 2,278 | 17,433 | 6.07 | 5.94 | 0 |
| wbuffix | 32K | 32K | 1 | 424.5 | 13,584 | 2,354 | 17,695 | 6.10 | 3.12 | 0 |
| wbuffix | 32K | 64K | 1 | 446.7 | 7,147 | 4,475 | 19,005 | 5.98 | 3.30 | 0 |
| wbuffix | 32K | 128K | 1 | 496.7 | 3,973 | 8,049 | 20,316 | 5.70 | 3.49 | 0 |
| wbuffix | 64K | 4K | 1 | 47.1 | 12,046 | 2,655 | 11,338 | 3.68 | 48.20 | 0 |
| wbuffix | 64K | 8K | 1 | 93.3 | 11,939 | 2,679 | 11,469 | 3.67 | 24.43 | 0 |
| wbuffix | 64K | 16K | 1 | 175.2 | 11,210 | 2,853 | 11,862 | 3.67 | 13.14 | 0 |
| wbuffix | 64K | 32K | 1 | 285.6 | 9,140 | 3,499 | 12,124 | 3.68 | 8.25 | 0 |
| wbuffix | 64K | 64K | 1 | 645.1 | 10,321 | 3,098 | 11,993 | 3.87 | 3.61 | 0 |
| wbuffix | 64K | 128K | 1 | 666.0 | 5,328 | 6,002 | 22,675 | 3.88 | 3.67 | 0 |
| wbuffix | 128K | 4K | 1 | 28.1 | 7,202 | 4,442 | 10,158 | 2.48 | 112.72 | 0 |
| wbuffix | 128K | 8K | 1 | 56.1 | 7,182 | 4,454 | 10,158 | 2.47 | 56.45 | 0 |
| wbuffix | 128K | 16K | 1 | 110.9 | 7,098 | 4,507 | 10,289 | 2.48 | 28.52 | 0 |
| wbuffix | 128K | 32K | 1 | 205.0 | 6,559 | 4,877 | 12,648 | 2.48 | 15.32 | 0 |
| wbuffix | 128K | 64K | 1 | 340.7 | 5,450 | 5,868 | 16,712 | 2.47 | 9.09 | 0 |
| wbuffix | 128K | 128K | 1 | 837.8 | 6,702 | 4,770 | 11,338 | 3.87 | 3.65 | 0 |

표 68. 회차별 결과 — seq3x3_20261008 (analysis/summary_runs.csv)

| 변형 | 매핑 | bs | 회 | MiB/s | IOPS | clat 평균 µs | clat p99 µs | 첫 GC s | WAF total | 채널모델 오류 줄 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| merge | 4K | 4K | 1 | 2,090 | 535,048 | 58.9 | 62 | 5.92 | 1.00 | 0 |
| merge | 4K | 4K | 2 | 2,084 | 533,396 | 59.0 | 63 | 5.94 | 1.00 | 0 |
| merge | 4K | 4K | 3 | 2,086 | 534,116 | 59.0 | 63 | 5.93 | 1.00 | 0 |
| merge | 4K | 16K | 1 | 2,233 | 142,885 | 222.5 | 228 | 5.69 | 1.00 | 0 |
| merge | 4K | 16K | 2 | 2,233 | 142,895 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| merge | 4K | 16K | 3 | 2,233 | 142,893 | 222.5 | 228 | 5.69 | 1.00 | 0 |
| merge | 4K | 32K | 1 | 2,233 | 71,449 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| merge | 4K | 32K | 2 | 2,233 | 71,448 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| merge | 4K | 32K | 3 | 2,233 | 71,448 | 446.0 | 449 | 5.69 | 1.00 | 0 |
| merge | 16K | 4K | 1 | 2,090 | 535,018 | 58.9 | 62 | 5.92 | 1.00 | 0 |
| merge | 16K | 4K | 2 | 2,093 | 535,788 | 58.8 | 62 | 5.90 | 1.00 | 0 |
| merge | 16K | 4K | 3 | 2,100 | 537,667 | 58.6 | 62 | 5.89 | 1.00 | 0 |
| merge | 16K | 16K | 1 | 2,233 | 142,886 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| merge | 16K | 16K | 2 | 2,232 | 142,880 | 222.5 | 228 | 5.69 | 1.00 | 0 |
| merge | 16K | 16K | 3 | 2,233 | 142,885 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| merge | 16K | 32K | 1 | 2,233 | 71,444 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| merge | 16K | 32K | 2 | 2,232 | 71,440 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| merge | 16K | 32K | 3 | 2,233 | 71,443 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| merge | 32K | 4K | 1 | 2,095 | 536,406 | 58.7 | 62 | 5.90 | 1.00 | 0 |
| merge | 32K | 4K | 2 | 2,095 | 536,262 | 58.7 | 62 | 5.90 | 1.00 | 0 |
| merge | 32K | 4K | 3 | 2,094 | 535,997 | 58.7 | 62 | 5.91 | 1.00 | 0 |
| merge | 32K | 16K | 1 | 2,232 | 142,866 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| merge | 32K | 16K | 2 | 2,232 | 142,861 | 222.6 | 226 | 5.69 | 1.00 | 0 |
| merge | 32K | 16K | 3 | 2,232 | 142,864 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| merge | 32K | 32K | 1 | 2,232 | 71,433 | 446.1 | 453 | 5.69 | 1.00 | 0 |
| merge | 32K | 32K | 2 | 2,232 | 71,431 | 446.1 | 453 | 5.69 | 1.00 | 0 |
| merge | 32K | 32K | 3 | 2,232 | 71,433 | 446.1 | 449 | 5.69 | 1.00 | 0 |
| wbuffix | 4K | 4K | 1 | 2,098 | 537,053 | 58.6 | 63 | 5.88 | 1.00 | 0 |
| wbuffix | 4K | 4K | 2 | 2,090 | 535,049 | 58.9 | 62 | 5.91 | 1.00 | 0 |
| wbuffix | 4K | 4K | 3 | 2,081 | 532,765 | 59.1 | 63 | 5.95 | 1.00 | 0 |
| wbuffix | 4K | 16K | 1 | 2,232 | 142,835 | 222.6 | 228 | 5.69 | 1.00 | 0 |
| wbuffix | 4K | 16K | 2 | 2,233 | 142,895 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| wbuffix | 4K | 16K | 3 | 2,233 | 142,892 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| wbuffix | 4K | 32K | 1 | 2,233 | 71,446 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| wbuffix | 4K | 32K | 2 | 2,233 | 71,448 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| wbuffix | 4K | 32K | 3 | 2,233 | 71,447 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| wbuffix | 16K | 4K | 1 | 334.3 | 85,582 | 372.9 | 3,654 | 5.69 | 5.40 | 0 |
| wbuffix | 16K | 4K | 2 | 335.2 | 85,807 | 371.9 | 3,654 | 5.69 | 5.40 | 0 |
| wbuffix | 16K | 4K | 3 | 335.2 | 85,809 | 371.9 | 3,621 | 5.69 | 5.40 | 0 |
| wbuffix | 16K | 16K | 1 | 2,232 | 142,879 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| wbuffix | 16K | 16K | 2 | 2,233 | 142,884 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| wbuffix | 16K | 16K | 3 | 2,233 | 142,880 | 222.5 | 226 | 5.69 | 1.00 | 0 |
| wbuffix | 16K | 32K | 1 | 2,232 | 71,431 | 446.1 | 449 | 5.69 | 1.00 | 0 |
| wbuffix | 16K | 32K | 2 | 2,233 | 71,444 | 446.0 | 449 | 5.69 | 1.00 | 0 |
| wbuffix | 16K | 32K | 3 | 2,233 | 71,442 | 446.0 | 453 | 5.69 | 1.00 | 0 |
| wbuffix | 32K | 4K | 1 | 209.5 | 53,634 | 595.5 | 2,277 | 5.69 | 9.15 | 0 |
| wbuffix | 32K | 4K | 2 | 209.5 | 53,635 | 595.5 | 2,245 | 5.69 | 9.15 | 0 |
| wbuffix | 32K | 4K | 3 | 209.5 | 53,635 | 595.5 | 2,245 | 5.69 | 9.15 | 0 |
| wbuffix | 32K | 16K | 1 | 369.0 | 23,618 | 1,353 | 8,356 | 5.69 | 3.40 | 0 |
| wbuffix | 32K | 16K | 2 | 369.0 | 23,618 | 1,353 | 8,356 | 5.69 | 3.40 | 0 |
| wbuffix | 32K | 16K | 3 | 369.1 | 23,620 | 1,353 | 8,356 | 5.69 | 3.40 | 0 |
| wbuffix | 32K | 32K | 1 | 2,232 | 71,421 | 446.2 | 453 | 5.69 | 1.00 | 0 |
| wbuffix | 32K | 32K | 2 | 2,232 | 71,434 | 446.0 | 449 | 5.69 | 1.00 | 0 |
| wbuffix | 32K | 32K | 3 | 2,232 | 71,432 | 446.1 | 449 | 5.69 | 1.00 | 0 |

표 69. 회차별 결과 — randbs_20261008 (analysis/summary_runs.csv)

| 변형 | 매핑 | bs | 회 | MiB/s | IOPS | clat 평균 µs | clat p99 µs | 첫 GC s | WAF total | 채널모델 오류 줄 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wbuffix | 4K | 8K | 1 | 438.1 | 56,081 | 569.4 | 16,581 | 5.74 | 3.33 | 0 |
| wbuffix | 4K | 8K | 2 | 437.2 | 55,963 | 570.6 | 16,581 | 5.74 | 3.32 | 0 |
| wbuffix | 4K | 8K | 3 | 435.7 | 55,771 | 572.6 | 16,581 | 5.74 | 3.31 | 0 |
| wbuffix | 4K | 64K | 1 | 486.3 | 7,781 | 4,110 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 4K | 64K | 2 | 486.3 | 7,782 | 4,109 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 4K | 64K | 3 | 486.3 | 7,781 | 4,110 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 16K | 8K | 1 | 217.3 | 27,809 | 1,149 | 16,581 | 5.91 | 5.90 | 0 |
| wbuffix | 16K | 8K | 2 | 217.5 | 27,835 | 1,148 | 16,450 | 5.90 | 5.90 | 0 |
| wbuffix | 16K | 8K | 3 | 217.2 | 27,800 | 1,150 | 16,581 | 5.90 | 5.89 | 0 |
| wbuffix | 16K | 64K | 1 | 486.3 | 7,780 | 4,110 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 16K | 64K | 2 | 486.5 | 7,784 | 4,108 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 16K | 64K | 3 | 486.5 | 7,783 | 4,109 | 19,005 | 5.69 | 3.53 | 0 |
| wbuffix | 32K | 8K | 1 | 131.9 | 16,882 | 1,894 | 15,008 | 6.07 | 8.97 | 0 |
| wbuffix | 32K | 8K | 2 | 131.8 | 16,874 | 1,895 | 15,008 | 6.07 | 8.96 | 0 |
| wbuffix | 32K | 8K | 3 | 132.1 | 16,913 | 1,891 | 15,139 | 6.07 | 8.99 | 0 |
| wbuffix | 32K | 64K | 1 | 447.1 | 7,154 | 4,470 | 19,005 | 5.97 | 3.31 | 0 |
| wbuffix | 32K | 64K | 2 | 447.1 | 7,154 | 4,470 | 19,005 | 5.97 | 3.31 | 0 |
| wbuffix | 32K | 64K | 3 | 447.2 | 7,155 | 4,470 | 19,005 | 5.97 | 3.31 | 0 |

# 부록 E. NVMeVirt 코드 감사 요약

실험 전에 에이전트 여러 개로 NVMeVirt 코드를 독립적으로 읽고(쓰기 경로·쓰기 버퍼 / 초기화·기하 구조 / GC·타이밍), 중요 주장마다 반박 시도 2 개(코드 경로 추적, 수치 예시 대조)로 검증했다. 결과 요약:

NVMeVirt 코드 감사 (워크플로 wf_b41bb861-a42, 2026-10-08 13:49–15:22 KST, 에이전트 51 개, 오류 0)

원본: snu-csl/nvmevirt 61c90f7 + 계획한 수정(매핑 단위, BLKS_PER_PLN 384, 64K/128K flash page = 매핑 단위)

단계: 읽기 2 (설정조사 보고서, 논문) → 감사 렌즈 3 (쓰기 경로/버퍼, 초기화/기하, GC/타이밍) → 주장마다 반박 시도 2 회

("반박"은 대개 세부 수치·범위의 정정이며 핵심 결론이 뒤집힌 것은 없다)

(전체 결과 JSON exp/report/audit_result.json 은 검증 판정문이 원본 모델(base) 측정값을 인용하고 있어, 2026-10-08 23:38 KST 에

사용자 요청에 따라 저장소 최신 상태에서 뺐다. 이 요약에서도 그 측정값을 뺐다.)

[blocker] 쓰기 버퍼 과다 반환 (bs < 매핑 단위 15 조합)  — 렌즈 3 개 모두 독립 발견, 검증 6 회 모두 핵심 확인

conv_write 는 요청 바이트를 할당(conv_ftl.c:961), 프로그램 완료 때 oneshot page 를 반환(conv_ftl.c:1007, io.c:671),

buffer_release 는 상한 없음(ssd.c:39-46). oneshot 당 과다 반환 = ONESHOT × (1 − bs/MAP).

WRITE_EARLY_COMPLETION=1 이라 버퍼가 NAND·GC 와 호스트를 잇는 유일한 연결 → bs<MAP 에서 끊김.

정정: 과다 반환이 있어도 GC 이전의 호스트 속도가 무한정 오르지는 않는다. 밀린 NAND 작업이 쌓일수록 io-worker 의

대기 항목 정렬 삽입(__insert_req_sorted, io.c, O(N))이 느려져 브레이크가 된다. 백로그가 393 ms 채널 창을 넘으면

요청마다 chmodel printk 가 찍혀(rate limit 없음) 타이밍이 에뮬레이터 산물이 된다.

수정안(채택, WBUF_FIX=1): 할당 = (end_lpn − start_lpn + 1) × pgsz.

[high] 과다 반환의 결과 — NAND 대기열 증가 → io-worker 작업 큐(워커당 16384) 고갈 시 명령이 조용히 버려짐 →

nvme 타임아웃·리셋 가능, 393 ms 채널 창 초과 시 '[chmodel_request] Need to increase array size' printk 폭주(rate limit 없음).

[high] 64K/128K 에서 FLASH_PAGE_SIZE = 매핑 단위 → NAND 모델 자체가 바뀜 (검증 4 회 확인)

die 당 wordline 시간 223.9 / 262.8 / 340.6 µs (32K/64K/128K), die 당 146 / 249 / 385 MB/s (1.70×, 2.63×),

16 die 합 2233 / 3805 / 5871 MiB/s, 쓰기 버퍼 1 / 2 / 4 MiB. 32K→64K 경계 차이는 매핑 단위만의 효과가 아님.

대안(미적용): NAND_PROG_LATENCY = 185000 × FLASH_PAGE_SIZE / 32K.

[high] 쓰기 조기 완료 — 호스트 완료 시각 = 4000 + 460×ceil(bs/4K) ns + PCIe 전송, NAND·GC 시간은 들어가지 않음.

bs≥MAP 에서 GC 는 버퍼가 찰 때의 SQ 재시도(시각 다시 잼)와 디스패처의 do_gc CPU 시간으로만 호스트에 보인다.

[high] 대상 장치는 모델명으로 찾아야 함(루트 FS 가 nvme0n1) — 스크립트에 반영(모델·크기·파티션·마운트·루트 검사).

[medium] 모델 한계 — PCIe 3357 MiB/s(110 크레딧/4 µs), 지연 하한 5.6–55.6 µs(4K–128K).

정정: GC 전 4k/4k 는 CPU 병목이 아니라 NAND 한계(2233 MiB/s)의 약 90 %(wbuffix 실측 2,001 MiB/s, 주 데이터셋 4K/4K 의 GC 이전 구간).

예전 64K IOPS(250 MiB/s)는 재적재 없이 연속 실행한 GC 정상 상태 값.

[medium] I/O 워커 선택은 SQ 기준((sqid−1) % nr_io_workers, nvmev.h:12 CONFIG_NVMEV_IO_WORKER_BY_SQ)

정정·추가: 이 서버에서 NVMeVirt 는 MSI-X 없이 레거시 IO-APIC IRQ 15 하나(nvme1q0, nvme1q1 공유)만 받아 I/O 큐가 1 개

('1/0/0 default/read/poll queues') → 모든 요청이 sqid 1 → 워커 0(cpu4)만 일한다. fio 의 CPU 위치와 무관하므로 반복 편차 원인이 아니다.

워커 1(cpu5)은 I/O 없이 폴링만 하고, IRQ 15 의 effective affinity 가 cpu5 라 호스트 완료 인터럽트를 cpu5 가 처리한다(irqbalance 미설치).

[medium] 쓰기 버퍼 크기·wordline 묶음이 매핑 단위에 따라 변함 — pgs_per_oneshotpg 8/4/2/1/1/1, WB 1/1/1/1/2/4 MiB.

매핑 4K 에서는 파티션당 최대 7 페이지 × 4 파티션 = 112 KiB 의 덜 찬 wordline 이 버퍼에 남는다.

[medium] RMW 미모델 — 부분 쓰기는 옛 페이지 읽기 없이 무효화 + 새 페이지 프로그램만 함(데이터 무결성은 논리 주소 저장이라 문제없음).

작은 bs·큰 매핑 단위의 불이익을 과소평가한다(실제 RMW 는 tR + 매핑 단위 채널 전송이 추가됨).

[medium] 매핑 표는 호스트 메모리 배열(DFTL/CMT 캐시 모델 없음) — L2P 크기 효과는 성능에 나타나지 않는다.

[medium] GC 시작 시각 — 파티션당 380 line(= 장치 전체 12,160 MiB 페이지 쓰기) 뒤, 매핑 단위와 무관하게 같다.

bs≥MAP 이면 12,160 MiB / BW. 60 s 평균은 GC 전후 구간이 조합마다 다른 비율로 섞인다 → 시계열·GC 시각으로 나눠 보고(반영함).

[medium] fio randommap 과 탐욕적 victim 선택 → GC 시작 직후 WA 절벽, 둘째 바퀴 동안 감소(끝 무렵 약 1.5), 셋째 바퀴 시작에서 다시 급등.

bs<MAP 는 WA 가 7.6–8.4 까지 서서히 오르고 물리 비용 = WA × MAP/bs. 60 s 창의 GC 구간은 정상 상태가 아니다.

[info] 모든 init assert 통과, 블록 정확히 2 MiB, 노출 용량 12,040,984,064 B·OP 7.01 % 는 모든 매핑 단위에서 같음,

MDTS 6 → 256 KiB 라 128K 요청은 나뉘지 않음(max_hw_sectors_kb 256), MAX_CH_XFER_SIZE 16K 는 쓰기에 무관,

4 KB 전용 상수(ssd.c:394, conv_ftl.c:867)는 쓰기 전용 실험에 무관, rmmod/insmod 는 FTL 을 완전히 초기화(저장 데이터는 남음),

지우기 지연 0·GC 읽기→쓰기 의존 미강제로 GC 비용은 조금 과소평가(모든 조합 공통).